Full factorial optimization of α-aminophosphonates synthesis using diphenylphosphinic acid as efficient Organocatalyst

Meriem FARREHa, Samia GUEZANE LAKOUDa*, Hacene BENDJEFFALb, Rym AISSA,a Mounia MERABET-KHELASSIa, Martial TOFFANOc, Louisa ARIBI-ZOUIOUECHEa

aEcocompatible Asymmetric Catalysis Laboratory (LCAE) Badji Mokhtar Annaba-University, B.P 12, 23000 Annaba, Algeria

bResearch Center in Biotechnology, Ali Mendjli Nouvelle Ville UV 03, BP E73 Constantine, Algeria

c Equipe de Catalyse Moléculaire-ICMMO Bât 420. Université Paris- Saclay.

*Corresponding Author:  Samia GUEZANE LAKOUD

Email: samia.guezanelakoud@univ-annaba.dz

Content:

- General ………………………………………………………………………………..3

- General procedure for α-aminophosphonates synthesis……………..………………...3

- 1H, 13C,  31P NMR and HRMS data…………………………………………….….3-7

- 1H, 13C, 31P NMR and HRMS spectra of α-aminophosphonates compounds….…8-18

1. General

All reagents were purchased from Sigma-Aldrich or Acros Company used without further purification. Reactions were monitored by thin-layer chromatography (TLC) carried out on 0.25-mm Merck silica gel plates (60F-254) using ultraviolet light (254 nm) as the visualizing agent and KMnO4 solution as developing agents. NMR spectra were recorded with Bruker spectrometers operating at (360MHz, 300 MHz, and 250 MHz for 1H, 90 MHz, 75 MHz, or 63 MHz for 13C, and 101 MHz or 121 MHz for 31P). The chemical shift of Solvent reference peaks used were CDCl3 (δ = 7.26 ppm) for 1H and (δ = 77ppm) for 13C NMR spectra, while H3PO4 was used as external standard for chemical shift references for 31P NMR. Couplings constants (J) are given in Hz, with the Following abbreviations multiplicity: s = singlet, d = doublet, t = triplet, q = quartet, m = multiplet, br = broad signal. Mass spectra were taken by a MicrOTOF-Q Bruker spectrometer using electrospray ionization (ESI) analysis. Melting points were measured using Buchi Melting Point B-545.

2. General procedure for the synthesis of α-aminophosphonates 4a-i.

The diphenylphosphinic acid (10 mol%) was added to the reaction mixture of aromatic aldehyde (122.12 mg, 1 mmol), aniline (93 mg, 1 mmol) and diethylphosphite (165 mg, 1.2 mmol) in ethanol (10	 mL). The reaction was stirred at 40°C for 30 min. The progress of the reaction was monitored by TLC. The solvent was removed and the resulting residue was treated with HCl (1N) then washed with water (10 mL) and extracted with dichloromethane (10 mL×2). The organic phases were combined and evaporated in vacuum. The crude product was purified by crystallization in hexane. Complete experimental data have been provided (NMR spectra and HRMS).

3. 1H, 13C,  31P NMR and HRMS data

3.1. Diethyl[phenyl(phenylamino)methyl]phosphonate (4a)

Yield: 92%, as a white crystalline solid; mp 88 °C. 1H NMR (250 MHz, CDCl3, 25°C): δ 7.52 -7.29 (m, 6H, ArH), 7.16 -7.10 (t, 1H,  J =7.9 Hz, ArH), 6.75-6.69 (t, 1H, , J =7.3 Hz, ArH), 6.64 -6.61 (m, 2H, ArH), 4.84-4.75 (d, 1H, JHP=  24.3 Hz, CHP), 4.18-4.10 (2H, m, -OCH2- CH3), 4.01-3.76 (2H, m, -OCH2-CH3), 3.74- 3.63 (1H, m, -OCH2 CH3), 1.31 (t, 3H, J=  7.1 Hz, -OCH2-CH3), 1.14 (t, 3H, J=  7.1 Hz, -OCH2-CH3). 13C NMR (63 MHz, CDCl3, 25°C): δ 146.33 (d, J = 14.5 Hz); 136.00, 129.29, 128.74, 128.71, 128.00, 127.03, 118.40, 113.87, 63.27 (d, J2CP = 6.9 Hz), 57.29, 54.90, 16.46, (d, J 3CP = 15.1), 16.37 (d, J 3CP = 5.6 Hz).  31P NMR (101 MHz, CDCl3, 25°C): δ 22.46 ppm. HRMS (ESI) m/z calcd for C17H23NO3P [M+ H+]: 320.1408; Found 320.1410.

3.2 Diethyl [(3-methoxyphenyl)(phenylamino)methyl] phosphonate (4b)

Yield: 98%, as a white crystalline solid; mp103 °C.1H NMR (300 MHz, CDCl3, 25 °C) δ 7.36–7.30 (m, 2H, HAr), 7.07–7.13 (m, 2H, HAr), 6.85 (d, J=7.3 Hz, 2H, H Ar), 6.66 (t,
J=16.0 Hz, 1H, HAr), 6.58 (d, 2H, J=8.5Hz, HAr), 4.72 (d, J=24.4 Hz, 1H, CHP), 4.15 (m, 2H, OCH2-CH3), 3.91 (m, 1H, -OCH2-CH3), 3.77 (s, 3H, -OCH3), 3.65–3.74 (m,
1H, -OCH2-CH3), 1.28 (t, 3H, J=7.0 Hz, -OCH2-CH3), 1.14 (t, 3H, J=7.0 Hz, -OCH2-CH3). 13C NMR (75 MHz, CDCl3, 25°C): δ 159.40, 143.54
(d, J= 15.4 Hz), 129.24, 129.08, 129.01, 127.77, 127.73, 118.43, 114.16, 114.13,
63.39 (dd, J2CP=6.9, 3.9 Hz), 56.45, 55.33, 54.43, 16.60 (d, J3CP=14.9 Hz), 16.52 (d,
J3CP = 5.9 Hz).31P NMR (121 MHz, CDCl3, 25 °C): δ 23.47 ppm.

3.3. Diethyl [4-nitrophenyl(phenylamino)methyl]phosphonate (4c)

Yield: 90%, as a yellow crystalline solid; mp 89.2°C. 1H NMR (400 MHz, CDCl3, 25°C): δ 8.22-8.19 (d, 2H, ArH), 7.68-7.64 (dd, 2H, JHP=8.9, 2.3 Hz, ArH), 7.19-6.95 (m, 2H, ArH), 6.76-6.71 (t, 1H, J=7.4 Hz, ArH), 6.56-6.52 (m, 2H, ArH), 4.88-4.81 (d, 1H, JHP= 25.6 Hz, CHP), 4.25-4.11 (m, 2H, OCH2-CH3), 3.99-4.02 (m, 1H, -OCH2-CH3), 3. 97-3. 78 (m, 1H, -OCH2-CH3), 1.32 (t, 3H, J= 7.1 Hz, -OCH2-CH3), 1.21 (t, 3H, J= 7.1Hz, -OCH2-CH3). 13C NMR (101 MHz, Chloroform-d) δ 147.59, 145.71 (d, J = 16.6 Hz), 143.92 (d, J = 2.5 Hz), 129.35, 128.64 (d, J = 4.7 Hz), 123.75, 119.12, 113.81, 63.63 (dd, J2CP = 27.4, 6.8 Hz), 56.78, 55.31, 16.34 (dd, J3cp = 17.2, 5.5 Hz). 31P NMR (162 MHz, Chloroform-d) δ 20.80. HRMS (ESI) m/z calcd for C17H21N2O5P [M + Na+]: 387.1094; Found 387.1080.

3.4. Diethyl[1-naphtyl(p-tolylamino)methyl]phosphonate (4d)

Yield: 90%, as a white crystalline solid; mp 145°C. 1H NMR (300 MHz, CDCl3, 25°C): δ 8.27 (d, 1H, JHP = 8.5 Hz, ArH), 7.91 (d, 1H, J = 7.9 Hz ArH), 7.80-7.77 (m, 2H, ArH), 7. 63-7. 48 (m, 2H, ArH), 7.45 (t, 1H, J = 7.7 Hz, ArH), 6.67 (d, 2H, J = 8.48 Hz, ArH), 6.48 (d, 2H, J = 8.4 Hz, ArH), 4.64 (d, 1H, JHP = 24.0 Hz, HCP), 4.23-4.15 (m, 2H, –OCH2–CH3), 3.77-3.73 (m, 1H, –OCH2–CH3), 3. 28-3.17 (m, 1H, –OCH2– CH3), 2.16 (s, 3H, CH3-Ph), 1.34 (t, 3H, J = 7.1 Hz, –OCH2–CH3), 0.75 (t, 3H, J = 7.1 Hz, –OCH2–CH3). 13C NMR (75 MHz, CDCl3) δ 143.83 (d, J = 24.8 Hz), 133.81, 131.6 (d, J=19.7 Hz), 129.70, 129.00, 128.40, 127.50, 126.22, 125.64, 125.35 (d, J = 6.0 Hz), 123.00, 113.67, 63.63 (dd, J2CP = 27.4, 6.8 Hz), 20.33, 16.49 (dd, J3cp =5.9, 5.9 Hz). 31P NMR (121 MHz, CDCl3, 25°C): δ 23.07 ppm. HRMS (ESI) m/z calcd for C22H26NO3P [M + Na+]: 406.1532; Found 406.1542.

3.5. Diethyl [benzo [1,3]dioxol-5-yl( (p-tolylamino) methyl]phosphonate (4e)

Yield: 91%, as a white crystalline solid; mp 115°C. 1H NMR (300 MHz, Chloroform-d): δ 7.51 – 7.33 (m, 1H, ArH), 7.00 – 6.89 (m, 4H, ArH), 6.78 (d, J=8.0 Hz, 1H, ArH), 6.53 (d, J=8.5 Hz, 2H, ArH), 5.97 – 5.91 (m, 2H,O-CH2-O), 4.66 (d, J HP = 24.0 Hz, 1H, HCP), 4.22 – 4.07 (m, 4H,-OCH2-CH3), 4.06 – 3.93 (m, 1H, -OCH2-CH3), 3.87 – 3.72 (m, 1H, -OCH2-CH3), 2.21 (s, 3H, CH3), 1.31 (t, J = 7.1 Hz, 3H, –OCH2–CH3), 1.19 (t, J = 7.1 Hz, 3H, –OCH2-CH3). 13C NMR (75 MHz, Chloroform-d) δ 147.64 (d, J = 49.1 Hz), 143.78, 129.74 (d, J = 11.6 Hz), 121.35 (d, J = 6.5 Hz), 113.99, 108.21 (d, J = 13.9 Hz), 101.10, 63.84-62.26 (m), 57.01, 54.99, 20.36, 16.37 (dd, J3cp = 12.4, 5.7 Hz). HRMS (ESI) m/z calcd for C19H24NNaO5P [M + Na+]: 400.128430; Found 400.1273. 31P NMR (121 MHz, Chloroform-d) δ 22.72.

3.6. Diethyl [(4-fluorophenyl)(4-tolylamino)methyl] phosphonate (4f)

Yield: 88%, as a white crystalline solid; mp 119°C. 1H NMR (300 MHz, Chloroform-d): δ 7.43 (dd, J = 12.0, 4.0 Hz, 2H, H Ar), 7.26 (m, 2H, HAr), 6.93 (d, 2H, J = 8.2 Hz, HAr), 6.48 (d, 2H, J = 8.5 Hz, HAr), 4.64–4.78 (m, 2H, CH+NH), 4.10–4.17 (m, 2H, -OCH2-CH3), 3.97–4.05 (m, 1H, -OCH2-CH3), 3.79–3.85 (m, 1H, -OCH2-CH3), 2.21 (s, 3H,
CH3–Ph), 1.31 (t, 3H, J=7.1 Hz, -OCH2-CH3), 1.18 (t, 3H,
J = 7.1 Hz, -OCH2-CH3). 13C NMR: (75 MHz, CDCl3, 25 °C): δ 143.85 (d,
J = 15.0 Hz), 134.84, 133.80, 133.76, 133.71, 129.83, 129.28, 128.21, 127.89,
127.86, 128.03, 114.06, 63.59 (dd, J2cp = 12.1, 7.0 Hz), 56.89, 54.89, 20.47, 16.59
(d, J3cp = 13.7 Hz), 16.52 (d, J3cp = 5.8 Hz). 31P NMR (121 MHz, Chloroform-d) δ 22.69.

HRMS (ESI) m/z calcd for C18H23FNNaO3P [M + Na+]: 374.1291; Found 374.1275. 31P NMR (121 MHz, Chloroform-d) δ 22.72.

3.7. Diethyl [(phenyl)(4-trifluoromethylamino)methyl] phosphonate (4g)

Yield: 88%, as white crystalline solid. mp 140 °C. 1H NMR (300 MHz, CDCl3, 25 °C): d = 1.12 (t, 3H, J=7.0 Hz, –OCH2–CH3), 1.31 (t, 3H, J = 7.1 Hz, –OCH2–CH3), 3.61–3.70
(m, 1H, –OCH2–CH3), 3.90–4.08 (dd, J=10.1,7.1Hz, 1H, -OCH2-CH3),
4.10–4.20 (m, 2H, –OCH2–CH3), 4.79 (dd, 1H, J = 24.2, 7.5 Hz, Ph–CH),
5.16–5.31 (m, 1H, NH), 6.62 (d, 2H, J = 8.5 Hz, H Ar), 7.26–7.40 (m, 5H, HAr),
7.49 (dd, J=7.7, 2.1 Hz, 2H, HAr). 13C NMR: (75 MHz, CDCl3, 25 °C):
d = 148.90 (d, J = 14.4 Hz), 135.13 (d, J = 3.0 Hz), 128.91, 128.87, 128.38,
128.34, 127.90, 127.83, 126.68, 126.63, 113.16, 63.75 (dd, J2cp= 22.9, 7.0 Hz),
56.77, 54.77, 16.59 (d, J3cp=19.0 Hz), 16.51 (d, J3cp= 5.8 Hz). 31P NMR:
(121 MHz, CDCl3, 25°C): δ 22.54 ppm.

3.8. Diethyl [3-phenoxyl(phenylamino)trifluoromethyl]phosphonate (4h)

Yield: 78%, as white crystalline solid. mp 155°C. 1H NMR (360 MHz, Chloroform-d) δ 7.43–7.25 (m, 5H, ArH), 7.20 (d, J = 7.5 Hz, 1H, ArH), 7.13 – 7.05 (m, 3H, ArH), 6.93 (d, J=7.6 Hz, 2H, ArH), 6.59 (d, J = 8.6 Hz, 2H, ArH), 4.74 (d, J = 24.4 Hz, 1H, HC*), 4.20 – 4.06 (m, 2H), 4.05 – 3.93 (m, 1H, –OCH2–CH3), 3.84– 3.69 (m, 1H, –OCH2–CH3), 1.29 (t, J =7.1 Hz, 3H, –OCH2–CH3), 1.16 (t, J =7.1 Hz, 3H, –OCH2–CH3). 13C NMR (91 MHz, Chloroform-d) δ 157.59, 156.82, 148.74 (d, J=14.3 Hz), 137.26, 129.92 (d, J = 30.6 Hz), 126.55, 123.45, 122.52 (d, J = 5.1 Hz), 120.35, 119.99, 118.83, 118.44, 118.14 (d, J = 5.3 Hz), 113.16, 63.49 (dd, J = 16.8, 6.9 Hz), 56.31, 54.64, 16.42 (d, J3=5.8 Hz), 16.24 (d, J3cp  = 5.7 Hz). 31P NMR (162 MHz, Chloroform-d) δ 21.54. HRMS (ESI) m/z calcd for C24H25F3NNaO4P [M + Na+]: 502.1365; Found 400.1343

3.9. Diethyl [benzo [1,3]dioxol-5-yl( (phenylamino)trifluoromethyl]phosphonate (4i)

Yield: 87%, as white crystalline solid. mp 148.2 °C. 1H NMR (400 MHz, Chloroform-d) δ 7.36 (d, J = 8.5 Hz, 2H), 6.97 (s, 1H), 6.95 – 6.90 (m, 1H), 6.80 (d, J = 8.0 Hz, 1H), 6.64 (d, J = 8.5 Hz, 2H), 5.96 (d, J=5.0 Hz, 1H), 5.18 (t, J = 8.4 Hz, 1H), 4.69 (dd, J = 23.9, 6.6 Hz, 1H), 4.15 (dq, J = 14.3, 7.1 Hz, 2H), 4.07 – 3.94 (m, 1H), 3.84 – 3.71 (m, 1H), 1.32 (t, J = 7.1 Hz, 3H), 1.19 (t, J = 7.1 Hz, 3H). 13C NMR (101 MHz, Chloroform-d) δ 148.88 (d, J = 34.5 Hz), 126.50, 121.32 (d, J =6.2 Hz), 120.14 (d, J = 34.6 Hz), 113.08, 108.42, 107.98 (d, J =4.2 Hz), 101.25, 63.56 (d, J=7.0 Hz), 63.28 (d, J = 7.0 Hz), 56.14, 54.62, 16.43 (d, J3cp =5.6 Hz), 16.27 (d, J=5.5 Hz). 31P NMR (162 MHz, Chloroform-d) δ 21.87. HRMS (ESI) m/z calcd for C19H21F3NNaO5P [M + Na+]: 454.1001; Found 454.0991.

4. 1H, 13C, 31P NMR and HRMS spectra of all α-aminophosphonates compounds:

1H, 13C, 31P NMR and HRMS of diethoxy (phenyl)(phenylamino)methylphosphoryle 4a:

- 1H, 13C and 31P NMR of Diethyl (3-methoxyphenyl) (phenylaminomethyl) phosphonate (4b)

- 1H, 13C, 31P NMR and HRMS of diethyl [4-nitrophenyl(phenylamino)methyl]phosphonate (4c)

1H, 13C, 31P NMR and HRMS of diethyl[1-naphtyl(p-tolylamino)methyl]phosphonate (4d)

1H, 13C, 31P NMR and HRMS of diethyl [benzo [1,3]dioxol-5-yl( (p-tolylamino) methyl]phosphonate (4e)

1H, 13C, 31P NMR and HRMS of diethyl [(4-fluorophenyl)(4-tolylamino)methyl] phosphonate (4f)

1H, 13C and 31P NMR of diethyl [(phenyl)(4-trifluoromethylamino)methyl] phosphonate (4g)

1H, 13C, 31P NMR and HRMS of diethyl [3-phenoxyl(phenylamino)trifluoromethyl]phosphonate (4h)

1H, 13C, 31P NMR and HRMS of diethyl [benzo [1,3]dioxol-5-yl( (phenylamino)trifluoromethyl]phosphonate (4i)