Supplementary Table 1. The list of antibodies used for immunohistochemistry in this study

|  | Antibodies (clone) |  | Cat. No. | Clone | Company | Dilution | Antigen retrieval | Endogenous peroxidase blocking | Incubation time with primary antibody | Secondary and chromogen | Machine |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | EBNA2 | Anti-EBNA2 antibody [PE2] | ab90543 | PE2 | abcam | 1:200 | 20 min with ER2 Buffer (pH 8.0) in 97 ℃ Bond-Rx | 5min | 15min with Bond-Rx autoimmunostainer (Leica Biosystem, Melbourne, Australia) | 10 min Bond-RX autoimmunostainer (Leica Biosystem, Melbourne, Australia) using Bond™ Polymer refine detection, DS9800 (Vision Biosystems, Melbourne, Australia) | Bond-RX autoimmunostainer |
| 2 | CD8 | CONFIRM anti-CD8 (SP57) Rabbit Monoclonal Primary Antibody | 790-4460 | SP57 | Ventana | RTU | 60min with CC1 in Ventana BenchMark XT (Standard-CC1) | 4min | 24min with Ventana BenchMark XT in 37 ℃ | ultraView Universal DAB Detection Kit (760-500) | BenchMark XT |
| 3 | CD68 | Novocastra Liquid Mouse Monoclonal Antibody CD68 | NCL-L-CD68 | 514H12 | Leica | 1:50 | 20 min with ER1 Buffer (pH 6.0) in 97 ℃ Bond-max | 10min | 15min with Bond-max autoimmunostainer (Leica Biosystem, Melbourne, Australia) | 10 min Bond-max autoimmunostainer (Leica Biosystem, Melbourne, Australia) using Bond™ Polymer refine detection, DS9800 (Vision Biosystems, Melbourne, Australia) | Bond max |
| 4 | PD-L1 | Rabbit Anti-Human PD-L1/CD274 Monoclonal Antibody (Clone SP142) | M4424 | SP142 | Spring Bioscience | 1:25 | 92min with CC1 in Ventana BenchMark XT (Standard-CC1) | 4min | 120min with Ventana BenchMark XT in 37 ℃ | 8min with OptiView Amplification Kit (860-099) + OptiView DAB IHC Detection kit (760-700) | BenchMark XT |