Supplementary data:

- The calculations of the CO2 eq./year for all (ENZ vs CHEM/AUT) processes due to thermal energy consumption were calculated as follows:

Given:

- From table 7_AUT, the daily steam consumption (CHEM/AUT) is equivalent to 22700 MJ

- 1 GJ of thermal energy generates 55.82 kg of CO2

1. Converting MJ to  GJ diving by 1000= 22700/1000 =22.7 GJ

2. Converting energy consumption in terms of GJ to the quantity of CO2 eq.

daily generation of CO2 kg CO2 eq./d=22.7 ×55.82=1267.1 kg CO2 eq

dividing by 1000

Daiy generation of CO2 (t CO2 eq.)=1267.1/1000=1.267 t CO2 eq

Annual generation of CO2=1.267 ×330=418  t CO2 eq. (table 7)

______________________________

- From table 7_ENZ, the daily steam consumption is equivalent to 1713 MJ

- 1 GJ of thermal energy generates 55.82 kg of CO2

1. Converting MJ to  GJ diving by 1000= 1713/1000 =1.713 GJ

2. Converting energy consumption in terms of GJ to the quantity of CO2 eq.

daily generation of CO2 kg CO2eqd=1.713 ×55.82=95.62 kg CO2 eq

dividing by 1000

Daiy generation of CO2 (t CO2 eq.)=95.62/1000=0.0956 t CO2 eq

Annual generation of CO2=0.0956 ×330=31.55  t CO2 eq. (table 7)

__________________________________________________________________

- The calculations of the CO2 eq./year for both processes due to electrical energy consumption were calculated as follows:

- Given_ 1 kWh equivalent to 438.64 g CO2 eq (Mata et al., 2018)

- Given in table 7_AUT/CHEM, the daily electrical energy consumption is 1800 kWh

Daily generation of t CO2 = (1800 × 438.64) / 1000000 = 0.7895 t CO2

Annual generation of t CO2 = 0.7895 × 330 = 260.55 t CO2 (see table 11)

- Given in table 7_ENZ, the daily electrical energy consumption is 125 kWh

Daily generation of t CO2 = (125 × 438.64) / 1000000 = 0.05483 t CO2

Annual generation of t CO2 = 0.05483 × 330 = 18 t Co2 (see table 11)

_______________________________________________________