Supplementary Information- Table S1-S5

Presence and risk assessment of pharmaceutically active compounds in neotropical small basins, Brazil

Archives of Environmental Contamination and Toxicology

Alessandro Minilloa*; William Deodato Isiqueb†; Claudia Andrea Lima Cardosoc†

Yzel Rondon Súarezd

a Center of Studies in Natural Resources, State University of Mato Grosso do Sul (UEMS), Dourados, MS, 79804-970. Brazil. E-mail: alminillo@yahoo.com.br

b Department of Civil Engineering, São Paulo State University (UNESP), Ilha Solteira, SP, 15385-000. Brazil. E-mail: isiquewd2020@gmail.com.

c Center of Studies in Natural Resources, State University of Mato Grosso do Sul (UEMS), Dourados, MS, 79804-970. Brazil. E-mail: claudia@uems.br.

d Center of Studies in Natural Resources, State University of Mato Grosso do Sul (UEMS), Dourados, MS, 79804-970. Brazil. E-mail: yzel@uems.br.

† These authors have contributed equally to this work.

* Correspondence: alminillo@yahoo.com.br  (Alessandro Minillo) - Tel: 55 67- 98107-7568;

Table S1

Basic information and physicochemical properties for target pharmaceutical compounds

| Therapeutic groups | Compounds | Molecular formula | Molecular weight | CAS | pKa | Water solubility at 25 ℃ (mg L-1) | logKow | Purity | Supplier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stimulants | caffeine | C8H10N4O2 | 194.19 | 58-08-2 | 14.0a; 10.0a; 0.7a | 21,600a | - 0.07 | 98% | Sigma-Aldrich |
| Anti-inflammatory | diclofenac | C14H11Cl2NO2 | 296.147 | 15307-86-5 | 4.15a | 2.37a | 4.51a | 97% | Sigma-Aldrich |
|  | ibuprofen | C13H18O2 | 206.285 | 15687-27-1 | 4.91a | 21a | 3.97a | 97% | Sigma-Aldrich |
|  | naproxen | C14H14O3 | 230.263 | 22204-53-1 | 4.15a | 15.9a | 3.18a | 97% | Sigma-Aldrich |
|  | piroxicam | C15H13N3O4S | 331.3 | 36322-90-4 | 6.3a | 23a | 2.58c | 97% | Sigma-Aldrich |
| Hormones | estriol | C18H24O3 | 288.4 | 50-27-1 | 10.54a | 441a | 2.45 | 98% | Sigma-Aldrich |
|  | estradiol | C18H24O2 | 272.4 | 50-28-2 | 10.46a | 3.6a | 4.01 | 98% | Sigma-Aldrich |
|  | ethinylestradiol | C20H24O2 | 296.4 | 57-63-6 | 10.20a | 11.3a | 3.67 | 98% | Sigma-Aldrich |

a Data from PubChem (https://pubchem.ncbi.nlm.nih.gov/);

b Data from Drugbank (https://www.drugbank.ca/);

c Data from Chemspider (http://www.chemspider.com/).

Table S2

Correlation coefficient, limit of detection (LOD), limit of quantification (LOQ) and recoveries for the target compounds.

| Compounds | r2 | LOD (ng/L) | LOQ (ng/L) | Recoveries (%) |
| --- | --- | --- | --- | --- |
| caffeine | 0.9972 | 0.55 | 1.1 | 90 |
| diclofenac | 0.9943 | 0.25 | 0.8 | 85 |
| ibuprofen | 0.9952 | 1.1 | 2.7 | 70 |
| naproxen | 0.9892 | 0.30 | 0.9 | 92 |
| piroxicam | 0.9982 | 1.2 | 3.3 | 75 |
| estriol | 0.9886 | 0.40 | 0.9 | 91 |
| estradiol | 0.9886 | 0.33 | 0.9 | 89 |
| ethinylestradiol | 0.9829 | 0.27 | 0.8 | 95 |

Table S3

Water quality parameters and time of concentrations of water in the streams sampled.

| Parameters | Sampling station |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Água Boa, mean ±SD |  | Engano, mean ±SD |  | Curral de Arame, mean ±SD |
|  | (minimum - maximum) |  | (minimum - maximum) |  | (minimum - maximum) |
| pH | 6.35 ±1.08 |  | 6.50 ±0.97 |  | 6.45 ±1.13 |
|  | (4.06 - 7.91) |  | (4.61 - 7.77) |  | (3.4 - 8.22) |
| Temperature (°C) | 23.70 ±2.91 |  | 22.88 ±2.93 |  | 23.24 ±2.50 |
|  | (19.43 - 28.9) |  | (18.84 - 28.64) |  | (19.31 - 27.67) |
| Conductivity (µS/cm) | 225.56 ± 92.40 |  | 108.33 ±50.31 |  | 45.23 ±9.30 |
|  | (100 - 470) |  | (63 - 219) |  | (28 - 63) |
| Dissolved oxygen (mg/L) | 4.89 ±2.28 |  | 8.59 ±4.08 |  | 8.55 ±4.39 |
|  | (0.85 - 10.49) |  | (4.09 - 20.4) |  | (2.57 - 19.81) |
| Oxygen saturation (%) | 57.94 ±28.53 |  | 98.23 ±50.03 |  | 103.61 ±55.40 |
|  | ( 10.70 - 137) |  | (46.8 - 256) |  | (31.50 - 251.70) |
| Turbidity (NTU) | 14.62 ± 14. 81 |  | 41.54 ±53.86 |  | 25.52 ±29.31 |
|  | (0 - 49.8) |  | (0.2 - 213) |  | (0 - 115) |

Table S4

Occurrence and frequency of pharmaceutical products in streams (ng/L) sampled.

| Compounds | Água Boa |  |  |  | Engano |  |  |  | Curral de Arame |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mean ±SD | Freq |  |  | mean ±SD | Freq |  |  | mean ±SD | Freq |
|  | [N]> LOQ a | (minimum - maximum) | (%) |  | [N]> LOQ a | (minimum - maximum) | (%) |  | [N]> LOQ a | (minimum - maximum) | (%) |
| CAF | 30 | 433.40 ±462.48 | 100 |  | 20 | 261.50 ±371.90 | 83.33 |  | 14 | 140.80 ±278.96 | 76.66 |
|  |  | (1.05 – 1,696.04) |  |  |  | (1.0 – 1,042.40) |  |  |  | (1.40 – 1,244.60) |  |
| NAP | 29 | 98.08 ±161.70 | 97 |  | 15 | 63.50 ± 148.40 | 62.5 |  | 23 | 20.60 ±29.0 | 60 |
|  |  | (0.90 - 681.80) |  |  |  | (1.40 – 554.60) |  |  |  | (1.40 – 121.50) |  |
| DIC | 27 | 173.0 ± 222.70 | 90 |  | 13 | 46.70 ±82.50 | 54.16 |  | 14 | 32.00 ±55.20 | 46.66 |
|  |  | (0.80 - 849.94) |  |  |  | (0.95 – 350.20) |  |  |  | (0.95 - 205.50) |  |
| E3 | 22 | 17.4 ±18.6 | 73.33 |  | 5 | 11.90 ±14.30 | 20.83 |  | 0 | < 0.90 | - |
|  |  | (0.90– 64.0) |  |  |  | (0.97– 28.80) |  |  |  |  |  |
| E2 | 20 | 118.30 ±192.70 | 66.66 |  | 5 | 201.80 ±243.30 | 20.83 |  | 0 | < 0.90 | - |
|  |  | (0.90– 587.30) |  |  |  | (0.97– 586.10) |  |  |  |  |  |
| EE2 | 22 | 347.40 ± 250.26 | 73.33 |  | 4 | 254.10 ±381.596 | 16.66 |  | 0 | < 0.80 | - |
|  |  | (0.80– 781.50) |  |  |  | (0.90– 790.90) |  |  |  |  |  |

a [N]>LD: number of samples with concentrations higher than the limit of quantification (LOQ) of the method; CAF.: Caffeine; NAP.: Naproxen; DIC.: Diclofenac; E3.: estriol; E2: Estradiol; EE2.: Ethinylestradiol

Table S5

Estimation of risk quotients for the pharmaceutical products contained in water of sampling stations. Predicted no-effect concentration (PNEC) most restrictive found in the literature and maximum environmental concentration (MEC) found in present study.

| Compounds |  |  | Sampling stations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PNEC |  |  | Água Boa |  |  |  | Engano |  |  |  | Curral de Arame |  |  |
|  |  | (ng/L) |  |  | MEC (ng/L) | RQ | Risk level |  | MEC (ng/L) | RQ | Risk level |  | MEC (ng/L) | RQ | Risk level |
| CAF |  | 320 a |  |  | 1,696.04 | 5.30 | High risk |  | 1042.40 | 3.25 | High risk |  | 1,244.80 | 3.89 | High risk |
| NAP |  | 1700 b |  |  | 681.80 | 0.40 | Medium risk |  | 554.60 | 0.32 | Medium risk |  | 121.53 | 0.07 | Low risk |
| DIC |  | 50 b |  |  | 849.94 | 16.99 | High risk |  | 350.20 | 7.00 | High risk |  | 205.50 | 4.11 | High risk |
| E3 |  | 60 c |  |  | 64.0 | 1.06 | High risk |  | 28.80 | 0.48 | Medium risk |  | n.d | - | - |
| E2 |  | 0,4 d |  |  | 587 | 1,467 | High risk |  | 586 | 1,465 | High risk |  | n.d | - | - |
| EE2 |  | 0.035 d |  |  | 781,50 | 22,328 | High risk |  | 790 | 22,571 | High risk |  | n.d | - | - |

a Zhou et al (2019); b Ecotox centre (2021); c Caldwell et al. (2012); d Loos et al. (2018);  CAF.: Caffeine; NAP.: Naproxen; DIC.: Diclofenac; E3.: estriol; E2: Estradiol; EE2.: Ethinylestradiol; n.d.: non determinate

References

Caldwell DJ, Mastrocco F, Anderson PD, Lange R, Sumpter JP (2012) Predicted-no-effect concentrations for the steroid estrogens estrone, 17b-estradiol, estriol, and 17a-ethinylestradiol Environmental Toxicology and Chemistry 31(6):1396-1406, https://doi.org/10.1002/etc.1825

ECOTOX CENTRE - Swiss Centre for Applied Ecotoxicology (2021) Proposals for Acute and Chronic Quality Standards, available at http://www.ecotoxcentre.ch/expert-service/quality-standards/proposals-for-acute-and-chronic-quality-standards/. Accessed 21 February 2022.

Loos R, Marinov D, Sanseverino I, Napierska D, Lettieri T (2018) Review of the 1st Watch List under the Water Framework Directive and recommendations for the 2nd Watch List, EUR 29173 EN, Publications Office of the European Union, Luxembourg. https://doi:10.2760/614367

Zhou SB, Paolo CD, Wu XD, Shao Y, Seiler TB, Hollert H (2019) Optimization of screening-level risk assessment and priority selection of emerging pollutants – The case of pharmaceuticals in European surface waters. Environ. Int.  128:1-10. https://doi.10.1016/j.envint.2019.04.034