Annex

Table A1. The verage concentration of metals and metalloids (µg/L) in water of Ethiopian surface waters from 2005-2022.

| Study area | Concentrations of metals and metalloids (µg/L) |  |  |  |  |  |  |  |  |  | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cu | Cd | Pb | Ni | Zn | Cr | Co | Hg | As | Se |  |
| Lake Hawassa | 4.03 | 0.30 | 0.84 | 0.66 | 50.8 | 3.05 | 0.96 | 0.09 | 1.68 | 2.66 | (Abebe et al., 2022) |
| Lake Hawassa | 3 | 0.01 | 0.2 | 0.7 | 3 | 1 | 0.1 | - | 2 | 0.7 | (Kassaye et al., 2016) |
| Lake Hawassa | 35 | 1.2 | 140 | 55 | 245 | 70 | 50 | - | - | - | (Nigussie et al., 2011) |
| Lake Hawassa | - | 1.55 | 1.3 | - | - | 1.5 | - | 0.1 | 1.6 | 0.05 | (Dsikowitzky et al., 2013) |
| Lake Hawassa | - | 5 | 10 | - | 5000 | 60 | - | 1 | - | - | (Amare et al., 2014) |
| Lake Hashenge | 2 | 9 | 3 | 2 | 938 | 3 | 4 | - | - | - | (Gebrekidan et al., 2013) |
| Lake Ziway | 5 | 0.02 | 2 | 8 | 25 | 9 | 1.3 | - | 2.5 | 0.8 | (Kassaye et al., 2016) |
| Koka Reservoir | 16 | 0.04 | 5 | 22 | 48 | 28 | 5 | - | 3 | 0.6 |  |
| Koka Reservoir | - | 0.1 | 0.6 | - | - | 3 | - | 0.1 | 1.8 | 0.1 | (Dsikowitzky et al., 2013) |
| Lake Ziway | 35 | 5 | 110 | 40 | 235 | 60 | 57 | - | - | - | (Nigussie et al.,2011) |
| Lake Haiq | 1100 | 0.1 | 84 | - | 75 | - | - | - | - | - | (Teklay & Amare, 2015) |
| Lake Chamo | 740 | 910 | 210 | 840 | 40 | - | 740 |  | - | - | (Reda& Ayu, 2016) |
| Tendaho Reservoir | 530 | 0.1 | 260 | 950 | 251 | 150 | 123 | - | - | - | (Asefa & Beranu, 2016) |
| Standards for drinking water (WHO) & environmental quality (EEPA and OECD) | 2000 | 3 | 10 | 70 | 3000 | 50 | - | 6 | 10 | 10 | (WHO, 1987) |
|  | 20 | 5 | 50 | 20 | 1000 | - | - | - | - | - | (EEPA, 2003) |
|  | 20.0 | - | 2.50 | 8.0 | 70.0 | 50.0 | - | - | 50 | - | (OECD, 2007) |

Table A2. The average concentration of metals and metalloids (µg/L) in sediment of Ethiopian surface waters from 2005-2022.

| Study area | Concentration of metals and metalloids (mg/Kg dw) |  |  |  |  |  |  |  |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cu | Cd | Pb | Ni | Zn | Cr | Co | As | Se | Hg |  |
| Lake Hawassa, Ethiopia | 71.9 | 0.7 | 59.9 | 230 | 664 | 397 | 68 | 21 | 7.3 | 0.3 | (Abebe et al., 2022) |
| Lake Hawassa, Ethiopia | 5 | 0.2 | 11 | 11 | 1.5 | 13 | 6 | 4 | 0.1 | - | (Kassaye et al., 2016) |
| Lake Ziway, Ethiopia | 13 | 0.3 | 14 | 15 | 166 | 21 | 5.5 | 2.8 | 0.6 | - |  |
| Koka Reservior, Ethiopia | 37 | 0.1 | 17 | 67 | 161 | 95 | 18 | 4.8 | 0.3 | - |  |
| Lake Hawassa, Ethiopia | 34.5 | 8.5 | 19 | 17.5 | 184 | 9.5 | 28.5 | - | - | - | (Haile et al., 2015) |
| Lake Hawassa, Ethiopia | 32.5 | 6 | 14.5 | 14 | 119.5 | 4-9 | 25.5 | - | - | - | (Nigussie et al., 2011) |
| Lake Ziway, Ethiopia | 36.5 | 20 | 15.5 | 19.5 | 142.5 | 6.5 | 20 | - | - | - |  |
| Lake Hashenge, Ethiopia | 56 | 208 | 3 | 39424 | 1129 | 87 | 34 | - | - | - | (Gebrekidan et al., 2013) |
| Lake Hawassa, Ethiopia | - | 0.1 | 1.1 | - | 3.5 | 0.55 | - | - | - | 0.05 | (Amare et al., 2014) |
| Tendaho Reservoir, Ethiopia | 10 | 1 | 6 | 5 | 22 | 2.5 | 3 | - | - | - | (Asefa & Beranu, 2016) |
| TEC values | 31.6 | 0.99 | 35.8 | 22.7 | 121 | 43.4 | - | 9.8 | - | 0.18 | (MacDonald et al., 2000) |
| PEC values | 150 | 5 | 130 | 49 | 460 | 110 | - | 33 | - | 1.1 |  |

Table A3. The average concentration of metals and metalloids (mg/Kg) in edible fish species muscle of Ethiopian surface waters (where ON= O. niloticus, CG=C. gariepinus and BI= B. intermedius) from 2005-2022.

| Location |  | Metals and metalloids concentration (mg/kg dw) |  |  |  |  |  |  |  |  |  | References |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cd | Pb | Cr | Co | Ni | Cu | Zn | As | Se | Hg |  |
| Lake Hawassa, Ethiopia | ON | 0.01 | 0.03 | 1.90 | 0.06 | 0.31 | 1.18 | 29.7 | 0.37 | 1.91 | 0.08 | (Abebe et al., 2022) |
|  | CG | 0.02 | 0.03 | 3.22 | 0.34 | 0.51 | 1.47 | 30.5 | 0.22 | 1.29 | 0.29 |  |
|  | BI | 0.01 | 0.03 | 1.83 | 0.03 | 0.22 | 1.16 | 31.0 | 0.48 | 1.51 | 0.28 |  |
| Lake Hawassa, Ethiopia | ON | 0.5 | 0.15 | 0.4 | 1.5 | 0.01 | 0.17 | 8 | 1 | 10.5 | 0.7 | (Ataro et al., 2005; Dsikowitzky et al., 2013; Haile et al., 2015; Kebede & Wondimu, 2004) |
|  | CG | 0.2 | 0.45 | 2.05 | 0.16 | 0.065 | 0.05 | - | - | - | - | (Ataro et al., 2005; Desta et al., 2008; Dsikowitzky et al., 2013) |
|  | BI | 0.07 | 3.05 | 0.65 | 0.31 | 0.007 | 0.1-0.2 | - | - | - | - | (Desta et al., 2006, Dsikowitzky et al., 2013) |
| Koka Reservoir, Ethiopia | ON | 0.005 | 0.7 | 0.15 | 0.25 | 0.0008 | 0.65 | - | - | - | - | (Dsikowitzky et al., 2013) |
| Lake Hashenge, Ethiopia | ON | 0.6 | 1.24 | 0.4 | 1.61 | 0.41 | 3.4 | 25 | - | - | - | (Gebrekidan et al., 2013) |
| Lake Ziway, Ethiopia | ON | 0.7 | 0.6 | - | 1.85 | - | - | 28 | 1.0 | 31.0 | 0.7 | (Ataro et al., 2005; Kebede & Wondimu, 2004; Tadiso et al., 2011) |
| Lake Ziway, Ethiopia | CG | 0.2 | 0.3 | - | 1.7 | - | - | - | - | - | - | (Ataro et al., 2005; Tadiso et al., 2011) |
| Koka Reservoir | BI | 0.01 | 1.1 | 0.55 | 0.6 | 0.0008 | 0.56 | - | - | - | - | (Dsikowitzky et al., 2013) |
| Lake Chamo, Ethiopia | ON | 2.5 | 0.3 | - | 1.7 | 1.83 | 10.5 | 1.7 | - | - | - | (Ayeu and Reda, 2016) |
|  | CG | 2.5 | 0.5 | - | 2.2 | 1.6 | 5.4 | 0.3 | - | - | - |  |

Table A4. Estimated Daily Intake (EDI) levels, Hazard Quotient (HQ) and Maximum Edible Amount (MEA) of metals and metalloids when consuming the three commercially important fish species muscle (ON=O. niloticus, CG=C. gariepinus and BI=B. intermedius) from 2005-2022.

| Variables | EDI and HQ for mean concentration of metals and metalloids in fish muscle |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cd | Pb | Cr | Co | Ni | Cu | Zn | As | Se | Hg |
| RfD | 0.00001 | 0.0040 | 0.0009 | 0.0100 | 0.0200 | 0.0100 | 0.3000 | 0.00003 | 0.1700 | 0.0002 |
| ON | 0.7192 | 0.5033 | 0.7125 | 1.1617 | 0.5122 | 3.1800 | 18.4800 | 0.7900 | 14.4700 | 0.4933 |
| EDI | 0.0003 | 0.0002 | 0.0003 | 0.0005 | 0.0002 | 0.0014 | 0.0083 | 0.0004 | 0.0065 | 0.0002 |
| HQ | 32.3625 | 0.0566 | 0.3563 | 0.0523 | 0.0115 | 0.1431 | 0.0277 | 11.8500 | 0.0383 | 0.9652 |
| CG | 0.7300 | 0.3200 | 2.6350 | 1.1000 | 0.7250 | 2.3067 | 15.4000 | 0.2200 | 1.2900 | 0.2900 |
| EDI | 0.0003 | 0.0001 | 0.0012 | 0.0005 | 0.0003 | 0.0010 | 0.0069 | 0.0001 | 0.0006 | 0.0001 |
| HQ | 32.8500 | 0.0360 | 1.3175 | 0.0495 | 0.0163 | 0.1038 | 0.0231 | 3.3000 | 0.0034 | 0.5674 |
| BI | 0.0300 | 1.3933 | 1.0100 | 0.3133 | 0.0759 | 0.8600 | 31.0000 | 0.4800 | 1.5100 | 0.2800 |
| EDI | 0.00001 | 0.0006 | 0.0005 | 0.0001 | 0.0000 | 0.0004 | 0.0140 | 0.0002 | 0.0007 | 0.0001 |
| HQ | 1.3500 | 0.1568 | 0.5050 | 0.0141 | 0.0017 | 0.0387 | 0.0465 | 7.2000 | 0.0040 | 0.5478 |
| EDI and MEA for maximum concentration of metals and metalloids in fish muscle |  |  |  |  |  |  |  |  |  |  |  |
| ON | 2.5000 | 1.2400 | 1.9000 | 1.8500 | 1.8300 | 10.5000 | 29.7000 | 1.0000 | 31.0000 | 0.7000 |
| EDI | 0.0011 | 0.0006 | 0.0009 | 0.0008 | 0.0008 | 0.0047 | 0.0134 | 0.0005 | 0.0140 | 0.0003 |
| MEA | 0.0002 | 0.0960 | 0.0216 | 0.2400 | 0.4800 | 0.2400 | 7.2000 | 0.0007 | 4.0800 | 0.0055 |
| CG | 2.5000 | 0.5000 | 3.2200 | 2.2000 | 1.6000 | 5.4000 | 30.5000 | 0.2200 | 1.2900 | 0.2900 |
| EDI | 0.0011 | 0.0002 | 0.0014 | 0.0010 | 0.0007 | 0.0024 | 0.0137 | 0.0001 | 0.0006 | 0.0001 |
| MEA | 0.0002 | 0.0960 | 0.0216 | 0.2400 | 0.4800 | 0.2400 | 7.2000 | 0.0007 | 4.0800 | 0.0055 |
| BI | 0.0700 | 3.0500 | 1.8300 | 0.6000 | 0.2200 | 1.1600 | 31.0000 | 0.4800 | 1.5100 | 0.2800 |
| EDI | 0.00003 | 0.0014 | 0.0008 | 0.0003 | 0.0001 | 0.0005 | 0.0140 | 0.0002 | 0.0007 | 0.0001 |
| MEA | 0.0086 | 3.4286 | 0.7714 | 8.5714 | 17.1429 | 8.5714 | 257.1429 | 0.0257 | 145.7143 | 0.1971 |

Figures

Figure A1. The average concentration of metals and metalloids in water (µg/L) and sediment (mg/Kg dw) in Ethiopian surface waters.

Figure A2. The average concentration of metals and metalloids in three fish species muscle (mg/Kg dw) in Ethiopian surface waters.