SUPPLEMENTARY INFORMATION

Determination of subpicogram levels of airborne polycyclic aromatic hydrocarbons for personal exposure monitoring assessment

Barend L. van Drooge, Raimon M. Prats, Clara Jaén Flo, Joan O. Grimalt

Institute of Environmental Assessment and Water Research, Spanish National Research Council (IDÆA-CSIC). c/Jordi Girona 18-26, 08034. Barcelona, Spain.

Table S1. Relative uncertainties of the deuterated compounds and PAH compounds in the SRM2260a including the uncertainties (u%) of the compounds and analytical uncertainties from the micro-balance (1%) in consecutive dilution steps. The expanded error (EU%) is two times the average overall uncertainty of the calibration standard, expressed as EU% = 2 x √(udeut2 + uSRM2). The deuterated compounds that were used for the calibration curves are placed in front of the corresponding PAH compound (for example, naphthalene-D8 was used for biphenyl).

| Calibration curve | u% | Compound | 0.5 pg | 2.5 pg | 5 pg | 50 pg | 500 pg | EU% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Naphthalene-D8 | 1.5 | Biphenyl | 2.8 | 2.8 | 2.7 | 2.6 | 2.5 | 6.2 |
| Naphthalene-D8 | 1.5 | Naphthalene | 3.0 | 2.9 | 2.9 | 2.8 | 2.7 | 6.4 |
| Acenaphthylene-D8 | 1.2 | Acenaphthylene | 2.9 | 2.9 | 2.8 | 2.7 | 2.6 | 6.1 |
| Acenaphthene-D10 | 1.1 | Acenaphthene | 2.7 | 2.6 | 2.6 | 2.5 | 2.4 | 5.6 |
| Fluorene-D10 | 1.2 | Fluorene | 2.8 | 2.7 | 2.7 | 2.6 | 2.4 | 5.8 |
| Phenanthrene-D10 | 1.2 | Dibenzothiophene | 3.4 | 3.3 | 3.3 | 3.2 | 3.1 | 6.9 |
| Phenanthrene-D10 | 1.2 | Phenanthrene | 2.5 | 2.4 | 2.4 | 2.3 | 2.2 | 5.3 |
| Anthracene-D10 | 1.2 | Anthracene | 2.5 | 2.5 | 2.4 | 2.3 | 2.2 | 5.3 |
| Phenanthrene-D10 | 1.2 | 4H-Cyclopenta[def]phenanthrene | 3.5 | 3.5 | 3.4 | 3.4 | 3.3 | 7.2 |
| Phenanthrene-D10 | 1.2 | Fluoranthene | 2.5 | 2.4 | 2.4 | 2.3 | 2.1 | 5.3 |
| Pyrene-D10 | 1.3 | Pyrene | 2.6 | 2.5 | 2.4 | 2.3 | 2.2 | 5.5 |
| Benz[a]anthracene-D12 | 1.6 | Cyclopenta[cd]pyrene | 2.7 | 2.6 | 2.6 | 2.5 | 2.4 | 6.0 |
| Benz[a]anthracene-D12 | 1.6 | Benz[a]anthracene | 2.7 | 2.7 | 2.6 | 2.5 | 2.4 | 6.1 |
| Chrysene-D12 | 1.3 | Chrysene | 2.8 | 2.7 | 2.7 | 2.6 | 2.5 | 5.9 |
| Chrysene-D12 | 1.3 | Triphenylene | 3.2 | 3.1 | 3.1 | 3.0 | 2.9 | 6.6 |
| Benzo[b]fluoranthene-D12 | 1.6 | Benzo[b]fluoranthene | 2.7 | 2.7 | 2.6 | 2.5 | 2.4 | 6.1 |
| Benzo[k]fluoranthene-D12 | 1.6 | Benzo[j+k]fluoranthene | 2.7 | 2.6 | 2.6 | 2.5 | 2.4 | 6.0 |
| Benzo[k]fluoranthene-D12 | 1.6 | Benzo[a]fluoranthene | 2.4 | 2.3 | 2.2 | 2.1 | 2.0 | 5.4 |
| Benzo[a]pyrene-D12 | 1.6 | Benzo[e]pyrene | 2.7 | 2.7 | 2.6 | 2.5 | 2.4 | 6.1 |
| Benzo[a]pyrene-D12 | 1.6 | Benzo[a]pyrene | 3.0 | 2.9 | 2.9 | 2.8 | 2.7 | 6.5 |
| Benzo[a]pyrene-D12 | 1.6 | Perylene | 2.7 | 2.6 | 2.6 | 2.5 | 2.4 | 6.1 |
| Indeno[1,2,3-cd]pyrene-D12 | 1.6 | Indeno[1,2,3-cd]pyrene | 2.7 | 2.6 | 2.6 | 2.5 | 2.3 | 6.0 |
| Dibenzo[a,h]anthracene-D14 | 1.1 | Dibenzo[a,h]anthracene | 2.5 | 2.4 | 2.3 | 2.2 | 2.1 | 5.1 |
| Benzo[ghi]perylene-D12 | 1.2 | Benzo[ghi]perylene | 2.5 | 2.4 | 2.4 | 2.3 | 2.2 | 5.3 |
| Benzo[ghi]perylene-D12 | 1.2 | Coronene | 2.5 | 2.5 | 2.4 | 2.3 | 2.2 | 5.3 |

Table S2. Relative uncertainty of the calibration curve was calculated from two analyses that were separated by one month, based on a one-to-one comparison of the five standard concentrations that were injected in GC-Orbitrap-MS. nd = not detected.

| Compound | 0.5 pg | 2.5 pg | 5 pg | 50 pg | 500 pg | EU% |
| --- | --- | --- | --- | --- | --- | --- |
| Biphenyl | nd | 21.5 | 18.8 | 5.3 | 0.4 | 23.0 |
| Naphthalene | 2.2 | 0.3 | 0.0 | 1.3 | 0.6 | 1.7 |
| Acenaphthylene | 3.9 | 3.6 | 1.5 | 0.8 | 0.2 | 4.0 |
| Acenaphthene | 0.5 | 1.8 | 0.4 | 0.2 | 0.3 | 1.3 |
| Fluorene | 2.5 | 1.8 | 0.0 | 1.2 | 0.1 | 2.2 |
| Dibenzothiophene | 53.5 | 20.5 | 20.1 | 7.0 | 2.7 | 41.6 |
| Phenanthrene | 3.9 | 0.6 | 0.4 | 0.0 | 0.3 | 2.1 |
| Anthracene | 1.5 | 6.5 | 7.3 | 6.5 | 1.9 | 9.5 |
| 4H-Cyclopenta[def]phenanthrene | 0.2 | 2.2 | 3.3 | 4.0 | 3.0 | 5.1 |
| Fluoranthene | 6.5 | 1.4 | 1.7 | 0.2 | 0.5 | 4.2 |
| Pyrene | 6.3 | 2.6 | 0.3 | 1.2 | 0.2 | 4.3 |
| Cyclopenta[cd]pyrene | 12.7 | 7.4 | 5.3 | 2.8 | 1.9 | 12.0 |
| Benz[a]anthracene | 2.3 | 2.4 | 1.8 | 0.8 | 0.3 | 3.0 |
| Chrysene | 1.6 | 1.3 | 1.4 | 0.6 | 0.7 | 2.3 |
| Triphenylene | 0.8 | 0.3 | 0.9 | 1.9 | 1.6 | 2.2 |
| Benzo[b]fluoranthene | 7.7 | 4.1 | 1.4 | 1.7 | 1.7 | 6.7 |
| Benzo[j+k]fluoranthene | 5.5 | 2.4 | 3.0 | 1.2 | 0.3 | 5.0 |
| Benzo[a]fluoranthene | 28.9 | 40.6 | 31.7 | 12.2 | 7.5 | 48.4 |
| Benzo[e]pyrene | 5.2 | 3.9 | 5.4 | 2.9 | 2.6 | 8.0 |
| Benzo[a]pyrene | 3.2 | 0.0 | 2.9 | 2.3 | 2.9 | 4.5 |
| Perylene | 9.9 | 3.4 | 1.2 | 2.7 | 0.9 | 7.3 |
| Indeno[1,2,3-cd]pyrene | 28.9 | 3.1 | 1.4 | 2.6 | 1.2 | 14.9 |
| Dibenzo[a,h]anthracene | 12.1 | 4.4 | 0.9 | 1.6 | 1.2 | 8.1 |
| Benzo[ghi]perylene | 21.6 | 6.0 | 1.7 | 2.4 | 0.4 | 12.8 |
| Coronene | nd | nd | 28.9 | 20.5 | 11.9 | 40.9 |

Figure S1. Aethalometer AE51 filter strip and sample punch fraction (Ø = 3.4 mm; 0.144 m3) in analytical vial.

Figure S2. Calibration curve of benzo[a]pyrene of the two analyses by GC-Orbitrap-MS, where the y-axis represents the area/area-D and x-axis the amount/amount-D.

Figure S3. Benzo[a]pyrene concentrations in punch samples from PM2.5 filters collected in Manlleu (semi-rural site in Spain). Triangles are GC-Orbitrap-MS measurements and black dots are measurements done by the regional authorities (Generalitat de Catalunya).