Supplementary information

A novel inverse membrane bioreactor for efficient bioconversion from methane gas to liquid methanol using a microbial gas-phase reaction

Yan-Yu Chen1, Masahito Ishikawa1, Katsutoshi Hori1,*

1 Department of Biotechnology, Graduate School of Engineering, Nagoya University, Furo-cho, Chikusa-ku, Nagoya 464-8603, Japan.

*Corresponding authors: Katsutoshi Hori

Department of Biomolecular Engineering, Graduate School of Engineering, Nagoya University, Furo-cho, Chikusa-ku, Nagoya 464-8603, Japan

Tel.: +81-52-789-3339; Fax: +81-52-789-3218

E-mail address: khori@chembio.nagoya-u.ac.jp

Additional file 1. Simulation of gaseous flow

The flow dynamics in the gas chamber were simulated by Autodesk CFD 2019. The material was set as an incompressible flow composed of 20% CH4 and 80% air at 101325 Pa and 315.15 K. The boundary conditions were a volume flow rate of 1 cm3 min-1 with a fully developed flow at the inlet and a gauge pressure of 0 Pa at the outlet. The mesh size of the model was set by automatic sizing with a minimum refinement length of 0.1 mm. The model was solved by the advection mode of ADV 1 (monotone streamline upwind) in a laminar state without heat transfer. The velocity magnitude is normalized in the range from 0 to 5 mm min-1. A velocity higher than 5 mm min-1 is included in red.