Table S1. PET-MRI sequences and acquisition parameters.

|  | Sequence |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Whole body |  |  |  | Abdominal |  |  |  |
|  | Axial T2WI(HASTE) | Axial DWI-fs | Axial T1WI DOXON | Coronal T2WI (HASTE) | Coronal T2WI-TSE-fs | Axial T2WI-TSE-fs | Axial DWI-fs | Axial T1WI-fs vibe |
| Repetition time/echo time( ms) | 1400/95 | 6400/78 | 4.04/1.24 | 2000/91 1400/96 | 3912~4854/102 | 3462.5~5239.0/86 | 5310/73, 8300/72 | 4.56/2.03 |
| Flip angle | 90° | 90° | 14° | 134°~154° | 80° | 62°~86° | 90° | 9° |
| Acquisition type, scanning sequence | 2D-SE | 2D-EP | 3D-GR | 2D-SE | 2D-SE | 2D-SE | 2D-EP | 3D-GR |
| In-plane resolution(mm) | 1.25*1.25 | 1.56*1.56 | 1.24*1.24 | 1.19*1.19~ 1.48*1.48 | 1.19*1.19 | 1.25*1.25 | 2.90*2.90 | 1.19*1.19 |
| Number of section | 132 | 132 | 120 | 35~25 | 25 | 30 | 28 | 72 |
| Section thickness(mm) | 6 | 6 | 6 | 5 | 3.5 | 5.5 | 5, 6 | 3 |
| b-Value(s/mm2) | - | 50, 800 | - | - | - | - | 50, 800 | - |
| Number of averages | 1 | 2 | 1 | 1 | 1 | 1 | 4, 3 | 1 |

T2WI, T2 weighted imaging; HASTE, half-Fourier acquisition single-shot fast spin-echo; DWI, diffusion weighted imaging, fs, fat saturation or fat suppression; TSE, turbo spin-echo; 2D, two-dimensional; SE, spin-echo; 3D, three-dimensional; GR, gradient-recalled; EP, echo-planar; vibe, volumetric interpolated breath-hold examination

Table S2. List of the Extracted Features

| Category | Features |
| --- | --- |
| Shape features N=4 | SHAPE_Sphericity SHAPE_Compacity SHAPE_Volume (mL) SHAPE_Volume (voxels) |
| Histogram features N=5 | HISTO_Skewness HISTO_Kurtosis HISTO_Entropy_log10 HISTO_Entropy_log2 HISTO_Energy |
| Texture features(N=32) |  |
| Gray Level Cooccurrence Matrix (GLCM) N=7 | GLCM_Homogeneity GLCM_Energy GLCM_Contrast GLCM_Correlation GLCM_Entropy_log10 GLCM_Entropy_log2 GLCM_Dissimilarity |
| Gray-Level Run Length Matrix (GLRLM) N=11 | GLRLM_SRE GLRLM_LRE GLRLM_LGRE GLRLM_HGRE GLRLM_SRLGE GLRLM_SRHGE GLRLM_LRLGE GLRLM_LRHGE GLRLM_GLNU GLRLM_RLNU GLRLM_RP |
| Neighborhood Gray-Level Different Matrix (NGLDM) N=3 | NGLDM_Coarseness NGLDM_Contrast NGLDM_Busyness |
| Gray-Level Zone Length Matrix (GLZLM) N=11 | GLZLM_SZE (Short-Zone Emphasis) GLZLM_LZE (Long-Zone Emphasis) GLZLM_LGZE (Low Gray-level Zone Emphasis) GLZLM_HGZE (High Gray-level Zone Emphasis) GLZLM_SZLGE (Short-Zone Low Gray-level Emphasis) GLZLM_SZHGE (Short-Zone High Gray-level Emphasis) GLZLM_LZLGE (Long-Zone Low Gray-level Emphasis) GLZLM_LZHGE (Long-Zone High Gray-level Emphasis) GLZLM_GLNUz (Gray-Level Non-Uniformity for zone) GLZLM_ZLNU (Zone Length Non-Uniformity) GLZLM_ZP (Zone Percentage) |
| Conventional metabolic parameters in PET N=5 | SUVmaximum SUVmean SUVstandarddeviation SUVpeaksphere TLG |

SRE: Short-Run Emphasis, LRE: Long-Run Emphasis, LGRE: Low Gray-level Run Emphasis, HGRE: High Gray-level Run Emphasis, SRLGE: Short-Run Low Gray-level Emphasis, SRHGE: Short-Run High Gray-level Emphasis, LRLGE: Long-Run Low Gray-level Emphasis, LRHGE: Long-Run High Gray-level Emphasis, GLNU: Gray-Level Non-Uniformity for run, RLNU: Run Length Non-Uniformity

RP: Run Percentage, SZE: Short-Zone Emphasis, LZE: Long-Zone Emphasis, LGZE: Low Gray-level Zone Emphasis, HGZE: High Gray-level Zone Emphasis, SZLGE: Short-Zone Low Gray-level Emphasis, SZHGE: Short-Zone High Gray-level Emphasis, LZLGE: Long-Zone Low Gray-level Emphasis, LZHGE: Long-Zone High Gray-level Emphasis, GLNU: Gray-Level Non-Uniformity for zone, ZLNU: Zone Length Non-Uniformity, ZP: Zone Percentage

Fig. S1. The receiver operator characteristic (ROC) curves for the radiomics nomogram in the independent group.