Appendix 
Pulse Study Flow Chart

22 volunteers (NHS staff)

Eligibility Criteria

Patient information leaflet and consent form signed

Lying flat in hospital bed in training room of ICU

Baseline HR and BP

Thigh tourniquet on but not inflated

Pulse oximeters on 2nd and 3rd  toe of both feet

Screenshot of waveform at 15, 30, 60 and 90 seconds

Leave for 90 seconds to allow the waveform to equilibrate

Screenshot of waveform 15 seconds (timepoint 0, Baseline) before thigh tourniquet inflation

Tourniquet then put up at 300mmHg for 90 seconds

Screenshot at 30, 60, 90 seconds (T 30, T 60 (timepoint 1, inflation), T 90 timepoint 2, just prior to deflation)) after the tourniquet has been inflated.

Let tourniquet down

Screenshot 60 seconds after the tourniquet has gone down. Timepoint 3, after deflation

At the end download data (Pulse Rate, Saturations, and Perfusion Index) for the whole time period from the central workstation in ICU – including print off of the pulse oximeter trace, upload/email screenshots to trust email account

Then do the same for the upper limb – pressure set to 250mmHg

Table 6: Linear mixed models’ results for legs’ data

|  | Control | Experimental | Difference‡ (95% CI), p-value |
| --- | --- | --- | --- |
| Phillips data ˤ |  |  |  |
| Baseline | 2.29 (1.44-3.14)† | 1.60 (0.74-2.45)† | -0.70 (-1.36 to -0.03), 0.042 |
| Deflation | 2.78 (1.93-3.63)† | 2.29 (1.44-3.14)† | -0.49 (-1.15 to 0.18), 0.152 |
| Difference§ (95% CI), p-value | 0.49 (-0.18 to 1.16), 0.148 | 0.70 (0.03 to 1.37), 0.041 |  |
| Masimo data ˤ |  |  |  |
| Baseline | 1.53 (0.82-2.25)† | 1.68 (0.97-2.40)† | 0.15 (-0.47 to 0.78), 0.624 |
| Deflation | 2.04 (1.32-2.75)† | 3.04 (2.33-3.76)† | 1.01 (0.38 to 1.63), 0.002 |
| Difference§ (95% CI), p-value | 0.51 (-0.12 to 1.13), 0.110 | 1.36 (0.74 to 1.98), <0.001 |  |

CI = Confidence interval; ˤ Separate models; † Mean (95% CI); ‡ Experimental - Control; § Deflation - Baseline

Table 7: Comparing Phillips and Masimo arms’ data when there is no inflation

| Time point | Group | Mean difference† (95% CI), p-value | Pearson’s correlation (95% CI) | ICC |
| --- | --- | --- | --- | --- |
| Baseline | Experimental Control | -0.58 (-1.57, 0.41), 0.237 -0.60 (-1.53, 0.32), 0.189 | 0.74 (0.42, 0.90) 0.72 (0.50, 0.87) | 0.72 0.71 |
| Inflation (Pulse trace lost) | Control | -0.41 (-1.38, 0.55), 0.380 | 0.76 (0.54, 0.91) | 0.76 |
| Inflation (minimum value) | Control | -0.21 (-0.97, 0.56), 0.584 | 0.81 (0.63, 0.93) | 0.82 |
| Deflation | Experimental Control | -0.23 (-1.41, 0.96), 0.695 -0.24 (-1.59, 1.12), 0.722 | 0.61 (0.33, 0.79) 0.53 (0.32, 0.82) | 0.61 0.54 |

†Phillips – Masimo; CI = Confidence interval; ICC = Intraclass correlation coefficient

Table 8: Comparing Phillips and Massimo arms’ data during inflation

|  | Spearman’s correlation (95% CI) | Kendall’s Tau (τ) (95% CI) | p-value † |
| --- | --- | --- | --- |
| During inflation (Pulse trace lost) | 0.16 (-0.24, 0.49) | 0.08 (-0.17, 0.31) | 0.017 |
| During inflation (Minimum value) | 0.63 (0.28, 0.84) | 0.53 (0.24, 0.75) | 0.346 |

† Wilcoxon signed-rank test; CI = Confidence interval

Table 9: Comparing Phillips and Massimo legs’ data when there is no inflation

| Time point | Group | Mean difference† (95% CI), p-value | Pearson correlation (95% CI) | ICC |
| --- | --- | --- | --- | --- |
| Baseline | Experimental Control | -0.09 (-0.77, 0.60), 0.788 0.76 (0.12, 1.40), 0.023 | 0.21 (-0.40, 0.75) 0.79 (0.20, 0.94) | 0.23 0.63 |
| Inflation (Pulse trace lost) | Control | 0.59 (0.01, 1.16), 0.047 | 0.82 (0.66, 0.93) | 0.78 |
| Inflation (minimum value) | Control | 0.41 (-0.25, 1.07), 0.208 | 0.81 (0.59, 0.90) | 0.76 |
| Deflation | Experimental Control | -0.75 (-1.81, 0.31), 0.157 0.74 (0.04, 1.45), 0.039 | 0.23 (-0.57, 0.70) 0.77 (0.45, 0.91) | 0.20 0.69 |

†Phillips – Masimo; CI = Confidence interval; ICC = Intraclass correlation coefficient

Table 10: Comparing Phillips and Massimo legs’ data during inflation

|  | Spearman’s correlation (95% CI) | Kendall’s Tau (τ) (95% CI) | p-value † |
| --- | --- | --- | --- |
| During inflation (Pulse trace lost) | 0.07 (-0.37, 0.53) | 0.05 (-0.39, 0.49) | 0.527 |
| During inflation (Minimum value) | 0.18 (-0.22, 0.74) | 0.24 (-0.19, 0.62) | 0.243 |

† Wilcoxon signed-rank test; CI = Confidence interval