Supplementary Material

Instrument

Triple quadrupole ultra performance chromatography-mass spectrometry(Waters, Acquity Xevo H/TQ-S, USA); Automated solid phase extraction(SPE, Shimadzu, QUA Trace ASPE799, Japan); High-speed freezing centrifuge(Beckman, Avanti J-E, USA); Ultrasonic cleaning machine(Scientz, SB-5200DTDN, China); Electronic balance(Mettler-Toledo, AL104, Swiss).

Preparation of standard solution and samples

Standard solution

The 12 kinds of chiral pesticide racemic standards were prepared with acetonitrile to mixed standard solutions who has a concentration of 1mg/L and stored in a refrigerator at -20 ºC. Their isotope internal standards were subjected to the same treatment as above.

Surface water samples

10 μL of the isotope internal standard mixture (1 mg/L) was added to 500 mL water, and then filtered through a 0.45 μm glass filter paper after thorough mixing. The target compounds in the samples were extracted using solid phase extraction (SPE) with Agilent SampliQ Poly-DVB 6 mL/150 mg cartridge. Before sample inputting, The DVB column was activated with 5 mL acetonitrile and 5 mL ultrapure water. Then, the water sample was passed through the DVB column at a flow rate of 7-8 mL/min. After inputting, the DVB column was washed with 5mL of ultrapure water and dried for 3min. The target compound was eluted twice with 3 mL acetonitrile at a flow rate of about 1mL/min each time. The eluant was collected, blown to near dryness with nitrogen, re-dissolved in methanol to 1 mL, passed through a 0.22 μm filter membrane, and then tested on the machine.

Surface sediment samples

A total of 5.00 g of the treated sample was placed into a 50 mL centrifuge tube, added with 10 mL acetonitrile, 100 μL acetic acid and 2 mL ultrapure water, sonicated for 15 min, and added with 10 μL isotope internal standard mixture with a concentration of 1 mg/L. The centrifuge tube was added with 1 g NaCl and 2 g anhydrous MgSO4, after being vortexed for 1 min and centrifuged at 6000 rpm for 10 min, its supernatant was transferred to a new 10 mL centrifuge tube. 200 mg MgSO4, 50 mg PSA, 20 mg C18 and 20 mg neutral activated alumina were added to the centrifuge tube containing supernatant. Vortexed for 1 min and centrifuged at 6000 rpm for 10 min, the supernatant was removed to a 10 mL tube. The supernatant in the tube was blown to near dryness with nitrogen, re-dissolved in methanol to 1 mL, passed through a 0.22 μm filter membrane, and then tested on the machine.

Table S1. Mass spectrometry parameters of four chiral pesticides

| Enantiomer | Parent ion (M/Z) | Daughter ion*/Daughter ion (M/Z) | Cone voltage (V) | Collisional energy (V) |  |
| --- | --- | --- | --- | --- | --- |
| Napropamide-S | 272.200 | 129.30*/171.30 | 24 | 16/19 |
| Napropamide-R |  |  |  |  |
| Metalaxyl-S | 280.200 | 192.30*/220.00 | 32 | 17/12 |
| Metalaxyl-R |  |  |  |  |
| Myclobutanil-S | 289.122 | 69.99*/124.95 | 40 | 16/32 |
| Myclobutanil-R |  |  |  |  |
| Triadimefon-S | 294.100 | 69.30*/197.20 | 31 | 20/15 |
| Triadimefon-R |  |  |  |  |
| Paclobutrazol-S | 294.100 | 70.20*/125.10 | 36 | 20/38 |
| Paclobutrazol-R |  |  |  |  |
| Flutriafol-S | 302.110 | 70.20*/233.01 | 32 | 18/29 |
| Flutriafol-R |  |  |  |  |
| Tebuconazole-S | 308.153 | 69.97*/124.94 | 40 | 20/36 |
| Tebuconazole-R |  |  |  |  |
| Hexaconazole-S | 314.200 | 69.96*/158.89 | 45 | 18/30 |
| Hexaconazole-R |  |  |  |  |
| Triticonazole-S | 318.100 | 70.20*/125.00 | 30 | 16/35 |
| Triticonazole-R |  |  |  |  |
| Metconazole-1 | 320.200 | 70.40*/125.02 | 38 | 22/36 |
| Metconazole-2 |  |  |  |  |
| Metconazole-3 |  |  |  |  |
| Metconazole-4 |  |  |  |  |
| Benalaxyl-S | 326.200 | 91.00*/148.05 | 28 | 35/21 |
| Benalaxyl-R |  |  |  |  |
| Bitertanol-1 | 338.100 | 70.03*/99.10 | 21 | 8/16 |
| Bitertanol-2 |  |  |  |  |

Note: ions with * are also used for quantification.

Table S2. Gradient elution parameters of chiral pesticides

| Time(min) | Flow(mL/min) | %A(aqueous phase) | %B(organic phase) |
| --- | --- | --- | --- |
| Initial | 0.3 | 95 | 5 |
| 21 | 0.3 | 5 | 95 |
| 22 | 0.3 | 95 | 5 |
| 30 | 0.3 | 95 | 5 |

Fig.2 Separation chromatogram of chiral pesticide

QA/QC:

In order to avoid background contamination, one sample blank and procedure blank were added for every 10 samples. The 12 chiral pesticides were diluted into 1 μg/L, 5 μg/L, 10 μg/L, 25 μg/L, 50 μg/L and 100 μg/L and added to 10 μL of the isotopic internal standard mixture with a concentration of 1 mg/L for quantitative detection, and then the standard curves were plotted. The mixed standard solutions with concentrations of 0.1, 1, 10 and 100 μg/L were injected 7 times consecutively, and the mean values of responses and their relative standard deviations of the instrument were calculated. The detection limit of the instrument and the quantification limit of the method were calculated according to proper equations.

Table S3. IDL, MQL and Rr of chiral pesticides in surface waters

| Enantiomer | IDL(μg/L) | Rr(%) | MQL(ng/L) |
| --- | --- | --- | --- |
| R-Napropamide | 0.0250 | 118.525 | 0.1266 |
| S-Napropamide | 0.0253 | 112.600 | 0.1347 |
| S-Metalaxyl | 0.0167 | 101.150 | 0.0991 |
| R-Metalaxyl | 0.0215 | 98.975 | 0.1304 |
| R-Myclobutanil | 0.0191 | 100.000 | 0.1145 |
| S-Myclobutanil | 0.0124 | 100.100 | 0.0746 |
| (-)-R-Triadimefon | 0.0082 | 99.300 | 0.0494 |
| (+)-S-Triadimefon | 0.0125 | 101.350 | 0.0737 |
| S-Paclobutrazol | 0.0143 | 92.025 | 0.0933 |
| R-Paclobutrazol | 0.0113 | 92.525 | 0.0735 |
| S-Flutriafol | 0.0158 | 91.500 | 0.1035 |
| R-Flutriafol | 0.0159 | 91.600 | 0.1041 |
| (+)-S-Tebuconazole | 0.0118 | 99.100 | 0.0715 |
| (-)-R-Tebuconazole | 0.0130 | 99.075 | 0.0786 |
| (+)-Hexaconazole | 0.0102 | 100.300 | 0.0609 |
| (-)-Hexaconazole | 0.0097 | 97.675 | 0.0594 |
| Metconazole-1 | 0.0171 | 106.175 | 0.0964 |
| Metconazole-2 | 0.0376 | 103.475 | 0.2181 |
| Metconazole-3 | 0.0366 | 99.025 | 0.2215 |
| Metconazole-4 | 0.0186 | 104.250 | 0.1069 |
| R-Triticonazole | 0.0139 | 100.675 | 0.0827 |
| S-Triticonazole | 0.0128 | 98.500 | 0.0782 |
| (-)-R-Benalaxyl | 0.0228 | 103.350 | 0.1322 |
| (+)-S-Benalaxyl | 0.0173 | 98.750 | 0.1053 |
| Bitertanol-1 | 0.0626 | 100.150 | 0.3752 |
| Bitertanol-2 | 0.0392 | 99.425 | 0.2363 |

5 g pretreated control sample was blown dry with nitrogen, then re-dissolved with methanol and passed through 0.22 μm filter membrane, and the concentration was noted as C0 (blank sample concentration) in the test; 5 g control sample was pretreated, and was blown dry with nitrogen, and 10 μL of mixed and internal standard with a concentration of 1 mg/L were added, then fixed to 1mL with methanol, and the concentration was C1 in the test; Without adding sample matrix in the pretreatment, just blown dry with nitrogen, and 10 μL of mixed and internal standard with a concentration of 1 mg/L were added, then fixed to 1mL with methanol, and the tested concentration on the machine was C2.

Table S4. Rr, IDL, MQL and ME of chiral pesticides in surface substrates

| Enantiomer | Rr(%) | IDL(μg/L) | MQL(ng/kg) | ME(%) |
| --- | --- | --- | --- | --- |
| R-Napropamide | 83.250 | 0.0250 | 18.0310 | 96.20 |
| S-Napropamide | 86.225 | 0.0253 | 17.5907 | 95.54 |
| S-Metalaxyl | 82.075 | 0.0167 | 12.2146 | 92.52 |
| R-Metalaxyl | 79.425 | 0.0215 | 16.2515 | 94.09 |
| R-Myclobutanil | 103.975 | 0.0191 | 11.0097 | 98.53 |
| S-Myclobutanil | 102.150 | 0.0124 | 7.3105 | 100.54 |
| (-)-R-Triadimefon | 95.400 | 0.0082 | 5.1433 | 95.40 |
| (+)-S-Triadimefon | 100.600 | 0.0125 | 7.4267 | 99.18 |
| S-Paclobutrazol | 85.825 | 0.0143 | 10.0031 | 98.86 |
| R-Paclobutrazol | 106.350 | 0.0113 | 6.3973 | 99.63 |
| S-Flutriafol | 78.875 | 0.0158 | 12.0088 | 101.04 |
| R-Flutriafol | 111.350 | 0.0159 | 8.5605 | 101.85 |
| (+)-S-Tebuconazole | 101.975 | 0.0118 | 6.9468 | 99.14 |
| (-)-R-Tebuconazole | 99.725 | 0.0130 | 7.8072 | 100.54 |
| (+)-Hexaconazole | 103.825 | 0.0102 | 5.8855 | 97.99 |
| (-)-Hexaconazole | 102.250 | 0.0097 | 5.6744 | 102.08 |
| Metconazole-1 | 100.675 | 0.0171 | 10.1687 | 103.30 |
| Metconazole-2 | 118.850 | 0.0376 | 18.9864 | 107.24 |
| Metconazole-3 | 106.825 | 0.0366 | 20.5369 | 101.84 |
| Metconazole-4 | 102.150 | 0.0186 | 10.9084 | 99.50 |
| R-Triticonazole | 104.675 | 0.0139 | 7.9515 | 100.76 |
| S-Triticonazole | 104.150 | 0.0128 | 7.3941 | 101.66 |
| (-)-R-Benalaxyl | 88.600 | 0.0228 | 15.4224 | 92.55 |
| (+)-S-Benalaxyl | 95.150 | 0.0173 | 10.9301 | 96.57 |
| Bitertanol-1 | 98.700 | 0.0626 | 38.0668 | 102.22 |
| Bitertanol-2 | 106.700 | 0.0392 | 22.0180 | 103.58 |