Equipment and specific operation process of treatments/ interventions

Instruments and equipment used for the intervention

- HFNC: Model: PT101AZ, trade name and manufacturer AIRVO2, Fisher & Paykel Healthcare, Auckland, New Zealand;

- Respiratory training: positive expiratory pressure，PEP. Model and manufacturer: Acapella choice，Smith medical，American;

- High-flow line and connector model: Heating breathing line 900PT561, nasal plug catheter OPT942 (small), OPT944 (medium), OPT946 (Large).

Specific operation process of the intervention measures

High flow humidification oxygen therapy was administered using a transnasal high flow oxygen therapy humidifier. Respiratory training was performed using PEP Acapella. The patients for respiratory training are instructed by a respiratory therapist / physician. The specific steps of routine oxygen therapy and the operation procedures and videos of respiratory training have been produced. Before the start of the study, it was confirmed that the bed nurses and respiratory therapists reviewed the oxygen therapy and respiratory training videos to confirm that they were competent for the work. During the research process, the bed nurses and respiratory therapists who perform oxygen therapy and breathing training operations should be trained if necessary. Relevant information and training videos will be saved. The implementation of the intervention will be recorded in the Case Report Form.

HFNC therapy: Initial parameters: Flow 50L/min, FiO2 100%, T 37.0℃. Subsequently FiO2  will be down-regulated to the lowest value and SPO2 is maintained above 95%. The oxygen flow rate is at least 50L/min in the first 24 hours, and can be adjust to 60L/min as needed, then gradually reduce the FiO2 to maintain SPO2≥ 95%. The oxygen flow can be reduced if patients feel uncomfortable with the treatment. COT by venturi mask will be used only for significant discomfort or skin damage during nasal cannula until HFNC is tolerated again. High-flow oxygen therapy will last for at least 48 hours, then switch to COT. If the patients are transferred to the ICU after surgery, moving out of the ICU can be considered when FiO2 ≤ 35% and the SPO2 can be maintained at≥ 95%.

COT: COT is transfered oxygen through a nasal catheter, mask (with or without an oxygen storage bag, Venturi system), the oxygen flow is set to maintain SPO2≥ 95%. Patients who failed COT and without intubation will be treated with high-flow oxygen therapy with parameters as above. If the patients are transferred to the ICU after surgery, moving out of the ICU can be considered when the the oxygen flow is <6L / min and SPO2 ≥ 95%.

Once acute hypoxic respiratory failure occurred, and HFNC or NIV could not improve within two hours, endotracheal intubation and invasive ventilation should be considered. Other rescue measures shall be treated according to the routine diagnosis and treatment.

PEP Acapella (choice) vibration positive pressure ventilation treatment system operation steps are as following (Patients in the seated or semi-recumbent position):

- On first use, turn the frequency adjustment dial to the lowest frequency resistance setting marked "1";

- Put the nozzle in the mouth, instruct the patient to relax, and then take a deep breath (total lung capacity is not required);

- Hold breath for 2~3 seconds;

- Exhale slowly, making sure to keep the mouthpiece sealed as you exhale (the exhale time is about 3-4 times longer than the inhale time, no need to exhale forcefully to the "FRC");

- Adjust the dial to increase the resistance (adjust clockwise to increase the resistance of the vibrating hole, allowing the patient to exhale at a lower flow rate);

- Determin the scope;

- Repeat 10-20 PEP breaths, followed by 2-3 "puff" coughs as needed to cough up secretions;

- Repeat the above steps 4~8 times, no more than 20min.

Experimental method

Introduce the clinical trial purpose, background and related content, and enroll by randomization number.

Intervention measures: The combined treatment group received HFNC + respiratory training, and the conventional treatment group received oxygen by nasal catheter or mask. Start receiving oxygen therapy as described above after postoperative transfer out of the recovery room or transfer to ICU within 30 minutes after effusion.

Duration of the intervention measures: HFNC or COT lasted for more than 48h, adjusted for nasal catheter oxygen administration or cessation of oxygen therapy according to the patient's condition, and PEP acapella choice was continued for at least 3 days, three times a day for no more than 20 minutes each time.

Follow-up observation: daily visit within 1 week after surgery, hospitalization process follow-up according to the situation, telephone visit for survival status at 28 and 90 days after surgery.

Duration of the clinical trial and reasons for determination: The entire trial and follow-up time of this study is not long. With reference to the case data of each center, the trial is expected to last for 6 years, which can ensure that there are enough candidates for the trial.