ELECTRONIC SUPPLEMENTARY MATERIALS (ESM)

PROPOSITION AND APPLICATION OF A MULTI-CRITERIA FRAMEWORK ASSOCIATED WITH CONSEQUENTIAL LIFE CYCLE ASSESSMENT FOR SELECT GREENER AND SUSTAINABLE (BIO)FUELS: A CASE STUDY IN BRAZIL

Table S1 – State-of-the-art of the relation among LCT, MCDA, and Fuzzy Logic as a decision-maker support approach.

| AUTHORS | PAPER DESCRIPTION | DECISION-MAKER SUPPORT |
| --- | --- | --- |
| (Geldermann et al. 2000) | HIGHLIGHTS: An approach that integrates fuzzy algebra into a Multi-Criteria Decision Making (MCDM) algorithm, called PROMETHEE, to support decision-makers on environmental assessments via the notion of weak preference and incomparability. In the final discussion of the political and industrial decision-makers, the “best available techniques” (BAT) can be determined. The limited data availability due to the varying measurements restricts the reliability of the obtained results. The subjective character of the set of weights and the preference parameters should be considered. So, it becomes evident that this approach only supports the decisions, which is left to the decision-makers themselves. APPLICATION AREA: Life Cycle Impact Assessment, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: State-of-the-art. STUDY CASE: YES (Sinter techniques in the iron and steel making industry). LCT APPROACH: Attributional. | - Defuzzied preference flows: Nodes with the defuzzied flows relations depicted as arcs showing the partial precedence order of the alternatives. |
| (Weckenmann and Schwan 2001) | HIGHLIGHTS: Use of Fuzzy membership functions as a tool to improve the interpretations of ecological evaluations, like LCA, through a single score. The single score resulted from the fuzzy arithmetic is called “Green Fitness” and only makes sense be used when two or more products are compared. APPLICATION AREA: Life Cycle Impact Assessment, Characterization, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Characterization factors. STUDY CASE: YES (Wave soldering process of an exemplary printed circuit board TX). LCT APPROACH: Attributional. | 1) Dominance analysis chart: Horizontal bar chart with the Green Fitness rate. |
| (Ardente et al. 2004) | HIGHLIGHTS: (Re)Calculates the Life Cycle Inventory (LCI) of products using the software FALCADE (Fuzzy Approach to Life Cycle Analysis and Decision Environment). The software uses the fuzzy representation to manage the complex relationships that arise in the aggregation of a single environmental score. In particular, it allows giving further attributes to the LCI numerical data based on expert knowledge. The uncertainty of data is defined via four fuzzy linguistic variables: temporal, technologic, statistical, and geographic representativeness of the data. This approach allows users to transparently investigate how the data uncertainties affect the final results, i.e., the results and every equation and matrix used to generate it. APPLICATION AREA: Life Cycle Inventory Analysis, Uncertainties, Probabilistic techniques. FUZZY WEIGHTS DEFINITION: Experts opinions. STUDY CASE: YES (Plaster material commonly used in building construction). LCT APPROACH: Attributional. | 1) Decision surfaces: Tridimensional surfaces comparing the fuzzy linguistic variables (uncertainties attributes). 2) Fuzzy Eco-profile table: Defuzzified LCI with the respective flow’s uncertainties. |
| (Güereca et al. 2007) | HIGHLIGHTS: Establishes a valuation method proposed to generate partial environmental impact indicators, calculated based on LCA results, emissions inventory, and the political targets and sustainability thresholds for a given area. The partial indicator obtained is transformed into a fuzzy linguistic descriptor, which permits the construction of a preference order amongst a series of alternatives. It considers a semantic scale with eleven terms, which permits the gradual definition of the performance of alternatives according to their level of pertinency. APPLICATION AREA: Life Cycle Impact Assessment, Uncertainties, Subjectivity, Normalization, Weighting, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Equations. STUDY CASE: YES (Biowaste management system scenarios.) LCT APPROACH: Attributional. | 1) Fuzzy measurements and values tables: Fuzzy measurements and values of the decision function obtained for the semantic term. |
| (Benetto et al. 2008) | HIGHLIGHTS: Propose a modified version of the fuzzy multicriteria method named NAIADE, fitting it better to the LCA requirements and including comprehensive uncertainty analysis capabilities. The integration of uncertainty results can focus on critical LCA issues, enabling an association of dominance, contribution, and gravity analysis with the LCA results, especially when the rankings are not entirely consistent. The new NAIADE can support engineers and scientists working in LCA discipline, and managers and decision-makers faced with problems based on LCA subjectivity. APPLICATION AREA: Life Cycle Impact Assessment, Uncertainties, Probabilistic techniques. FUZZY WEIGHTS DEFINITION: Decision-makers opinions. STUDY CASE: YES (Electricity production scenarios). LCT APPROACH: Attributional. | Not applicable |
| (Tan 2008) | HIGHLIGHTS: Uses fuzzy numbers to propagate uncertainties on the matrix-based LCI model, thereby allowing the fuzzy distributions of I/O flows to be determined from the fuzzy distributions of parametric uncertainties in the model inputs. In the same manner, as Monte Carlo analysis, the use of fuzzy numbers intended to determine the aggregate effect of multiple uncertain inputs data and not the individual effect of each parameter on the LCI results. Fuzzy uncertainty calculations have the advantage of requiring fewer runs than a Monte Carlo approach. It´s possible to use 11 runs and create a reasonably good picture of the output uncertainty (in the Monte Carlo approach, less than 1000 runs are unacceptable). APPLICATION AREA: Life Cycle Inventory Analysis, Uncertainties, Probabilistic techniques. FUZZY WEIGHTS DEFINITION: Equations. STUDY CASE: YES (Simple systems with variability in industrial efficiency and emissions data; Life cycle systems with co-products, and thus requires the inclusion of displaced processes; Comparing different carbon sequestration technologies.). LCT APPROACH: Attributional. | 1) Fuzzy LCI Table: Defuzzified LCI with the respective flow’s uncertainties. |
| (Heijungs and Tan 2010) | HIGHLIGHTS: Mathematically validates and discusses the Tan (2008) assumption: “Lower values in the technology matrix propagate as upper values of the LCI result and vice-versa.” The authors integrated fuzzy arithmetic into matrix-based LCI using decomposition into α-cut intervals (runs), providing a better reflection of epistemological uncertainties inherent to I/O data. Thus, the arguments presented here provide a rigorous proof that the algorithm developed for fuzzy matrix-based LCI is valid under specified conditions (the inverse of the technology matrix is non-negative). This conclusion gives the conditions to corroborate that the propagation of uncertainties in fuzzy LCI models is computationally efficient and strictly valid. APPLICATION AREA: Life Cycle Inventory Analysis, Uncertainties, Probabilistic techniques. FUZZY WEIGHTS DEFINITION: Not applicable STUDY CASE: Not applicable. LCT APPROACH: Attributional. | Not applicable |
| (Clavreul et al. 2013) | HIGHLIGHTS: Assess probability and possibility theories to represent the stochastic and epistemic uncertainties consistently, applying it to LCA. The combination used the Monte Carlo analysis (probability distribution) and alpha-cuts optimized for fuzzy intervals. The proposed method (Independent Random Set, IRS) generalizes random sampling to probability distributions and fuzzy intervals, thus making simultaneous use of both representations possible. While the probabilistic method of Monte Carlo generates a single probability distribution, the IRS method yields a family of probability distributions bounded by upper and lower distributions. The IRS method offers the advantage of putting the focus on the information rather than deciding a priori of how to represent it. If the information is abundant, then a purely statistical representation mode is adequate, but if the information is scarce, it may be better conveyed by possibility. APPLICATION AREA: Life Cycle Impact Assessment, Uncertainties, Probabilistic techniques. FUZZY WEIGHTS DEFINITION: Experts opinions STUDY CASE: YES (Willow cultivation for bioenergy production through co-firing in large-scale combined heat and power (CHP) plants.). LCT APPROACH: Attributional. | 1) Cumulative probability chart: Cumulative distribution functions of an environmental category (study-case: greenhouse gas emissions) obtained with three uncertainty propagation methods: - Monte Carlo, - Fuzzy and, - IRS method. |
| (Chan et al. 2014) | HIGHLIGHTS: Introduces a comprehensive method that integrates the Environmental Management Accounting (EMA) and Life Cycle Assessment (midpoint categories) concepts applying fuzzy logic associated with Analytical Hierarchical Process (AHP) to evaluate the environmental and organizational performance of different product design scenarios. A screening model was proposed to help designers reduce their reliance on LCA and present a case study to demonstrate that this approach provides a systematic method of evaluating alternative designs and identifying product design improvement possibilities. The measurement approach presented in this research can help companies reduce development lead time by screening out undesirable design options. More importantly, the approach can be modeled with the mere use of an electronic spreadsheet, which means limited resources are needed to implement the proposed method. APPLICATION AREA: Life Cycle Impact Assessment, Organizational performance, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Experts opinions. STUDY CASE: YES (Personal electronic product). LCT APPROACH: Attributional. | 1) Measurement charts: Vertical bar charts with the Environmental Risk, Organizational and Aggregated performance: |
| (Agarski et al. 2016) | HIGHLIGHTS: Presents a multi-criteria fuzzy logic methodology aimed to perform both impact categories weighting and sensitivity analysis within the environmental life cycle assessment. The proposed methodology integrates the subjective factors of the decision-maker with impact categories weighting factors using a fuzzy interference system. The fuzzy interference system allows the subjective and standards weights of the impact categories can be changed, providing a new approach to model the weighting factors using fuzzy membership functions. This multicriteria analysis technique can be successfully applied to carry out weighting and sensitivity analysis in the life cycle assessment. APPLICATION AREA: Life Cycle Impact Assessment, Weighting, Sensitivity Analysis, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Experts opinions. STUDY CASE: YES (Waste treatment processes). LCT APPROACH: Attributional. | 1) Total Environmental Impact chart: Vertical aggregated bar charts with the TEI of the scenarios: |
| (Wang et al. 2016) | HIGHLIGHTS: Develop a new framework of social life cycle impact assessment (SLCIA) method based on the United Nations Environment Program/Society of Environmental Toxicology and Chemistry (UNEP/SETAC) Guidelines for analyzing the social impact in Taiwan, particularly in the electronics industry. A consistent fuzzy preference relations (CFPR) method was used to determine the weighting values of each subcategory and indicator. Nine quantitative indicators were easily organized using available social data from government statistics as performance reference points (PRPs) to determine the social impact exerted by companies. The relative weights were determined to allow for an impact assessment and thus solve the limitation of their currently assumed equal weights. APPLICATION AREA: Social Life Cycle Assessment, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Experts opinions. STUDY CASE: YES (Electronic industry). LCT APPROACH: Attributional. | 1) Fuzzy weight tables: Local and global fuzzy weights of sub-categories and indicators. |
| (Maranduba et al. 2017) | HIGHLIGHTS. Associates fuzzy logic and multicriteria analysis into a Viability Fuzzy Controller (VFC) able to incorporate technical, environmental, economic, and social inputs. With the VFC, the decision-maker has more reliable conclusions and predictions on the results uncertainties once fuzzy parameters values were based on inputs averages and environmental policies. APPLICATION AREA: Life Cycle Sustainability Assessment, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Life Cycle Impact Assessment and international market prices averages. STUDY CASE: YES (Biofuels: diesel and biodiesel). LCT APPROACH: Attributional. | 1) Viability surfaces: Tridimensional viability surfaces with pairwise comparation between the VFC inputs behavior in relation to different viability perspectives in linguistical variables [very low, low, medium, high and very high]. 2) Viability product bar: Vertical bar charts with different perspectives scores (environmental and economic) for the products assessed. |
| (Kamble et al. 2017) | HIGHLIGHTS: Develop a hybrid life cycle assessment (LCA) based fuzzy multicriteria decision making (MCDM) methodology for the evaluation and selection of appropriate municipal wastewater treatment (WTT), identifying the critical evaluation criteria in the process of selecting WTT from a sustainability perspective. Twelve criteria, including sustainability and those derived from LCA and life cycle costing, were obtained consisting of both qualitative and quantitative criteria. In the third stage, the weights of the selection criteria as the measurable indices of the WWT technologies were determined by fuzzy pairwise comparison matrices of the analytic hierarchy process. Finally, the fuzzy technique for order of preference by similarity to ideal solution technique was used for ranking the alternatives. The developed technology assessment framework enables decision-makers to understand the complete evaluation and selection process better and provide a more accurate, practical, and systematic decision support framework. The framework is superior to existing methods since it can represent vague qualitative data and present all possible results with different degrees of membership. APPLICATION AREA: Life Cycle Assessment, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Experts opinions. STUDY CASE: YES (Municipal wastewater treatment). LCT APPROACH: Attributional. | 1) FTOPSIS ranking table: Ranking of alternatives scenarios based on the FTOPSIS method (alpha-level sets to perform defuzzification at the very end of the decision analysis process). |
| (Besné et al. 2018) | HIGHLIGHTS: It presents an eco-efficiency analysis method that takes into account the Life Cycle Assessment (LCA) and fuzzy logic, integrating economic and environmental impact categories through a fuzzy eco-efficiency index. The methodological framework's robustness was tested using a comparative analysis with a decision surface graph, an eco-efficiency index, and random runs to prove their variability. The stability of the fuzzy preference orders and the scenarios clusters with a performance equally eco-efficient were corroborated by the comparative analysis between the random series results and the decision surface graph. Thus, the robustness of the methodology proposed was validated. APPLICATION AREA: Life Cycle Sustainability Assessment, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Equal weights. STUDY CASE: YES (plastic products manufacturer). LCT APPROACH: Attributional. | 1) Fuzzy eco-efficiency index: Vertical bar charts with the order of preference from the fuzzy matrix and the position vector resulting from the average impact categories results. 2) Overall environmental impact surfaces: Bidimensional decision surface graph of the weighted normalized environmental impacts versus their respective normalized costs. |
| (Maciol 2018) | HIGHLIGHTS: Compares a variety of specialized multicriteria methods and knowledge-based methods used to aggregate the results from LCA. The research used two classical methods for multicriteria decision-making (AHP and TOPSIS), the method of conventional (crisp) reasoning, and Mamdani’s fuzzy inference method. The results demonstrate that among the methods analyzed, only crisp reasoning does not provide satisfactory results. APPLICATION AREA: Life Cycle Assessment, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Experts opinions STUDY CASE: YES (Private passenger vehicle). LCT APPROACH: Attributional. | 1) Multicriteria methods table: Table with LCA single score from different multicriteria methods (AHP, TOPSIS, Classical rule-based, Mamdani I and II). |
| (Opher et al. 2019) | HIGHLIGHTS: Integrates the Life Cycle Assessment (endpoint categories), Social Equity Assessment (midpoint categories), and Economic Evaluation (midpoint categories), applying Fuzzy Logic associated with Analytical Hierarchical Process (AHP). The integration results may also serve as proof of the feasibility of coupling life cycle methodologies with multicriteria decision analysis (MCDA) techniques to communicate the results of Life Cycle Sustainability Assessment (LCSA) more effectively. The proposed coupling of methods may thus increase the capacity of life cycle thinking to act as a knowledge base for decision making in various domains. APPLICATION AREA: LCSA, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Experts opinions STUDY CASE: YES (Reclaimed domestic wastewater in urban households for toilet flushing and garden irrigation.) LCT APPROACH: Attributional. | 1) Sustainability bar: Vertical aggregated bar charts with the environmental, social and economic scores rates [0.00 – 1.00], separately. 2) Sustainability panel: Colored matrix with averaged weights for the three sustainability pillars, separately, per expert cluster. |
| (Li et al. 2019) | HIGHLIGHTS: Combines the life cycle assessment (LCA) method and fuzzy rough sets (FRS) theory to build an environmental evaluation model. FRS allows partial membership of an object to the lower and upper approximations, and approximate equality between objects can be modeled through fuzzy indiscernibility relations and was used to identify the crucial activities and exchanges during its life cycle from cradle-to-grave. The proposed evaluation method could quickly evaluate the environmental effects of a product and, with the FRS, successfully and effectively identify the essential element and link among its life span. APPLICATION AREA: Life Cycle Assessment, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: - STUDY CASE: YES (Renewable Energy System: wind power, small-scale hydropower, photovoltaic, centralized solar thermal power plant and a biogas power plant). LCT APPROACH: Attributional. | 1) Pollutant emissions table: Typical pollutant emissions quantity to environment directly for each power generation technology and overall system with function unit. 2) Environmental endpoint impacts chart: Vertical bar chart with the environmental impact scores per product for endpoint area of protection. 3) Environmental impact scores charts: Uncertainty analysis results during the performance degradation process. |
| (Sansa et al. 2019) | HIGHLIGHTS: Improve the design approaches in a critical stage from an environmental, economic, and social perspective. The challenge is to identify the product design scenarios that are coherent with the organization's context (in terms of internal and external issues and the requirements of the stakeholders) and provide the optimal option from a high number of identified scenarios. The first methodological step is to define the operational scenarios considering strategical and tactical assessments and value chain-driven sustainable design strategies like SWOT (Strengths, Weakness, Opportunity, Threats), PESTEL (Political, Economic, Social, Technological, Environmental, Legal), 7S (Structure, Strategy, Shared Values, Style, Staff, Systems, Skills) to identify a multitude of scenarios. The second step is subdivided into three: to use LCA tools to compute sustainability impacts' indicators to assess the operational scenarios; to minimize the number of scenarios using the subadditive/superadditive integral of Choquet, and; to select the optimal operational scenario via fuzzy analytic network process. APPLICATION AREA: Life Cycle Impact Assessment, Uncertainties, Eco-design, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Equations. STUDY CASE: YES (Fine particulate matter control technology.) LCT APPROACH: Attributional. | 1) Score table: Computed score for each scenario proposed. 2) Sustainability impacts table: The rate of decline of the contribution of a life cycle phase (LCP) to sustainability impacts. |
| (Xiong et al. 2020) | HIGHLIGHTS: A fuzzy comprehensive evaluation (FCE) associated with the analytic hierarchy process (AHP) was employed to improve the quality of the weights and matrix calculation for selecting the best air pollution control technologies. The FCE + AHP brought advantages of comprehensiveness and systematization, being helpful for synergistic control of the air pollution complex. APPLICATION AREA: Life Cycle Sustainability Assessment, Uncertainties, Single Point, Decision Make. FUZZY WEIGHTS DEFINITION: Experts opinions STUDY CASE: YES (Fine particulate matter control technology.) LCT APPROACH: Attributional. | 1) Indicator table: Indicator quantification results. 2) Technology performance radars: Radar chart with the evaluation diagram for the primary and secondary indicators. |

Table S2 – Fuzzy set for each input variable

|  |  | Very Low | Low | Medium | High | Very High |
| --- | --- | --- | --- | --- | --- | --- |
| CC | kg CO2 eq/tkm | [0 0 0.082 0.123] | [0.082 0.123 0.165] | [0.123 0.165 0.206] | [0.165 0.206 0.247] | [0.206 0.247 0.3 0.3] |
| CExD | MJ-eq/tkm | [0 0 1.27 1.906] | [1.27 1.906 2.541] | [1.906 2.541 3.177] | [2.541 3.177 3.812] | [3.177 3.812 3.812 3.9 3.9] |
| Price | R$/tkm | [0 0 0.009 0.014] | [0.009 0.014 0.019] | [0.014 0.019 0.023] | [0.019 0.023 0.028] | [0.023 0.028 0.1 0.1] |

Table S3 - Fuzzy controller rules.

|  | IF | Climate Change | + | Cumulative Exergy Demand | + | Price at Supplier | = | Environmental sustainability | Exergetic sustainability | Economic sustainability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | IF | 1 | AND | 1 | AND | 1 | THEN | 1.00 | 1.00 | 1.00 |
| 2 | IF | 1 | AND | 1 | AND | 2 | THEN | 1.11 | 1.10 | 1.62 |
| 3 | IF | 1 | AND | 1 | AND | 3 | THEN | 1.21 | 1.20 | 2.25 |
| 4 | IF | 1 | AND | 1 | AND | 4 | THEN | 1.32 | 1.30 | 2.87 |
| 5 | IF | 1 | AND | 1 | AND | 5 | THEN | 1.42 | 1.40 | 3.49 |
| 6 | IF | 1 | AND | 2 | AND | 1 | THEN | 1.26 | 1.58 | 1.14 |
| 7 | IF | 1 | AND | 2 | AND | 2 | THEN | 1.37 | 1.68 | 1.76 |
| 8 | IF | 1 | AND | 2 | AND | 3 | THEN | 1.47 | 1.78 | 2.38 |
| 9 | IF | 1 | AND | 2 | AND | 4 | THEN | 1.58 | 1.88 | 3.01 |
| 10 | IF | 1 | AND | 2 | AND | 5 | THEN | 1.69 | 1.98 | 3.63 |
| 11 | IF | 1 | AND | 3 | AND | 1 | THEN | 1.52 | 2.16 | 1.27 |
| 12 | IF | 1 | AND | 3 | AND | 2 | THEN | 1.63 | 2.26 | 1.90 |
| 13 | IF | 1 | AND | 3 | AND | 3 | THEN | 1.73 | 2.36 | 2.52 |
| 14 | IF | 1 | AND | 3 | AND | 4 | THEN | 1.84 | 2.46 | 3.14 |
| 15 | IF | 1 | AND | 3 | AND | 5 | THEN | 1.95 | 2.56 | 3.77 |
| 16 | IF | 1 | AND | 4 | AND | 1 | THEN | 1.78 | 2.74 | 1.41 |
| 17 | IF | 1 | AND | 4 | AND | 2 | THEN | 1.89 | 2.84 | 2.04 |
| 18 | IF | 1 | AND | 4 | AND | 3 | THEN | 1.99 | 2.94 | 2.66 |
| 19 | IF | 1 | AND | 4 | AND | 4 | THEN | 2.10 | 3.04 | 3.28 |
| 20 | IF | 1 | AND | 4 | AND | 5 | THEN | 2.21 | 3.14 | 3.90 |
| 21 | IF | 1 | AND | 5 | AND | 1 | THEN | 2.04 | 3.32 | 1.55 |
| 22 | IF | 1 | AND | 5 | AND | 2 | THEN | 2.15 | 3.42 | 2.17 |
| 23 | IF | 1 | AND | 5 | AND | 3 | THEN | 2.25 | 3.52 | 2.80 |
| 24 | IF | 1 | AND | 5 | AND | 4 | THEN | 2.36 | 3.62 | 3.42 |
| 25 | IF | 1 | AND | 5 | AND | 5 | THEN | 2.47 | 3.72 | 4.04 |
| 26 | IF | 2 | AND | 1 | AND | 1 | THEN | 1.63 | 1.32 | 1.24 |
| 27 | IF | 2 | AND | 1 | AND | 2 | THEN | 1.74 | 1.42 | 1.86 |
| 28 | IF | 2 | AND | 1 | AND | 3 | THEN | 1.85 | 1.52 | 2.49 |
| 29 | IF | 2 | AND | 1 | AND | 4 | THEN | 1.95 | 1.62 | 3.11 |
| 30 | IF | 2 | AND | 1 | AND | 5 | THEN | 2.06 | 1.72 | 3.73 |
| 31 | IF | 2 | AND | 2 | AND | 1 | THEN | 1.89 | 1.90 | 1.38 |
| 32 | IF | 2 | AND | 2 | AND | 2 | THEN | 2.00 | 2.00 | 2.00 |
| 33 | IF | 2 | AND | 2 | AND | 3 | THEN | 2.11 | 2.10 | 2.62 |
| 34 | IF | 2 | AND | 2 | AND | 4 | THEN | 2.21 | 2.20 | 3.25 |
| 35 | IF | 2 | AND | 2 | AND | 5 | THEN | 2.32 | 2.30 | 3.87 |
| 36 | IF | 2 | AND | 3 | AND | 1 | THEN | 2.15 | 2.48 | 1.51 |
| 37 | IF | 2 | AND | 3 | AND | 2 | THEN | 2.26 | 2.58 | 2.14 |
| 38 | IF | 2 | AND | 3 | AND | 3 | THEN | 2.37 | 2.68 | 2.76 |
| 39 | IF | 2 | AND | 3 | AND | 4 | THEN | 2.47 | 2.78 | 3.38 |
| 40 | IF | 2 | AND | 3 | AND | 5 | THEN | 2.58 | 2.88 | 4.01 |
| 41 | IF | 2 | AND | 4 | AND | 1 | THEN | 2.41 | 3.06 | 1.65 |
| 42 | IF | 2 | AND | 4 | AND | 2 | THEN | 2.52 | 3.16 | 2.27 |
| 43 | IF | 2 | AND | 4 | AND | 3 | THEN | 2.63 | 3.26 | 2.90 |
| 44 | IF | 2 | AND | 4 | AND | 4 | THEN | 2.73 | 3.36 | 3.52 |
| 45 | IF | 2 | AND | 4 | AND | 5 | THEN | 2.84 | 3.46 | 4.14 |
| 46 | IF | 2 | AND | 5 | AND | 1 | THEN | 2.68 | 3.64 | 1.79 |
| 47 | IF | 2 | AND | 5 | AND | 2 | THEN | 2.78 | 3.74 | 2.41 |
| 48 | IF | 2 | AND | 5 | AND | 3 | THEN | 2.89 | 3.84 | 3.04 |
| 49 | IF | 2 | AND | 5 | AND | 4 | THEN | 2.99 | 3.94 | 3.66 |
| 50 | IF | 2 | AND | 5 | AND | 5 | THEN | 3.10 | 4.04 | 4.28 |
| 51 | IF | 3 | AND | 1 | AND | 1 | THEN | 2.27 | 1.64 | 1.48 |
| 52 | IF | 3 | AND | 1 | AND | 2 | THEN | 2.37 | 1.74 | 2.10 |
| 53 | IF | 3 | AND | 1 | AND | 3 | THEN | 2.48 | 1.84 | 2.73 |
| 54 | IF | 3 | AND | 1 | AND | 4 | THEN | 2.59 | 1.94 | 3.35 |
| 55 | IF | 3 | AND | 1 | AND | 5 | THEN | 2.69 | 2.04 | 3.97 |
| 56 | IF | 3 | AND | 2 | AND | 1 | THEN | 2.53 | 2.22 | 1.62 |
| 57 | IF | 3 | AND | 2 | AND | 2 | THEN | 2.63 | 2.32 | 2.24 |
| 58 | IF | 3 | AND | 2 | AND | 3 | THEN | 2.74 | 2.42 | 2.86 |
| 59 | IF | 3 | AND | 2 | AND | 4 | THEN | 2.85 | 2.52 | 3.49 |
| 60 | IF | 3 | AND | 2 | AND | 5 | THEN | 2.95 | 2.62 | 4.11 |
| 61 | IF | 3 | AND | 3 | AND | 1 | THEN | 2.79 | 2.80 | 1.75 |
| 62 | IF | 3 | AND | 3 | AND | 2 | THEN | 2.89 | 2.90 | 2.38 |
| 63 | IF | 3 | AND | 3 | AND | 3 | THEN | 3.00 | 3.00 | 3.00 |
| 64 | IF | 3 | AND | 3 | AND | 4 | THEN | 3.11 | 3.10 | 3.62 |
| 65 | IF | 3 | AND | 3 | AND | 5 | THEN | 3.21 | 3.20 | 4.25 |
| 66 | IF | 3 | AND | 4 | AND | 1 | THEN | 3.05 | 3.38 | 1.89 |
| 67 | IF | 3 | AND | 4 | AND | 2 | THEN | 3.15 | 3.48 | 2.51 |
| 68 | IF | 3 | AND | 4 | AND | 3 | THEN | 3.26 | 3.58 | 3.14 |
| 69 | IF | 3 | AND | 4 | AND | 4 | THEN | 3.37 | 3.68 | 3.76 |
| 70 | IF | 3 | AND | 4 | AND | 5 | THEN | 3.47 | 3.78 | 4.38 |
| 71 | IF | 3 | AND | 5 | AND | 1 | THEN | 3.31 | 3.96 | 2.03 |
| 72 | IF | 3 | AND | 5 | AND | 2 | THEN | 3.41 | 4.06 | 2.65 |
| 73 | IF | 3 | AND | 5 | AND | 3 | THEN | 3.52 | 4.16 | 3.27 |
| 74 | IF | 3 | AND | 5 | AND | 4 | THEN | 3.63 | 4.26 | 3.90 |
| 75 | IF | 3 | AND | 5 | AND | 5 | THEN | 3.73 | 4.36 | 4.52 |
| 76 | IF | 4 | AND | 1 | AND | 1 | THEN | 2.90 | 1.96 | 1.72 |
| 77 | IF | 4 | AND | 1 | AND | 2 | THEN | 3.01 | 2.06 | 2.34 |
| 78 | IF | 4 | AND | 1 | AND | 3 | THEN | 3.11 | 2.16 | 2.96 |
| 79 | IF | 4 | AND | 1 | AND | 4 | THEN | 3.22 | 2.26 | 3.59 |
| 80 | IF | 4 | AND | 1 | AND | 5 | THEN | 3.32 | 2.36 | 4.21 |
| 81 | IF | 4 | AND | 2 | AND | 1 | THEN | 3.16 | 2.54 | 1.86 |
| 82 | IF | 4 | AND | 2 | AND | 2 | THEN | 3.27 | 2.64 | 2.48 |
| 83 | IF | 4 | AND | 2 | AND | 3 | THEN | 3.37 | 2.74 | 3.10 |
| 84 | IF | 4 | AND | 2 | AND | 4 | THEN | 3.48 | 2.84 | 3.73 |
| 85 | IF | 4 | AND | 2 | AND | 5 | THEN | 3.59 | 2.94 | 4.35 |
| 86 | IF | 4 | AND | 3 | AND | 1 | THEN | 3.42 | 3.12 | 1.99 |
| 87 | IF | 4 | AND | 3 | AND | 2 | THEN | 3.53 | 3.22 | 2.62 |
| 88 | IF | 4 | AND | 3 | AND | 3 | THEN | 3.63 | 3.32 | 3.24 |
| 89 | IF | 4 | AND | 3 | AND | 4 | THEN | 3.74 | 3.42 | 3.86 |
| 90 | IF | 4 | AND | 3 | AND | 5 | THEN | 3.85 | 3.52 | 4.49 |
| 91 | IF | 4 | AND | 4 | AND | 1 | THEN | 3.68 | 3.70 | 2.13 |
| 92 | IF | 4 | AND | 4 | AND | 2 | THEN | 3.79 | 3.80 | 2.75 |
| 93 | IF | 4 | AND | 4 | AND | 3 | THEN | 3.89 | 3.90 | 3.38 |
| 94 | IF | 4 | AND | 4 | AND | 4 | THEN | 4.00 | 4.00 | 4.00 |
| 95 | IF | 4 | AND | 4 | AND | 5 | THEN | 4.11 | 4.10 | 4.62 |
| 96 | IF | 4 | AND | 5 | AND | 1 | THEN | 3.94 | 4.28 | 2.27 |
| 97 | IF | 4 | AND | 5 | AND | 2 | THEN | 4.05 | 4.38 | 2.89 |
| 98 | IF | 4 | AND | 5 | AND | 3 | THEN | 4.15 | 4.48 | 3.51 |
| 99 | IF | 4 | AND | 5 | AND | 4 | THEN | 4.26 | 4.58 | 4.14 |
| 100 | IF | 4 | AND | 5 | AND | 5 | THEN | 4.37 | 4.68 | 4.76 |
| 101 | IF | 5 | AND | 1 | AND | 1 | THEN | 3.53 | 2.28 | 1.96 |
| 102 | IF | 5 | AND | 1 | AND | 2 | THEN | 3.64 | 2.38 | 2.58 |
| 103 | IF | 5 | AND | 1 | AND | 3 | THEN | 3.75 | 2.48 | 3.20 |
| 104 | IF | 5 | AND | 1 | AND | 4 | THEN | 3.85 | 2.58 | 3.83 |
| 105 | IF | 5 | AND | 1 | AND | 5 | THEN | 3.96 | 2.68 | 4.45 |
| 106 | IF | 5 | AND | 2 | AND | 1 | THEN | 3.79 | 2.86 | 2.10 |
| 107 | IF | 5 | AND | 2 | AND | 2 | THEN | 3.90 | 2.96 | 2.72 |
| 108 | IF | 5 | AND | 2 | AND | 3 | THEN | 4.01 | 3.06 | 3.34 |
| 109 | IF | 5 | AND | 2 | AND | 4 | THEN | 4.11 | 3.16 | 3.96 |
| 110 | IF | 5 | AND | 2 | AND | 5 | THEN | 4.22 | 3.26 | 4.59 |
| 111 | IF | 5 | AND | 3 | AND | 1 | THEN | 4.05 | 3.44 | 2.23 |
| 112 | IF | 5 | AND | 3 | AND | 2 | THEN | 4.16 | 3.54 | 2.86 |
| 113 | IF | 5 | AND | 3 | AND | 3 | THEN | 4.27 | 3.64 | 3.48 |
| 114 | IF | 5 | AND | 3 | AND | 4 | THEN | 4.37 | 3.74 | 4.10 |
| 115 | IF | 5 | AND | 3 | AND | 5 | THEN | 4.48 | 3.84 | 4.73 |
| 116 | IF | 5 | AND | 4 | AND | 1 | THEN | 4.31 | 4.02 | 2.37 |
| 117 | IF | 5 | AND | 4 | AND | 2 | THEN | 4.42 | 4.12 | 2.99 |
| 118 | IF | 5 | AND | 4 | AND | 3 | THEN | 4.53 | 4.22 | 3.62 |
| 119 | IF | 5 | AND | 4 | AND | 4 | THEN | 4.63 | 4.32 | 4.24 |
| 120 | IF | 5 | AND | 4 | AND | 5 | THEN | 4.74 | 4.42 | 4.86 |
| 121 | IF | 5 | AND | 5 | AND | 1 | THEN | 4.58 | 4.60 | 2.51 |
| 122 | IF | 5 | AND | 5 | AND | 2 | THEN | 4.68 | 4.70 | 3.13 |
| 123 | IF | 5 | AND | 5 | AND | 3 | THEN | 4.79 | 4.80 | 3.75 |
| 124 | IF | 5 | AND | 5 | AND | 4 | THEN | 4.89 | 4.90 | 4.38 |
| 125 | IF | 5 | AND | 5 | AND | 5 | THEN | 5.00 | 5.00 | 5.00 |

FUZZY INFERENCE SYSTEM [MATHLAB SOURCE]

[System]

Name='MulticriteriaFuzztController_fis'

Type='mamdani'

Version=2.0

NumInputs=3

NumOutputs=3

NumRules=125

AndMethod='min'

OrMethod='max'

ImpMethod='min'

AggMethod='max'

DefuzzMethod='centroid'

[Input1]

Name='CC'

Range=[0 0.3]

NumMFs=5

MF1='Very Low':'trapmf',[0 0 0.0826083333333333 0.1239125]

MF2='Low':'trimf',[0.0826083333333333 0.1239125 0.165216666666667]

MF3='Medium':'trimf',[0.1239125 0.165216666666667 0.206520833333333]

MF4='High':'trimf',[0.165216666666667 0.206520833333333 0.247825]

MF5='Very high':'trapmf',[0.206520833333333 0.247825 0.3 0.3]

[Input2]

Name='CExD'

Range=[0 3.9]

NumMFs=5

MF1='Very Low':'trapmf',[0 0 1.27083333333333 1.90625]

MF2='Low':'trimf',[1.27083333333333 1.90625 2.54166666666667]

MF3='Medium':'trimf',[1.90625 2.54166666666667 3.17708333333333]

MF4='High':'trimf',[2.54166666666667 3.17708333333333 3.8125]

MF5='Very high':'trapmf',[3.177 3.813 3.9 3.9]

[Input3]

Name='Price'

Range=[0 0.1]

NumMFs=5

MF1='Very Low':'trapmf',[0 0 0.0095 0.01425]

MF2='Low':'trimf',[0.0095 0.01425 0.019]

MF3='Medium':'trimf',[0.01425 0.019 0.02375]

MF4='High':'trimf',[0.019 0.02375 0.0285]

MF5='Very high':'trapmf',[0.02375 0.0285 0.1 0.1]

[Output1]

Name='Environmental'

Range=[0 100]

NumMFs=5

MF1='Very Low':'trapmf',[0 0 10 20]

MF2='Low':'trimf',[15 30 45]

MF3='Medium':'trimf',[35 50 65]

MF4='High':'trimf',[55 70 85]

MF5='Very high':'trapmf',[75 90 100 100]

[Output2]

Name='Exergetic'

Range=[0 100]

NumMFs=5

MF1='Very Low':'trapmf',[0 0 10 20]

MF2='Low':'trimf',[15 30 45]

MF3='Medium':'trimf',[35 50 65]

MF4='High':'trimf',[55 70 85]

MF5='Very high':'trapmf',[75 90 100 100]

[Output3]

Name='Economic'

Range=[0 100]

NumMFs=5

MF1='Very Low':'trapmf',[0 0 10 20]

MF2='Low':'trimf',[15 30 45]

MF3='Medium':'trimf',[35 50 65]

MF4='High':'trimf',[55 70 85]

MF5='Very high':'trapmf',[75 90 100 100]

[Rules]

1 1 1, 5 5 5 (1) : 1

1 1 2, 5 5 5 (1) : 1

1 1 3, 5 5 4 (1) : 1

1 1 4, 5 5 3 (1) : 1

1 1 5, 5 5 2 (1) : 1

1 2 1, 5 5 5 (1) : 1

1 2 2, 5 5 5 (1) : 1

1 2 3, 5 5 4 (1) : 1

1 2 4, 5 4 3 (1) : 1

1 2 5, 5 4 2 (1) : 1

1 3 1, 5 4 5 (1) : 1

1 3 2, 5 4 4 (1) : 1

1 3 3, 5 4 4 (1) : 1

1 3 4, 4 4 3 (1) : 1

1 3 5, 4 4 2 (1) : 1

1 4 1, 5 3 5 (1) : 1

1 4 2, 4 3 4 (1) : 1

1 4 3, 4 3 3 (1) : 1

1 4 4, 4 3 3 (1) : 1

1 4 5, 4 3 2 (1) : 1

1 5 1, 4 3 5 (1) : 1

1 5 2, 4 2 4 (1) : 1

1 5 3, 4 2 3 (1) : 1

1 5 4, 4 2 2 (1) : 1

1 5 5, 4 2 2 (1) : 1

2 1 1, 5 5 5 (1) : 1

2 1 2, 5 5 4 (1) : 1

2 1 3, 4 5 4 (1) : 1

2 1 4, 4 5 3 (1) : 1

2 1 5, 4 5 2 (1) : 1

2 2 1, 4 4 5 (1) : 1

2 2 2, 4 4 4 (1) : 1

2 2 3, 4 4 3 (1) : 1

2 2 4, 4 4 3 (1) : 1

2 2 5, 4 4 2 (1) : 1

2 3 1, 4 4 5 (1) : 1

2 3 2, 4 4 4 (1) : 1

2 3 3, 4 3 3 (1) : 1

2 3 4, 4 3 3 (1) : 1

2 3 5, 4 3 2 (1) : 1

2 4 1, 4 3 5 (1) : 1

2 4 2, 4 3 4 (1) : 1

2 4 3, 3 3 3 (1) : 1

2 4 4, 3 3 2 (1) : 1

2 4 5, 3 2 2 (1) : 1

2 5 1, 3 2 5 (1) : 1

2 5 2, 3 2 4 (1) : 1

2 5 3, 3 2 3 (1) : 1

2 5 4, 3 2 2 (1) : 1

2 5 5, 3 2 1 (1) : 1

3 1 1, 4 5 5 (1) : 1

3 1 2, 4 5 4 (1) : 1

3 1 3, 4 4 3 (1) : 1

3 1 4, 4 4 3 (1) : 1

3 1 5, 3 4 2 (1) : 1

3 2 1, 4 4 5 (1) : 1

3 2 2, 3 4 4 (1) : 1

3 2 3, 3 4 3 (1) : 1

3 2 4, 3 4 2 (1) : 1

3 2 5, 3 3 2 (1) : 1

3 3 1, 3 3 5 (1) : 1

3 3 2, 3 3 4 (1) : 1

3 3 3, 3 3 3 (1) : 1

3 3 4, 3 3 2 (1) : 1

3 3 5, 3 3 1 (1) : 1

3 4 1, 3 3 4 (1) : 1

3 4 2, 3 2 4 (1) : 1

3 4 3, 3 2 3 (1) : 1

3 4 4, 3 2 2 (1) : 1

3 4 5, 2 2 1 (1) : 1

3 5 1, 3 2 4 (1) : 1

3 5 2, 2 2 3 (1) : 1

3 5 3, 2 2 3 (1) : 1

3 5 4, 2 1 2 (1) : 1

3 5 5, 2 1 1 (1) : 1

4 1 1, 3 4 5 (1) : 1

4 1 2, 3 4 4 (1) : 1

4 1 3, 3 4 3 (1) : 1

4 1 4, 3 4 2 (1) : 1

4 1 5, 3 4 1 (1) : 1

4 2 1, 3 4 4 (1) : 1

4 2 2, 3 3 4 (1) : 1

4 2 3, 3 3 3 (1) : 1

4 2 4, 2 3 2 (1) : 1

4 2 5, 2 3 1 (1) : 1

4 3 1, 2 3 4 (1) : 1

4 3 2, 2 3 3 (1) : 1

4 3 3, 2 3 3 (1) : 1

4 3 4, 2 2 2 (1) : 1

4 3 5, 2 2 1 (1) : 1

4 4 1, 2 2 4 (1) : 1

4 4 2, 2 2 3 (1) : 1

4 4 3, 2 2 3 (1) : 1

4 4 4, 2 2 2 (1) : 1

4 4 5, 2 2 1 (1) : 1

4 5 1, 2 1 4 (1) : 1

4 5 2, 2 1 3 (1) : 1

4 5 3, 2 1 2 (1) : 1

4 5 4, 1 1 2 (1) : 1

4 5 5, 1 1 1 (1) : 1

5 1 1, 2 4 4 (1) : 1

5 1 2, 2 4 4 (1) : 1

5 1 3, 2 4 3 (1) : 1

5 1 4, 2 4 2 (1) : 1

5 1 5, 2 3 1 (1) : 1

5 2 1, 2 3 4 (1) : 1

5 2 2, 2 3 3 (1) : 1

5 2 3, 2 3 3 (1) : 1

5 2 4, 2 3 2 (1) : 1

5 2 5, 1 3 1 (1) : 1

5 3 1, 2 2 4 (1) : 1

5 3 2, 2 2 3 (1) : 1

5 3 3, 1 2 2 (1) : 1

5 3 4, 1 2 2 (1) : 1

5 3 5, 1 2 1 (1) : 1

5 4 1, 1 2 4 (1) : 1

5 4 2, 1 2 3 (1) : 1

5 4 3, 1 1 2 (1) : 1

5 4 4, 1 1 1 (1) : 1

5 4 5, 1 1 1 (1) : 1

5 5 1, 1 1 4 (1) : 1

5 5 2, 1 1 3 (1) : 1

5 5 3, 1 1 2 (1) : 1

5 5 4, 1 1 1 (1) : 1

5 5 5, 1 1 1 (1) : 1