Appendix-A-

%%% DEVELOPMENT SOFTWARE PROGRAM FOR  FINDING PHOTOVOLTAIC CELL EQUIVALENT CIRCUIT PARAMETERS

%%% GENERAL EXPLANATİONS %%%%%%%

%% The program is written in general about the known panel system using the F(V)=I curve.

% The program is written using 5 points on the F(V)=I curve.

% (Ex: V1,I1;V2,I2;V3,I3;V4,I4;V5,I5)

% The state variables for the written program are as follows:

% x1=Iph; x2=Id; x3=m; x4=Rs; x5=Rsh ; taken as

% SOLUTION OF NONLINEAR EQUATIONS BY NEWTON-RAPHSON METHOD

% tol; It is the tolerance value

% max1; The maximum number of iterations that will be allowed.

% k; has occurred  iteration  number.

% X ; is the vector representing the root values.

% error; is the amount of error in iteration convergence.

% The initial conditions are included in the X vector.

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%  THE RESULT THAT SHOULD BE ACCORDİNG TO THE SIMULINK:  9.33    1.0649  5.88    0.2670  599.9994

%%%%%%%%%%%%%%%%%%%%%%%%%%%% RESULT %%%%%%%%%%%%%%%%%% iter= 24 %%%%%%%%%%%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% Result values of the NR program:         9.33  1.087    5.84    0.2695  593.13

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

clear all

clc

tol=0.000001; % NR- iteration stop tolerance

max1=10000; % number of maximum iterations

a=0.01;

x0=[3 1 2 0.11 145]; % NR-iteration starting values

q=1.602*1e-19;

T=25+273; %kelvin

K=1.381*1e-23;

C=q/(T*K);

%%%%%%%%%%%%%%%%%%%%%%% The SUNTECH   STP  275-20/Wfw   CATALOG VALUES WİLL BE USED %%%%%%%%%%%%%%%

%%%%%  5 VALUES OBTAİNED FROM V-I CURVE  of  The SUNTECH STP275-20/Wfw %%%%%%%%%

I1=9.329;      V1=0.326;  % f(V)=I obtained from the I-V curve (1)

I2=9.223;      V2=28.02;  % f(V)=I obtained from the I-V curve (2)

I3=8.82;       V3=31,2;   % f(V)=I obtained from the I-V curve  (3) MPPT point

I4=6.656;      V4=34.41;  % f(V)=I obtained from the I-V curve  (4)

I5=0.2771;     V5=37.98;  % f(V)=I obtained from the I-V curve  (5)

N=10; % NUMBER OF CELLS IN THE MODULE

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

S=1; % THE NUMBER OF SERIAL-CONNECTED MODULES  INCREASES, THE NUMBER OF "S" ALSO INCREASES. DETERMINE MPPT POINT BY "S" VALUE  %%%% %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

b=[3.1 1.11 2.92 2.45 4.09]; % Sensitivity values of state variables

x0=b.*x0;

X=x0;

syms x1 x2 x3 x4 x5

f1= x1-(1e-6)*x2*(exp(C*(V1+I1*x4)/(S*N*x3))-1)-((V1+I1*x4)/x5)-I1;

f2= x1-(1e-6)*x2*(exp(C*(V2+I2*x4)/(S*N*x3))-1)-((V2+I2*x4)/x5)-I2;

f3= x1-(1e-6)*x2*(exp(C*(V3+I3*x4)/(S*N*x3))-1)-((V3+I3*x4)/x5)-I3;

f4= x1-(1e-6)*x2*(exp(C*(V4+I4*x4)/(S*N*x3))-1)-((V4+I4*x4)/x5)-I4;

f5= x1-(1e-6)*x2*(exp(C*(V5+I5*x4)/(S*N*x3))-1)-((V5+I5*x4)/x5)-I5;

Y(1)=subs(f1,{x1,x2,x3,x4,x5},[x0(1) x0(2) x0(3) x0(4) x0(5)]);

Y(2)=subs(f2,{x1,x2,x3,x4,x5},[x0(1) x0(2) x0(3) x0(4) x0(5)]);

Y(3)=subs(f3,{x1,x2,x3,x4,x5},[x0(1) x0(2) x0(3) x0(4) x0(5)]);

Y(4)=subs(f4,{x1,x2,x3,x4,x5},[x0(1) x0(2) x0(3) x0(4) x0(5)]);

Y(5)=subs(f5,{x1,x2,x3,x4,x5},[x0(1) x0(2) x0(3) x0(4) x0(5)]);

%Y=feval('canki_3_alt_1',X);

for k=1:max1

disp('iter'),disp(k);

% The following line calculates the Jacobien matrix for the X values

J=jacobian([f1;f2;f3;f4;f5],[x1 x2 x3 x4 x5]);

J=subs(J,{x1,x2,x3,x4,x5},[X(1) X(2) X(3) X(4) X(5)]);

J1=double(J);

Q=X-a*(J1\Y')';

error=abs(Q-X);

%     vpa(Q(1),6)

%     vpa(X(1),6)

if Q(1)-X(1)>0

a=0.5*a;

disp('it started to move away from data values')

%pause

end

error =vpa(max(error),5)

if (max(error)<tol)

disp('number of iterations performed')

disp(k)

disp('searched root values ')

format short

disp(['     Iph', '        Id', '         m', '        Rs', '      Rsh'])

disp(double(X))

break

else

X=Q;

Z(1)=subs(f1,{x1,x2,x3,x4,x5},[Q(1) Q(2) Q(3) Q(4) Q(5)]);

Z(2)=subs(f2,{x1,x2,x3,x4,x5},[Q(1) Q(2) Q(3) Q(4) Q(5)]);

Z(3)=subs(f3,{x1,x2,x3,x4,x5},[Q(1) Q(2) Q(3) Q(4) Q(5)]);

Z(4)=subs(f4,{x1,x2,x3,x4,x5},[Q(1) Q(2) Q(3) Q(4) Q(5)]);

Z(5)=subs(f5,{x1,x2,x3,x4,x5},[Q(1) Q(2) Q(3) Q(4) Q(5)]);

Y=double(Z);

end

end

if k==max1

disp('Iteration stopped because max1 iteration count has been exceeded')

end