# 3 Verification of Calculation Procedure

## 3.1 Precision estimates

Smirnov et al25,26 point out that estimating precision and errors accumulation is necessary for large scale simulations of complex chemically reacting gas dynamics. A definite error occurs in integration at each step, which is dependent on spatial resolution and numerical solver. This error can be expressed in absolute or relative values. Accumulation of error takes place for successive time steps.

The relative error of integration in one-dimensional case S1 is proportional to the mean ratio of cell size ΔL to the domain size L1 in the direction of integration in power depending on scheme accuracy25:

(31)

Where k is the order of accuracy of numerical scheme. In the presence of several directions in integration, the errors25 are written as

(32)

The allowable value of total error Smax25 is defined by the user of the simulator. For initial and boundary conditions are usually not known with higher accuracy, Smax should be between 1% and 5%. Then the following inequality should be satisfied.

(33)

Where n is the number of time steps in Navier-Stokes equations integration.

Another important characteristic is introduced for the results of supercomputer simulations. The ratio of the maximal allowable number of time steps to the actual number of time steps is defined as25:

(34)

Table 3 shows the accumulation of error for the grid resolution and physical time simulations. The allowable error was assumed at 5%. For present numerical simulations, all results demonstrate high reliability, but it is not always appropriate to simulate longer steps.

Table 3 Error estimates

| Case | TCM2 | HEG |
| --- | --- | --- |
| Allowable error | 5% | 5% |
| Grid resolution | 1251×201 | 1251×201 |
| Physical time simulated(ms) | 3.5 | 5.0 |
| Number of time steps | 3500 | 5000 |
| Accumulated error | 1.23×10-7 | 1.23×10-7 |
| Allowable number of time steps | 1.63×1011 | 1.63×1011 |
| Reliability Rs | 4.67×107 | 3.27×107 |

## 3.2 Verification of the calculation method

To verify the calculation method, the comparisons with the TCM24 and HEG5 results are made. For the TCM2 case, the initial condition is based on case 1 in Table 2, and the normal scale is selected. Fig.2 shows the centerline vibrational temperature TV and translational temperature T distribution. The TV,N2 is the vibrational temperature of N2 and is about 2400K, and TV,O2 is the vibrational temperature of O2 and is about 1000K. The unified vibrational temperature Tv is about 2290K in this paper. Considering the mass ratio of N2 to O2 in the flow, this value is reasonable. The centerline evolution of vibrational temperature and translational temperature are plotted, as shown in Fig.3. The results of the freezing position and temperature are good agreement with the reference. Fig.4 shows the centerline Mach number distribution, and the variation of about 5% is predicted. The discrepancy can be explained by the different vibrational modeling, resulting in different translational temperature T (static temperature). Table 4a shows the comparison of the Mach number and species mass fraction in the nozzle exit. The accuracy of the static pressure P and translational temperature T is verified by the HEG nozzle. For the HEG case, the initial condition is based on Case2 in Table 2. The calculation results are compared with the results for the reference5, as shown in Table 4b.

Fig.2 Comparison of the centerline temperature distributions

Fig.3 Comparison of the centerline O2 mass fraction distributions

Fig.4 Comparison of the centerline Mach number distributions

Table 4a Comparison of flow parameters in the TCM2 nozzle

| Results | T (K) | Ma | CN2 (%) | Co2 (%) | CNO (%) | CO (%) | CN (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TCM2 | 356 | 11.2 | 73.23 | 17.43 | 7.44 | 2.04 | -- |
| 7S2T | 358 | 11.6 | 73.27 | 17.82 | 7.60 | 1.31 | 2.6E-7 |
| 5S2T | 360 | 11.5 | 73.17 | 17.57 | 7.41 | 1.93 | 1.5E-8 |

Table 4b Comparison of flow parameters in the HEG nozzle

| Results | P (Pa) | T (K) | u∞ (m/s) | CN2 (%) | Co2 (%) | CNO (%) | CO (%) | CN (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HEG | 713 | 694 | 4776 | 73.56 | 13.40 | 5.09 | 7.96 | 1.0E-8 |
| 7S2T | 701 | 723 | 4842 | 74.01 | 13.75 | 4.96 | 7.25 | 1.4E-7 |
| 5S2T | 705 | 720 | 4823 | 73.94 | 13.48 | 5.03 | 7.83 | 1.8E-8 |

The calculation results of the nozzles of TCM2 and HEG in this paper agree well with the reference values4,5. The discrepancy calculated by comparing the main reference values is nominally 5 percent or less for all the cases considered. The variation can be explained. There are three main reasons for these differences. First, by the different vibrational modeling. Second, by the different transport coefficients. The species NO+ and e- belong to the micro-species. The results verify the positive determination of the relevant parameter settings and the accuracy of the procedure. Third, the turbulence energy could decrease some other energy in an isolated system. When the total temperature exceeds 6000K, there are trace species such as electrons,NO+. It is more appropriate to use 7-species, so the following calculation uses 7-species.