Table 1: Recording of drilling data for a 100 mm depth hole

| Time (sec) | Z-axis (mm) | Motion |  |  | Time (sec) | Z-axis (mm) | Motion |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | + 2 |  | Rapid at staring position |  | 18 | -85 |  | Return to current depth level |
| 5 | - 40 |  | Drill depth of Zone 1 (F500, S2000) |  | 19 | -90 |  | 4th peck in Zone 3 (F300 S1400) |
| 6 | - 50 |  | 1st peck in Zone 2 (F400 S1800) |  | 19 | -82 |  | Rapid retraction (8 mm) |
| 6 | - 46 |  | Rapid retraction (4 mm) |  | 20 | -90 |  | Return to current depth level |
| 7 | - 50 |  | Return to current depth level |  | 20 | -92 |  | 1st peck in Zone 4 (F200 S1000) |
| 9 | - 60 |  | 2nd peck in Zone 2 (F400 S1800) |  | 21 | -86 |  | Rapid retraction (8 mm) |
| 9 | - 56 |  | Rapid retraction (4 mm) |  | 21 | -92 |  | Return to current depth level |
| 9 | - 60 |  | Return to current depth level |  | 22 | -94 |  | 2nd peck in Zone 4 (F200 S1000) |
| 11 | - 70 |  | 3rd peck in Zone 2 (F400 S1800) |  | 22 | -86 |  | Rapid retraction (8 mm) |
| 11 | - 66 |  | Rapid retraction (4 mm) |  | 23 | -94 |  | Return to current depth level |
| 12 | - 70 |  | Return to current depth level |  | 24 | -96 |  | 3rd peck in Zone 4 (F200 S1000) |
| 13 | -75 |  | 1st peck in Zone 3 (F300 S1400) |  | 24 | -88 |  | Rapid retraction (8 mm) |
| 13 | -67 |  | Rapid retraction (8 mm) |  | 25 | -96 |  | Return to current depth level |
| 14 | -75 |  | Return to current depth level |  | 26 | -98 |  | 3rd peck in Zone 4 (F200 S1000) |
| 15 | -80 |  | 2nd peck in Zone 3 (F300 S1400) |  | 26 | -90 |  | Rapid retraction (8 mm) |
| 15 | -72 |  | Rapid retraction (8 mm) |  | 27 | -98 |  | Return to current depth level |
| 16 | -80 |  | Return to current depth level |  | 28 | - 100 |  | Last peck (F200, S1000) |
| 17 | -85 |  | 3rd peck in Zone 3 (F300 S1400) |  | 29 | - 100 |  | Dwell 10 seconds |
| 17 | -77 |  | Rapid retraction (8 mm) |  | 40 | + 2 |  | Full retraction at starting position |