An in-situ study of the thermal decomposition of AIBN radical chemistry using a dual mode EPR resonator.

Giuseppina Magri*, Michael Barter, Jack Fletcher-Charles, Heungjae Choi, Daniel Slocombe, Emma Richards, Andrea Folli*, Adrian Porch, Damien M. Murphy*

Supporting Information

Signal evolution of DMPO-CPO over time					Page 1

Gas flow under conventional heating conditions				Page 2

Multifrequency dielectric measurements					Page 3

Signal evolution of DMPO-CPO over time at a constant temperature

To ensure the ST●-CPO adduct was generated by temperature (rather than a slow accumulation of the adduct with time), a sample of the AIBN solution containing the ST was held at 75 °C under conventional heating conditions for ca. 2.5 hrs to assess whether the signal of the DMPO●-CP adduct could be detected. The EPR signal at 75 °C as a function of time (Fig. S1) illustrates the occurrence of the DMPO●-CPO adduct over the 2.5 hrs timescale, offering no evidence to suggest the DMPO●-CP adduct is formed at this temperature under completely aerobic conditions. As such, it can be inferred that the temperature of the reaction medium influences the radical distribution (CPO● vs CP●) more so than the reaction time.

Fig. S1. Absolute value of the X-band CW EPR spectra recorded at 75 °C of 20 mM AIBN + 3 eq. DMPO in toluene over ca. 2.5 hrs duration under conventional heating conditions.

There appears to be an overall decrease in radical concentration as time progresses. This is likely due to the fact that all of the starting substrate has been consumed (CPO●). Throughout the course of the measurements, there is a continual supply of radicals being generated but simultaneously a decay in radical-adduct (due to limited lifetime of the adduct itself) causing the decrease in signal.

Gas flow using conventionally heating

To monitor the heating rate in the conventional Bruker cavity, a mass-flow controller was used on the N2 supply (prior to the heating coils). A temperature increase from RT - 70°C could be achieved within 80 secs. using a flow rate of 0.08 L/min. The results are shown below in Fig. S2

Fig. 2. Temperature vs. time graph for differing flow rates of N2 using conventional heating on a Bruker EMX system. Blue asterisk indicates start of heating.

Dielectric multifrequency measurements

# Dielectric measurements were performed at different microwave frequencies using a combination of two microwave host cavities. The first of these was a copper stripline resonator, resonant at frequencies of 1, 2, 3 and 4 GHz, the second a TM aluminium cylindrical cavity operating in its 2.5 GHz (TM010) and 5.7 GHz (TM020) modes.[1] Samples were held in quartz tubes of inner diameters of 4.2 mm and measurements of the resonant frequencies and quality factors Q were performed using an Agilent E5071C vector network analyser. The resonant frequency shift Df was computed for each mode, defined to be the difference in resonant frequency with and without the sample liquid in the tube, together with the equivalent difference in reciprocal Q factors D(1/Q); the former quantifies the polarisation e’  1 of the sample and the latter the loss term e’’. For the TM cavity, these are computed using simple first order perturbation theory via the equations e’  1 + 2kDf and e’’  kDf(1/Q), where for each mode the geometry factor k is found by measurements the same volume of a known liquid (in this case pure toluene). For the stripline resonator we use a modified form of perturbation analysis that includes the depolarisation effects of the microwave electric field being perpendicular to the axis of each (cylindrical) sample, again using pure toluene as a reference liquid. All measurements are performed by undertaking a Lorentzian curve fit to the transmitted power coefficient |S21|2 through the two port cavities, which are weakly coupled to their input/output circuits to remove any systematic errors linked to the process of converting loaded to unloaded Q. The two coupling structures are near-identical loops that terminate each of the RG405 coaxial input/output feedlines in a short circuit, thus providing inductive coupling to the magnetic field of each mode.

Under aerobic conditions for the T-jump experiments, it was observed that -5.3 dBm (0.3 mW) of microwave power applied at 6.1 GHz was initially required to heat the PBN samples to 95 °C, whereas only -6.4 dBm (0.23 mW) was necessary to heat the DMPO samples to the same temperature. This prompted an investigation into the dielectric properties of the AIBN/PBN and AIBN/DMPO solutions in toluene. To this end, multifrequency dielectric measurements were recorded to determine the dielectric permittivity (ε’) and loss (ε’’) of each sample (Fig. S3).

Fig. S3. Multifrequency dielectric measurements indicating the (a) permittivity (ε’) and (b) loss (ε’’) of various AIBN solutions in toluene. Standard errors are based on triplicate measurements (the error in the loss e’’ is too small to be visible).

Toluene was used as the control solvent for comparison, and our ε’ and ε’’ measurements accurately mimic literature data when matching dielectric permittivity (or dielectric constant, ε’ = 2.38 at 25 °C).[2] Whilst no clear relationship between applied frequency and permittivity could be determined for the samples, in all cases the solution containing DMPO yielded the highest permittivity value. In contrast, there is a clear trend in the loss ε’’ parameter. Measuring at higher frequencies was found to give an increase in loss of all samples. It is important to note that multifrequency dielectric measurements are not possible on the dual-mode resonator, as such these measurements were performed on a parallel plate resonator operating at 4 unique modes (1 – 4 GHz) as well as a resonator operating at 5.7 GHz. This is the closest measurement frequency available to 6.1 GHz, which is the operating frequency of the heating mode within the dual-mode resonator. Encouragingly, at 5.7 GHz, the loss (ε’’) of the DMPO solution is greater than the PBN solutions by ca. 6%. Ultimately, due to the lower loss of the AIBN/PBN/Toluene solutions, this is the reason why more power is required to induce dielectric heating, elucidating a slight difference in dielectric properties between these two solutions effected by the alteration in spin trap.

References

1. J. A. Cuenca, K. Bugler, S. Taylor, D. Morgan, P. Williams, J. Bauer, and A. Porch, J. Phys. Condens. Matter 28, (2016).

2. G. Akerlof, J. Am. Chem. Soc. 54, 4125 (1932).