Table 2

Failure factors of gathering pipelines.

(a) Third-party damage

| First level | Second level | Weight | Description |
| --- | --- | --- | --- |
| Third party damage | Buried depth | Buried (28) | >160cm (7); 100cm~160cm (14); 50cm~100cm (21); <50cm (28) |
|  |  | Above ground and across (28) | Well protection (5); A degree of protection (16); None (28) |
|  | Ground facilities | a Line mark (15) | 100% (3); 80%-100% (5); 60%-80% (8); 40% -60% (10); <40% or none (15) |
|  |  | b Building facilities (7) | None (2); 1-3 (3); >3 (5) |
|  | Activity level | c Construction (6) | None (1); 0~3 months (2); 3~12 months (4); >12 months (6). |
|  |  | Transportation (8) | None (2); Traffic branch (4); Traffic artery/main road (6); Rail or road traffic trunk lines (8) |
|  | Patrol efficiency | Frequency (10) | Once a day (2); Once every two days (5); Once a week (7); Once a month (10) |
|  |  | Method (7) | Step by step (1); High-risk pipe section (3); Valve chamber (7) |
|  | Malicious damage | Possibility (6) | Low (2); Medium (3); High (6) |
|  | Natural disasters | History (5) | None (0); Occurred but did not cause incident (2); Occurred and caused pipeline failure (5) |
|  |  | Protection (8) | Disaster monitoring and normal operation (1); In-line inspection (4); Pigging and corrosion inhibitor (8) |
| Total | 100 |  |  |

a. Completeness of warning signs

b. The evaluation range is 7 meters on both sides of the pipeline

c. Construction duration

(b) Corrosion

| First level | Second level | Weight | Third level | Weight | Description |
| --- | --- | --- | --- | --- | --- |
| Corrosion | External corrosion | Heavy oil (13/20)a; Thin oil (2/5); Gas (21/20); Water, Steam (13/20) | Soil corrosion | Resistivity (7) | >50Ω•m (2); 20~50Ω·m (4); <20Ω·m or none (7) |
|  |  |  |  | pH (8) | 6.5~8.5 (2); 4.5~6.5 (4); <4.5 (8) |
|  |  |  | External coating | Type (6) | 3-Layer Polyethylene (1); Coal tar enamel/coal tar epoxy/epoxy powder (2); Yellow jacket polyurethane foam/polythene (3); Asphalt glass fabric/phenolic resins (4); Anti-rust oil (5), None (6) |
|  |  |  |  | Quality (9) | Complete (2); Partial loss (5), Mass shedding (9) |
|  |  |  | Cathodic protection (10) |  | Normal use (3); Not used (6); None (10) |
|  | Internal corrosion | Heavy oil (24/29) Thin oil (1) Gas (4/8) Water/Steam (1) | Oil (58) | Water (6) | <50% (2); 50%~75% (3); 75%~95% (5); >95% (6) |
|  |  |  |  | Pressure (5) | <0.6 MPa (1); 0.6~0.8 MPa (3); 0.8~1.5 MPa (4); >1.5MPa (5) |
|  |  |  |  | CI- (10) | <1000mg/L (2); 1000~3000mg/L (3); 3000~5000mg/L (5); 5000~7000mg/L (7); 7000~9000 (8); >9000 mg/L (10) |
|  |  |  |  | H2S (10) | <0.005mg/L (2); 0.005~0.01mg/L (4); 0.01~0.015mg/L (7); >0.015mg/L (10) |
|  |  |  |  | Temperature (4) | <50℃ (2); 50~70℃ (3); >70℃ (4) |
|  |  |  |  | pH (8) | 7~10 (2); 4~7 (4); <4 (6); >10 (8) |
|  |  |  |  | Hardness (5) | <100 mg/L (2); 100~300 mg/L (4); >300 mg/L (5) |
|  |  |  | Gas (40) | Water (10) | <5% (4); 5%~10% (6); 10%~15% (8); >15% (10) |
|  |  |  |  | Pressure (8) | <1MPa (1); 1~4 MPa (2); 4~10 MPa (4); 10~20 MPa (6); >20 MPa (8) |
|  |  |  |  | Sulfide (12) | <42.68mg/L (3); 42.68~142.26mg/L (6); >142.26 mg/L (12) |
|  |  |  |  | CO2 (10) | <0.207%mol/mol (3), 0.207%~2.073%mol/mol (7), >2.073% mol/mol (10). |
|  |  |  | Water (38) | Oil content (6) | <2 mg/L (2); 100~500 mg/L (4); 2~100 mg/L (6) |
|  |  |  |  | CI- (6) | <5 mg/L (2), 5~10 mg/L (4), >10 mg/L (6). |
|  |  |  |  | Salinity (9) | <1000mg/L (1), 1000~3000 mg/L (3), 3000~10000 mg/L (5), 10000~50000 mg/L (7), >50000 mg/L (9). |
|  |  |  |  | pH (5) | 7~10 (1); 4~7 (3); <4 (4); >10 (5) |
|  |  |  |  | Temperature (6) | <50℃ (2), >60℃ (3), 50~60℃ 6. |
|  |  |  |  | Pressure (6) | <1 MPa (1); 1~2.5 MPa (2); 2.5~16 MPa (3); 16~20 MPa (4); 20~25 MPa (5); >25 MPa (6) |
|  |  |  | Steam (38) | Pressure (9) | <2.5MPa (2); 2.5~10 MPa (4); 10~12 MPa (5) 12~14 MPa (6); >14 MPa (9) |
|  |  |  |  | CI- (12) | <5mg/L (4); 5~10mg/L (7); >10mg/L (12) |
|  |  |  |  | pH (9) | 7~8 (3); 6~7 (5); 5~6 (6); <5 (8); >8 (9) |
|  |  |  |  | Temperature (8) | <200℃ (3); 200~300℃ (5); >300℃ (8) |
|  | Other corrosion factors (17) |  | Pipe grade (3) |  | X70 (1); X52 (1.5); X42 (2); Lower grade (3) |
|  |  |  | Service years (8) |  | <5 years (2); 5~10 years (4); 10~20 years (6); >20 years (8) |
|  |  |  | Corrosion-induced failure history (6) |  | ≤1 (2); 1~3 (4); >3 (6) |
|  | Corrosion inspection (9) |  | External inspection (4) |  | <1 per 1 year (1); 1 per 1~2 years (2); 1 per > 2 years (4) |
|  |  |  | In-line inspection (5) |  | <1 per 1 year (2); 1 per 1~2 years (4); 1 per >2 years (5) |
| Total | 100 |  |  |  |  |

a For different media, the weight should be multiplied after the evaluation, e.g., it is necessary to multiply the weight 26/40 after evaluating the external corrosion of the heavy oil pipeline.

(c) Design

| First level | Second level | Weight | Description |
| --- | --- | --- | --- |
| Design | Pipeline strength | Safety design factor (34) | : >1.8 (0); 1.61~1.8 (3.5); 1.41~1.6 (7); 1.20~1.4 (14); 1.1~1.20 (21); 1.0~1.1 (28); <1 (34) |
|  | Water hammer | Elevation change (9) | Gently topography (0); Design pressure - MAOP > Maximum elevation pressure (4); Design pressure - MAOP < Maximum elevation pressure (9) |
|  |  | Protective (6) | Slow closing device (2); Pressure relief valve (3); Buffer tank (5); Operating procedures for preventing water hammer (6) |
|  | Integrity assurance | Usage time (10) | 0～5years (2), 5～15 years (4), 15～20 years (6), 20～25 years (8), >25 years (10). |
|  |  | Inspection time (7) | <1 year (1); 1~2 years (2); 2~3 years (3); 3~4 years (4); > 4 years (7) |
|  | Welding quality | Welding level (10) | Strictly follow the operation procedure, i.e., high quality (2); According to the operation procedure, i.e., average quality (6); Random welding, i.e., poor quality (10) |
|  |  | Inspection rate (10) | 100% (2); 80%~100% (6); 60%~80% (8); <60% (10) |
|  | Backfill quality | Backfill depth (8) | >76.2 cm (2); 7.62~76.2cm (4); <7.62cm or uncovered (8). |
|  |  | Backfilling method (6) | Both process and method are correct (2); Process is correct but the method is incrorrect (4); Both backfill process and method are incorrect (5); None (6). |
| Total |  | 100 |  |

(d) Misoperation

| First level | Second level | Weight | Description |  |
| --- | --- | --- | --- | --- |
| Misoperation | Misoperation during operation | Operating procedures (20) |  | Equipment operation, maintenance and calibration procedures are complete and strictly followed (5); Regulations have not been implemented (13); None (20) |
|  |  | Communication (10) |  | Dedicated communication tool (4); Communication equipment is not dedicated (6); Communication equipment failure (10) |
|  |  | Staff training 10 |  | Regular training (3); Occasional training (7); None (10) |
|  |  | Safety measures (10) |  | Safety responsibility system is sound and strictly implemented (3); There is a safety responsibility system but not implemented (7); None (10) |
|  | Maintenance misoperation | Maintain documentation (10) |  | Complete (3); Incomplete (6); None (10) |
|  |  | Maintenance measurement (10) |  | Replacement or no need for repairing (3); Repairment (5); Maintenance (8); None (10) |
|  |  | Maintenance plan (10) |  | Regular maintenance (3); Irregular maintenance (7); None (10) |
|  |  | Maintenance procedures (10) |  | Strict implementation of maintenance procedures (2); Maintenance procedures are not fully implemented (5); No procedures but maintenance records (8); None (10) |
|  |  | Maintenance personnel ability (10) |  | Maintenance personnel are of high quality and strong sense of responsibility, and no accident occurred (3); Maintenance personnel are competent for their jobs, and only one accident occurred (6); Maintenance personnel are unqualified and irresponsible and there were more than 2 liability accidents (10) |
| Total |  | 100 |  |  |

(e) Consequences of failure

| First level | Second level | Weight | Description |
| --- | --- | --- | --- |
| Consequences of failure | Medium hazard | Combustibility (20) | None (2); Flash point>93℃ (4); 38℃<Flash point>93℃ (8); Flash point<38℃ and ignition point<38℃ (12); Flash point<23℃ and ignition point<38℃ (20) |
|  |  | Toxic hazard (20) | None (2); Minor chronic injuries (5); To avoid temporary incapacity, medical measures must be taken immediately (10); Can cause severe temporary or sequelae injuries (15); Short-term exposure can cause death or serious injury (20) |
|  | Receptor | Population density (26) | Level 1 first class region (5); Level 1 second class area region (8); Level 2 region (15); Level 3 region (20); Level 4 region (26) |
|  |  | Environmental sensitivity a (22) | None (8); Vegetation or farmland (14); Culverts, rivers, fresh water, marshes and silted land, wetlands (18); Animal sanctuary, scenic spot or scenic area sanctuary (22) |
|  |  | Internal influence in oil and gas filed b (12) | Well-metering station/metering station-well/water supply-injection branch (6); Metering station-transfer station/transfer station-metering station/metering station-processing station/metering station-pull station/gathering station-transfer station (8); Transfer station-processing station/transfer station-transfer station/empty station-processing station/processing station-empty station (9); Oil depot-oil depot/oil depot-outside the station/processing station-oil depot (12) |
| Total |  | 100 |  |

a The range of influence is within 200m;

b Gathering pipeline connects the two stations or facilities, e.g., well-metering station represents the well and the metering station are connected by a pipeline.