Removal efficiencies of emissions of volatile organic compounds by adsorption systems installed in small-scale auto-repair painting operations

Haejoon Chun1,2, Min Young Song1*

1. Division of climate & Environmental Research, Seoul Institute of Technology, Seoul, 03909,

Republic of Korea

2. Department of Environmental Health Sciences, Graduate School of Public Health,

Seoul National University, Seoul, Korea

*Correspondence: mysong@sit.re.kr

Haejoon Chun (ORCID) 0000-0002-9602-7990

Min Young Song (ORCID) 0000-0003-4980-2023

Supplementary Information

Table S1 Conditions applied for GC/MS analysis

| MAKERS Unity-xr/Ultra-xr/AirServer-xr |  | GC/MSD (HP6890/5973, Hewlett Packard, USA) |  |
| --- | --- | --- | --- |
| Flow path temperature | 120 °C | GC column | DB-1(0.32 mm, 60 m, 3 µm) |
| Pre-purge time | 1.0 min | Initial temperature | 50 °C (10 min) |
| Desorb temperature | 300 °C | Oven ramp rate | 5 °C/min |
| Desorb time | 10 min | Final temperature | 220 °C (10 min) |
| Trap flow | 50 mL/min | Post run | 250 °C (5 min) |
| Trap purge time | 1 min | Column flow | 1.2 mL/min |
| Trap purge flow | 30 mL/min | Detector type | Quadrupole |
| Trap low temperature | −20 °C | Q-pole temperature | 150 °C |
| Trap high temperature | 300 °C | MS Source temperature | 230 °C |
| Trap desorb time | 4 min | Mass range | 35–350 amu |
| Split flow | 15 mL/min | Electron energy | 70 eV |

Table S2 Target VOCs for analysis

| TO-14 |  | Ozone precursor |  | CHEM |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | VOC species | No. | VOC species |  | No. | VOC species |  |  |
| 1 | Freon 12 | 1 | Isobutane |  | 1 | Methyl ethyl ketone |  |  |
| 2 | Methylchloride | 2 | 1-Butene |  | 2 | Isobutyl alcohol |  |  |
| 3 | Freon 114 | 3 | Butane |  | 3 | Benzene |  |  |
| 4 | Vinylchloride | 4 | trans-2-Butene |  | 4 | Methyl isobutyl ketone |  |  |
| 5 | 1,3-Butadiene | 5 | cis-2-Butene |  | 5 | Toluene |  |  |
| 6 | Methylbromide | 6 | Isopentane |  | 6 | Butyl Acetate |  |  |
| 7 | Ethylchloride | 7 | 1-Pentene |  | 7 | Ethylbenzene |  |  |
| 8 | Freon 11 | 8 | Pentane |  | 8 | m,p-Xylene |  |  |
| 9 | Acrylonitrile | 9 | Isoprene |  | 9 | Styrene |  |  |
| 10 | 1,1-Dichloroethene | 10 | trans-2-Pentene |  | 10 | o-Xylene |  |  |
| 11 | Dichloromethane | 11 | cis-2-Pentene |  | ※ Shaded: Repeated species |  |  |
| 12 | 3-Chloropropylene | 12 | 2,2-Dimethylbutane |  |  |  |  |
| 13 | Freon 113 | 13 | Cyclopentane |  |  |  |  |
| 14 | 1,1-Dichloroethane | 14 | 2-Methylpentane |  |  |  |  |
| 15 | cis-1,2-Dichloroethylene | 15 | 3-Methylpentane |  |  |  |  |
| 16 | Chloroform | 16 | 1-Hexene |  |  |  |  |
| 17 | 1,2-Dichloroethane | 17 | 2-Hexane |  |  |  |  |
| 18 | Methyl chloroform | 18 | Methylcyclopentane |  |  |  |  |
| 19 | Benzene | 19 | 2,4-Dimethylpentane |  |  |  |  |
| 20 | Carbontetrachloride | 20 | Benzene |  |  |  |  |
| 21 | 1,2-Dichloropropane | 21 | Cyclohexane |  |  |  |  |
| 22 | Trichloroethylene | 22 | 2-Methylhexane |  |  |  |  |
| 23 | cis 1,3-Dichloropropene | 23 | 2,3-Dimethylpentane |  |  |  |  |
| 24 | trans 1,3-Dichloropropene | 24 | 3-Methylhexane |  |  |  |  |
| 25 | 1,1,2-Trichloroethane | 25 | 2,2,4-Trimethylpentane |  |  |  |  |
| 26 | Toluene | 26 | Heptane |  |  |  |  |
| 27 | 1,2-Dibromoethane | 27 | Methylcyclohexane |  |  |  |  |
| 28 | Tetrachloroethylene | 28 | 2,3,4-Trimethylpentane |  |  |  |  |
| 29 | Chlorobenzene | 29 | Toluene |  |  |  |  |
| 30 | Ethylbenzene | 30 | 2-Methylheptane |  |  |  |  |
| 31 | m,p-Xylene | 31 | 3-Methylheptane |  |  |  |  |
| 32 | Styrene | 32 | Octane |  |  |  |  |
| 33 | 1,1,2,2-Tetrachloroethane | 33 | Ethylbenzene |  |  |  |  |
| 34 | o-Xylene | 34 | m,p-Xylene |  |  |  |  |
| 35 | 1-Ethyl-4-methylbenzene | 35 | Styrene |  |  |  |  |
| 36 | 1,3,5-Trimethylbenzene | 36 | o-Xylene |  |  |  |  |
| 37 | 1,2,4-Trimethylbenzene | 37 | Nonane |  |  |  |  |
| 38 | m-Dichlorobenzene | 38 | Isopropylbenzene |  |  |  |  |
| 39 | p-Dichlorobenzene | 39 | n-Propylbenzene |  |  |  |  |
| 40 | o-Dichlorobenzene | 40 | m-Ethyltoluene |  |  |  |  |
| 41 | 1,2,4-Trichlorobenzene | 41 | p-Ethyltoluene |  |  |  |  |
| 42 | Hexachloro 1,3-butadiene | 42 | 1,3,5-Trimethylbenzene |  |  |  |  |
|  |  | 43 | o-Ethyltoluene |  |  |  |  |
|  |  | 44 | 1,2,4-Trimethylbenzene |  |  |  |  |
|  |  | 45 | Decane |  |  |  |  |
|  |  | 46 | 1,2,3-Trimethylbenzene |  |  |  |  |
|  |  | 47 | m-Diethylbenzene |  |  |  |  |
|  |  | 48 | p-Diethylbenzene |  |  |  |  |
|  |  | 49 | n-Undecane |  |  |  |  |
|  |  | 50 | n-Dodecane |  |  |  |  |

Table S3 Information on operations and materials of the three auto-repair shops

|  | Old 1-1 |  | Old 1-2 |  | New 1-1 |  | New 1-2 |  | New 2-1 |  | New 2-2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Painting |  |  |  |  |  |  |  |  |  |  |  |
|  | Paint | Dilution | Paint | Dilution | Paint | Dilution | Paint | Dilution | Paint | Dilution | Paint | Dilution |
| Dilution ratio | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 3 | 1 | 2 | 1 |
| Paint usage(g) | 384.8 | 128.3 | 408.0 | 136.0 | 498.8 | 166.3 | 362.3 | 120.8 | 134.3 | 44.8 | 209.3 | 104.7 |
| Total Paint (g) | 513 |  | 544 |  | 665 |  | 483 |  | 179 |  | 314 |  |
| Operation time | 13 |  | 10 |  | 7 |  | 6 |  | 10 |  | 3 |  |
|  | Drying |  |  |  |  |  |  |  |  |  |  |  |
| Operation time | 30 |  |  |  |  |  |  |  |  |  |  |  |

Table S4 Concentrations of top five VOCs species

|  | Old 1-1 |  |  | Old 1-2 |  |  | New 1-1 |  |  | New 1-2 |  |  | New 2-1 |  |  | New 2-2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IN | OUT | IN |  | OUT | IN |  | OUT | IN |  | OUT | IN |  | OUT | IN |  | OUT |
| Butyl acetate | 1,231 | 1,478 | 1,256 |  | 2,465 | 2,503 |  | 2,081 | 2,997 |  | 2,338 | 1,034 |  | 743 | 1,256 |  | 902 |
| Toluene | 77 | 214 | 143 |  | 201 | 1,015 |  | 957 | 1,428 |  | 1,151 | 182 |  | 114 | 205 |  | 117 |
| 1,2,3-Trimethylbenzene | 198 | 221 | 280 |  | 290 | 311 |  | 234 | 831 |  | 288 | 78 |  | 61 | 209 |  | 96 |
| m,p-Xylene | 95 | 155 | 136 |  | 226 | 260 |  | 213 | 587 |  | 351 | 528 |  | 329 | 740 |  | 374 |
| Ethylbenzene | 45 | 121 | 73 |  | 135 | 283 |  | 199 | 589 |  | 350 | 123 |  | 83 | 158 |  | 95 |

Table S5 Removal efficiencies of top five VOCs species.

|  | Old 1-1 | Old 1-2 | New 1-1 | New 1-2 | New 2-1 | New 2-2 |
| --- | --- | --- | --- | --- | --- | --- |
| Butyl Acetate | −20.1 | −96.2 | 16.9 | 22.0 | 28.1 | 28.2 |
| Toluene | −178.4 | −40.4 | 5.8 | 19.4 | 37.3 | 42.9 |
| 1,2,3-Trimethylbenzene | −11.6 | −3.5 | 24.8 | 65.3 | 21.4 | 54.0 |
| m,p-Xylene | −64.1 | −66.0 | 18.1 | 40.2 | 37.8 | 49.4 |
| Ethylbenzene | −170.1 | −86.5 | 29.7 | 40.7 | 32.8 | 40.2 |