Supplementary Material

Spatial distribution, sources, and risk assessment of metal ions in sediments from industrial areas in the Kuye River Basin in Shenmu, China

Zhu Wanga,b, Yong Jun Liua,b,*, Chun Xiao Weia,b, Xing She Liua,b, Zhuang Zhuang Yanga,b, Lu Yanga,b, Ai Ning Zhanga,b, Zhi Hua Lia,b

a School of Environmental and Municipal Engineering, Xi’an University of Architecture and Technology, Yan Ta Road. No.13, Xi’an 710055, China

b Key Lab of Northwest Water Resource, Environment and Ecology, Ministry of Education, Xi’an University of Architecture and Technology, Xi’an 710055, China

*Corresponding author: Yongjun Liu

School of Environmental and Municipal Engineering, Xi’an University of Architecture and Technology, Yan Ta Road. No.13, Xi’an 710055, China

E-mail address: liuyongjun@xauat.edu.cn (YongJun Liu); Tel: +86 13572535980

Table S1：The specific coordinates of the sampling site

| Site | Longitude | Latitude | Elevation |
| --- | --- | --- | --- |
| M1 | 110.221 | 39.043 | 1161.532 |
| M2 | 110.264 | 39.034 | 1139.156 |
| M3 | 110.291 | 39.043 | 1117.398 |
| M4 | 110.333 | 39.054 | 1074.345 |
| M5 | 110.353 | 39.052 | 1073.950 |
| M6 | 110.390 | 39.037 | 1026.548 |
| M7 | 110.421 | 39.011 | 994.559 |
| M8 | 110.432 | 39.003 | 982.156 |
| M9 | 110.440 | 39.005 | 976.999 |

Table S2: The correlation between metal ion EF, Igeo, PLI, and pollution level (Khadanga et al., 2022; Lin et al., 2022; Rahman et al., 2022)

| EF |  | Igeo |  | PLI |  |
| --- | --- | --- | --- | --- | --- |
| EF<1 | No enrichment | Igeo<0 | Practically uncontaminated | PLI < 1 | Unpolluted |
| EF<3 | Minor enrichment | 0<Igeo<1 | Unpolluted to moderately contaminated | 1 ≤ PLI < 2 | Moderate |
| 3<EF<5 | Moderate enrichment | 1<Igeo<2 | Moderately contaminated | 2 ≤ PLI < 3 | High |
| 5<EF<10 | Moderately severe enrichment | 2<Igeo<3 | Moderately to strongly contaminated | 3 ≤ PLI < 5 | Significantly high |
| 10<EF<25 | Severe enrichment | 3<Igeo<4 | Strongly contaminated | PLI≥5 | Extremely polluted |
| 25<EF<50 | Very severe enrichment | 4<Igeo<5 | Strongly to extremely contaminated | — | — |
| EF>50 | Extremely severe enrichment | Igeo>5 | Extremely contaminated | — | — |

Table S3: Toxicity coefficients of metal ions (Astuti et al., 2022; Lin, 2009; Zhang et al., 2018)

|  | Ag | As | Cd | Cr | Cu | Pb | Ni | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tri | 15 | 10 | 30 | 2 | 5 | 5 | 5 | 1 |

Table S4: Pollution level of Cfi and potential ecological risk level to Ei r and RI of metals ions (Hakanson, 1980; Rahman et al., 2022; Wang et al., 2022)

| single–factor pollution coefficient Cfi | Pollution level of Cfi | Individual Potential ecological risk coefficient, Ei r | Potential ecological risk level of Ei r | The sum of potential ecological risk index, RI | Potential ecological risk level of RI |
| --- | --- | --- | --- | --- | --- |
| Ci f< 0.7 | Clean | Ei r< 40 | Low | RI <150 | Low |
| 0.1 ≤ Ci f< 1 | Basically clean | 40 ≤ Ei r< 80 | Moderate | 150 ≤ RI <300 | Moderate |
| 1 ≤ Ci f< 2 | Low | 80 ≤ Ei r< 160 | High | 300 ≤ RI <600 | High |
| 2 ≤ Ci f< 3 | Moderate | 160 ≤ Ei r< 320 | Significantly high | 600 ≤ RI | Significantly high |
| 3 ≤ Ci f< 6 | High | 320 ≤ Ei r | Extremely polluted | — | — |
| 6 ≤ Ci f | Significantly high | — | — | — | — |

Table S5: Significance and value of human health risk assessment calculation model variables

| Exposure parameter | Meaning | Unit | Value |
| --- | --- | --- | --- |
| ADDinh/ing/der | exposure doses through inhalation, ingestion, and dermal contact | mg/(kg·d) |  |
| C | concentration of pollutant in soil | ng/g |  |
| IngR | ingestion rate of soil | mg/day | 14.5 for adults and 7.5 for children |
| EF | soil exposure frequency | d/year | 350 d/year |
| ED | exposure duration | year | 6 years for children and 26 years for adults |
| BW | body weight | kg | children: 15.9 kg, adults: 56.8 kg |
| AT | average time | day | 365×30 for children and 365×70 for adults |
| SA | surface area of the skin | cm2 | 5700 for adult and 2800 for children |
| AF | skin adherence factor for soil | mg/cm2 | 0.2 mg/cm2 |
| ABS | dermal absorption factor |  | set at 0.001 |
| PEF | particle emission factor | m3/kg | 1.36×109 m3/kg |
| SF | cancer slope factor | kg·d/mg | Table S6 |
| RfDinh/ing/der | corresponding reference doses through ingestion, dermal contact and inhalation | mg/kg/d | Table S6 |
| InhR | contact rate | m3/d | 20 m3/d for adults and 7.6 m3/d for children |

Table S6: Absorption reference dose and carcinogenic slope factor of each metal ion (Ministry of Ecology and Environment, PRC, 2019; USEPA, 2016)

| Heavy metal |  | Absorbed reference dose |  | Carcinogenic slope factor |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RfDing (mg/kg/d) | RfDinh (mg/kg/d) | RfDder (mg/kg/d) | SFing (kg·d/mg) | SFinh (kg·d/mg) | SFder (kg·d/mg) |
| As | 3.00E-04 | 8.60E-06 | 1.23E-04 | 1.50E+00 | 1.50E+01 | 3.66E+00 |
| Cd | 1.00E-03 | 1.00E-03 | 1.00E-05 | 3.80E-01 | 6.30E+00 | 3.00E+00 |
| Ni | 2.00E-02 | 2.60E-05 | 5.40E-03 | 8.40E-01 | 1.70E+00 | 4.25E+01 |
| Cr | 3.00E-03 | 2.90E-05 | 1.50E+00 | 5.00E-01 | 4.20E+01 | 2.00E+01 |
| Zn | 3.00E-01 | 3.00E-01 | 6.00E-02 | — | — | — |
| Cu | 4.00E-02 | 4.02E-02 | 1.20E-02 | — | — | — |
| Pb | 3.50E-03 | 3.52E-03 | 5.25E-04 | — | — | — |
| Ag | — | — | — | — | — | — |

Table S7: Pollution load index for each site

| Site | M1 | M2 | M3 | M4 | M5 | M6 | M7 | M8 | M9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PLI | 1.378 | 1.233 | 1.245 | 1.100 | 0.961 | 0.998 | 1.122 | 1.1421 | 1.836 |

Fig S1: The percentage of metal ions

Table S8: Total variance explained and component matrixes for heavy metal contents

| Element | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Ag | 0.138 | -0.87 | 0.025 |
| As | -0.279 | 0.811 | -0.312 |
| Cd | 0.991 | 0.104 | -0.059 |
| Cr | 0.097 | 0.169 | 0.925 |
| Cu | 0.022 | -0.306 | -0.281 |
| Ni | 0.951 | -0.024 | -0.212 |
| Pb | 0.983 | -0.002 | -0.022 |
| Zn | 0.91 | 0.283 | 0.118 |
| Eigenvalue | 3.79 | 1.63 | 1.09 |
| % of variance explained | 47.35% | 20.36% | 13.69% |
| % of cumulative | 47.35% | 67.71% | 81.40% |

Table S9: Ci f and Ei r of each metal and Bacteria count

| Site | Ag |  | Cd |  | Cr |  | As |  | Cu |  | Pb |  | Zn |  | Ni |  | Bacterial Count (PCS/ml) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ci f | Ei r | Ci f | Ei r | Ci f | Ei r | Ci f | Ei r | Ci f | Ei r | Ci f | Ei r | Ci f | Ei r | Ci f | Ei r |  |
| M1 | 1.064 | 15.966 | 0.593 | 17.784 | 2.131 | 4.262 | 3.104 | 31.036 | 1.447 | 7.233 | 1.163 | 5.817 | 1.624 | 1.624 | 5.688 | 5.688 | 38000 |
| M2 | 1.534 | 23.011 | 0.593 | 17.784 | 1.517 | 3.034 | 1.121 | 11.214 | 1.469 | 7.343 | 1.273 | 6.365 | 1.492 | 1.492 | 6.208 | 6.208 | 2100 |
| M3 | 1.159 | 17.386 | 0.593 | 17.784 | 0.507 | 1.013 | 2.935 | 29.348 | 2.060 | 10.298 | 1.131 | 5.654 | 1.434 | 1.434 | 8.476 | 8.476 | 30400 |
| M4 | 1.117 | 16.761 | 0.572 | 17.165 | 1.094 | 2.189 | 2.808 | 28.080 | 1.692 | 8.462 | 1.163 | 5.817 | 1.196 | 1.196 | 2.323 | 2.323 | 1900 |
| M5 | 1.155 | 17.330 | 0.567 | 17.010 | 0.908 | 1.816 | 1.011 | 10.107 | 0.950 | 4.750 | 1.169 | 5.846 | 1.073 | 1.073 | 5.093 | 5.093 | 18200 |
| M6 | 1.345 | 20.170 | 0.562 | 16.856 | 0.370 | 0.741 | 1.318 | 13.179 | 2.268 | 11.341 | 1.160 | 5.798 | 0.919 | 0.919 | 5.502 | 5.502 | 5400 |
| M7 | 1.273 | 19.091 | 0.562 | 16.856 | 1.049 | 2.098 | 1.211 | 12.107 | 1.847 | 9.235 | 1.246 | 6.231 | 0.764 | 0.764 | 7.881 | 7.881 | 1600 |
| M8 | 1.216 | 18.239 | 0.557 | 16.701 | 2.516 | 5.033 | 0.786 | 7.857 | 2.294 | 11.470 | 1.256 | 6.279 | 1.202 | 1.202 | 3.123 | 3.123 | 3400 |
| M9 | 1.254 | 18.807 | 0.809 | 24.278 | 1.369 | 2.738 | 0.964 | 9.643 | 1.821 | 9.107 | 4.935 | 24.673 | 2.611 | 2.611 | 20.520 | 20.520 | 1000 |

Reference

Astuti, R.D.P., Mallongi, A., Choi, K., Amiruddin, R., Hatta, M., Tantrakarnapa, K., Rauf, A.U. (2022) Health risks from multiroute exposure of potentially toxic elements in a coastal community: a probabilistic risk approach in Pangkep Regency, Indonesia. Geomatics Natural Hazards & Risk 13, 705-735. https://doi.org/10.1080/19475705.2022.2041110.

Hakanson, L. (1980) An ecological risk index for aquatic pollution control.a sedimentological approach. Water Research 14, 975-1001. https://doi.org/10.1016/0043-1354(80)90143-8.

Khadanga, M.K., Mishra, R.K., Sahu, B.K. (2022) Assessment of Pollution and Ecological Risk Index of Heavy Metals in the Surface Sediment of Estuary and the Coastal Environment of Bay of Bengal. Journal of Environmental Informatics 39, 35-48. https://doi.org/10.3808/jei.202100469.

Lin, K.-N., Lim, Y.-C., Chen, C.-W., Chen, C.-F., Kao, C.-M., Dong, C.-D. (2022) Spatiotemporal Variation and Ecological Risk Assessment of Heavy Metals in Industrialized Urban River Sediments: Fengshan River in Southern Taiwan as a Case Study. Applied Sciences-Basel 12, 1013. https://doi.org/10.3390/app12031013.

Lin, L.Q. (2009) Assessment of heavy metal toxicity using toxicological safety assessment data, International Symposium on Heavy metal Pollution monitoring, Risk Assessment and Remediation technology, 60-63.                https://kns.cnki.net/kcms/detail/detail.aspx?FileName=BJCX200908003010&DbName=CPFD201.

Ministry of Ecology and Environment, PRC (2019). Technical guidelines for risk assessment of soil contamination of land for construction. from https://www.mee.gov.cn/ywgz/fgbz/bz/bzwb/trhj/201912/t20191224_749893.shtml.

Rahman, M., Saima, J., Rima, S.A., Hossain, M.I.S., Das, D.K., Bakar, M.A., Siddique, M.A.M. (2022) Ecological risks of heavy metals on surficial sediment of Nijhum Dweep (Island), an important biodiversity area of Bangladesh. Marine Pollution Bulletin 179, 113688.   https://doi.org/10.1016/j.marpolbul.2022.113688.

USEPA (2016). Regional Screening Levels (RSLs) User’s Guide. from https://www.epa.gov/expobox/about-exposure-factors-handbook.

Wang, Q., Duan, W., Yao, X., Guo, X., Liu, D., Gao, W., Zhang, J. (2022) Distribution and ecological risk assessment of heavy metals in the sediments of Changli ecological monitoring area, northwest of Bohai Bay, China. Environmental Pollutants and Bioavailability 34, 180-189. https://doi.org/10.1080/26395940.2022.2069599.

Zhang, Z., Lu, Y., Li, H., Tu, Y., Liu, B., Yang, Z. (2018) Assessment of heavy metal contamination, distribution and source identification in the sediments from the Zijiang River, China. Science of the Total Environment 645, 235-243. https://doi.org/10.1016/j.scitotenv.2018.07.026.