| Application | Total HPLC run time for the cited drugs | Elution mode | LOQ (μg/mL) |  |  | LOD (μg/mL) |  |  | Linearity range (μg/mL) |  |  | Parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MET | LIN | EMP | MET | LIN | EMP | MET | LIN | EMP |  |
| Real Trijardy® tablet Impurity profiling Green analysis | 3.92 | Isocratic | 0.71 | 0.27 | 0.15 | 0.23 | 0.09 | 0.05 | 1.0-250.0 | 0.3-9.0 | 0.2-8.0 | Current HPLC methoda |
| Laboratory synthetic tablet not a real tablet Impurity profiling Green analysis | 8.53 | Gradient | 8.93 | 3 7.62 | 27.52 | 2.68 | 2.29 | 8.26 | 15–300 | 12.5–200 | 30–400 | Reported HPLC method A [21] |
| Single tablets from different companies not combined dosage form | 5.99 | Gradient | 174.32 | 3.43 | 5.32 | 57.32 | 1.10 | 1.32 | 500.0–3500.0 | 10.0-70.0 | 20.0-140.0 | Reported HPLC method B [15] |
| Quality by design no tablet or other applications | 8.42 | Gradient | 2.98 | 26.87 | 16.05 | 0.98 | 8.77 | 5.29 | 10.0-70.0 | 10.0-70.0 | 30.0-210.0 | Reported HPLC method C [18] |

Table S1 : Comparison between the current HPLC method for the determination of EMP, LIN, and MET along with MEL and CYG as MET potential official impurities and three other published HPLC methods in terms of sensitivity and simplicity.

a The most sensitive and simplest method

EMP, empagliflzin; LIN, linagliptin; MET, metformin

| EMP | EMP found | % Recovery a of EMP | LIN | LIN found | % Recovery a of LIN | MET | MET found | % Recovery a of MET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.30 | 0.30 | 100.23 | 0.50 | 0.49 | 99.50 | 4.00 | 4.03 | 100.76 |
| 0.50 | 0.50 | 101.56 | 2.00 | 1.99 | 99.58 | 8.00 | 8.17 | 102.19 |
| 2.00 | 1.96 | 98.40 | 4.00 | 3.93 | 98.32 | 15.00 | 14.98 | 99.92 |
| 5.00 | 4.95 | 99.13 | 5.00 | 4.92 | 98.42 | 30.00 | 29.66 | 98.87 |
| 7.00 | 6.92 | 98.95 | 7.00 | 7.06 | 100.94 | 150.00 | 150.63 | 100.42 |
| Mean ± SD |  | 99.56±1.257 | Mean ± SD |  | 99.35±1.056 | Mean ± SD |  | 100.43±1.217 |

Table S2: Accuracy results of the current HPLC method for the determination of EMP, LIN, and MET along with MEL and CYG as MET potential official impurities.

a Mean of three determinations

EMP, empagliflzin; LIN, linagliptin; MET, metformin.

Table S3 : Results a of robustness testing of the current HPLC method for the determination of EMP, LIN, and MET along with MEL and CYG as MET potential official impurities.

| Condition | Drug | Rt | N | T | RS |
| --- | --- | --- | --- | --- | --- |
| Change mobile phase composition acetonitrile: 0.05 M phosphate buffer (91:9 v/v), (90:10, v/v) | MET | 0.456 | 1.344 | 0.546 | 1.123 1.234 0.765 0.743 |
|  | MEL | 0.467 | 0.675 | 0.975 |  |
|  | CYG | 1.766 | 0.986 | 1.546 |  |
|  | LIN | 0.976 | 0.754 | 0.542 |  |
|  | EMP | 0.876 | 0.459 | 0.832 |  |
| Change pH of buffer pH (4.0±0.1;4.1 and 3.9) | MET | 0.654 | 0.560 | 1.241 | 0.676 1.543 0.655 1.534 |
|  | MEL | 0.654 | 1.854 | 1.356 |  |
|  | CYG | 0.457 | 1.534 | 1.656 |  |
|  | LIN | 0.986 | 1.256 | 1.876 |  |
|  | EMP | 1.543 | 1.745 | 1.345 |  |

a % RSD values

Rt, retention time; N, number of theoretical plates; T, tailing factor; Rs, resolution.

MET, metformin; MEL, melamine; CYG, cyanoguanidine; LIN, linagliptin; EMP, empagliflozin.