Appendix A1

Section 1: CT Image Acquisition Parameters

Site 1: All patients (Zhangzhou Affiliated of Fujian Medical University) underwent an non-contrast CT scan on a 16-slice SOMATOM scanner [Emotion 16; Shanghai, China].The acquisition parameters were as follows: 130kVp exposure; automatic tube current (90-120 mA);1.25-5 mm slice thickness; and matrix, 512 × 512( transverse plane) 643 × 512( coronal plane), 512 × 541(sagittal plane). The images are photographed at lung (1600 HU window width; -650 HU window level) and mediastina (350 HU window width; 50 HU window level) settings.

Site 2: All patients (Nanpin Affiliated of Fujian Medical University) underwent an non-contrast CT scan on a 64-slice SOMATOM Definition Flash scanner (Siemens AG Healthcare Sector, Erlangen ,Germany). The acquisition parameters were as follows: 120kVp exposure; automatic tube current (90-120 mA); 1.5-5 mm slice thickness; and matrix, 512 × 512( transverse plane) 484 × 484 ( coronal plane), 584 × 584 (sagittal plane). The images are photographed at lung (1200 HU window width; -600 HU window level) and mediastina (350 HU window width; 50 HU window level) settings.

Site 3: All patients (Affiliated Fuzhou First Hospital of Fujian Medical University) underwent an non-contrast CT scan on a 64-slice SOMATOM Definition Flash scanner (Siemens AG Healthcare Sector, Erlangen ,Germany). The acquisition parameters were as follows: 120kVp exposure; automatic tube current (90-120 mA); 1.5-5 mm slice thickness; and matrix, 512 × 512( transverse plane) 484 × 484 ( coronal plane), 584 × 584 (sagittal plane). The images are photographed at lung (1200 HU window width; -600 HU window level) and mediastina (350 HU window width; 50 HU window level) settings.

Section 2: CT Image segmentation of ROI

All patients’ pulmonary windows images were separated into transverse planes, coronal planes and sagittal planes by manual. To reduce the spending of manual segmentation lesions, the lung parenchyma as region of interest (ROI) was automatically segmented by a hybrid method of fuzzy c-means and gray-level co-occurrence matrix. It was performed by using MATLAB software (version 2020a; The MathWorks Inc., Natick, MA, USA).The process of segmentation of ROI as follows:

Step 1 batch input CT original image  as X with gray-level matrix.

Step 2 use fuzzy c-means to cluster the X matrix as 2 clusters, one cluster was background area, another was the chest area(X1). Then extract the whole chest area(X1) by max connected domain method. Homomorphic filtering was used to reduce the difference light caused by lung lesions.

Step 3 Calculate the gray-level co-occurrence matrix (GLCM) of the whole chest area(X1) and return the scaled version of the image(SI), used by gray-level co-occurrence matrix (References[1][2]). We found when SI=3 and 4 can display the lung parenchyma area.

Step 3 use infill and morphology method (parameter: ‘disk’, 4 ) ,  extracted  the max 2 areas (transverse plane, coronal plane ) and the max area(sagittal plane) , and then used infill and morphology method (parameter: ‘disk’, 3 ) again to segment the lung parenchyma area(X2).

Step 4  reshape the image matrix X2 into 512× 512 size and output the ROI.

To verify the validity and accuracy of the method, we performed on the Kaggle’s lung segmentation database and a private lung database from Affiliated Fuzhou First Hospital of Fujian Medical University. The performances of this method compared with Region Growing method were in the following Table1:

Table1 Comparison of two methods based on Kaggle database and private database

| Private Database |  | ACC | SEN | SPE | DSI |
| --- | --- | --- | --- | --- | --- |
| Transverse Plane | Our Method | 0.9830 | 0.9955 | 0.9814 | 0.9230 |
|  | Region Growing Method | 0.9825 | 0.9363 | 0.9874 | 0.9092 |
| Coronal Plane | Our Method | 0.9775 | 0.9913 | 0.9742 | 0.9132 |
|  | Region Growing Method | 0.9803 | 0.9594 | 0.9838 | 0.9221 |
| Sagittal Plane | Our Method | 0.9910 | 0.9997 | 0.9898 | 0.9629 |
|  | Region Growing Method | 0.9941 | 0.9774 | 0.9963 | 0.9775 |
| Kaggle Database |  |  |  |  |  |
| Transverse Planes | Our Method | 0.9811 | 0.9868 | 0.9792 | 0.9574 |
|  | Region Growing Method | 0.9877 | 0.9857 | 0.9884 | 0.9757 |

ACC: Accuracy; SEN: Sensitivity; SPE: Specificity; DSI: Dice similarity index.

We found the sensitivity of our method were great than that of the region growing method, it mean that the pixels of the lung were more correctly segmented. And the region growing method needs a seed point by manual; it is not as convenient to operate as our unsupervised method. The average time of segmenting a CT image based on our method is 0.3151s，the average time of segmenting a CT image based on region growing method is 3.5518s.

The two radiologists assessed the results of segmentation and reached a consensus by discussion.  But there also were some errors about the segmentation. The errors of the segmentation in the following example:

These segmentations of images were all from the same patient, because the patient had a big range of lesion, so the segmentation of whole lung could not do well. This might infect the result of the radiomic quantitative analysis. In the future, we will continue to improve the segmentation technology of the whole lung.

Code of step2-step4 as follows:

function [pre_img2]=segepicture_szgz(img)

img1=im2double(img);

[s1 s2]=size(img1);

data=reshape(img1,[s1*s2 1]);

[center,U,obj_fcn]=fcm(data,2);% fcm

[B,IX]=sort(center,'descend');%

t=IX(1);

u1=U(t,:);

u1=reshape(u1,[s1 s2]);

%se1=strel('disk',3);

%A1=imerode(u1,se1);

imBw1 = im2bw(u1);

imLabel1 = bwlabel(imBw1);

stats1 = regionprops(imLabel1,'Area');

area1 = cat(1,stats1.Area);

index1 = find(area1 == max(area1));

img2 = ismember(imLabel1,index1);

imgg=imfill(img2, 'holes');

idx=find(imgg==0);img1(idx)=0;

pre_img=img1;

%% Homomorphic filtering

pre_img=tongtailb(pre_img);

%% lung 2

[glcm, SI] = graycomatrix(pre_img); %glcm

nn1=bwlabel(SI==3); nn2=bwlabel(SI==4); nn3=bwlabel(SI==5); nn4=bwlabel(SI==6);nn5=bwlabel(SI==7);

NN=nn3+nn4+nn5; %kaggle :3456nn1+nn2+ Fuzhou first hospital：567 covid:567

x2=imfill(NN, 'holes');

imBw = im2bw(x2);

imLabel = bwlabel(imBw );

stats = regionprops(imLabel,'Area');

area = cat(1,stats.Area);

[B1,IX1]=sort(area,'descend');

if length(B1)>1

index2 = find(area == max(area));

index3 = find(area == B1(2));

img4 = ismember(imLabel,index2);

img5 = ismember(imLabel,index3);

img6=img4+img5;

else

index2 = find(area == max(area));

img4 = ismember(imLabel,index2);

img6=img4;

end

img6=imfill(img6, 'holes'); %

se2=strel('disk',1);% img6=imdilate(img6,se2);

img6= bwareaopen(img6,500);

idx2=find(img6==0);

pre_img2=img1;pre_img2(idx2)=0; %segment lung

end%

References

[1] Haralick, R.M., K. Shanmugan, and I. Dinstein, "Textural Features for Image Classification", IEEE Transactions on Systems, Man, and   Cybernetics, Vol. SMC-3, 1973, pp. 610-621.

[2] Haralick, R.M., and L.G. Shapiro. Computer and Robot Vision: Vol. 1,  Addison-Wesley, 1992, p. 459.

Section 3: radiomics features extracted

The following radiomic features were extracted from the ROI region (lung parenchyma):

(a) Hu invariant moment features, (b) Gray-level co-occurrence matrix(GLCM) features,(c)Local entropy features,(d) Statistics features including shape, sizes, extremes and so on, (e) Gray-level run-length matrix (GLRLM) features, (f)Entropy feature, (g)Gabor wavelet filter features, (h)Hessian matrix features, (i) Histogram of Oriented Gradients (HOG) features, (j)Phase congruency features. Feature extraction was executed using MATLAB software (version 2020a; The MathWorks Inc., Natick, MA, USA). And these codes of feature extraction can be download from the internet. Some parameters were set as follow:

Category: HU invariant moment features (7 descriptors)

For the integral area of S , if we have a given two-dimensional continuous function f(x,y), the (p+q)  order moment is defined as follows,

mpq=xpyqfx,ydxdy,p,q=0,1,2,…  (1)

If the digital image f(x,y) is a piecewise continuous function, and has non-zero value in a limited part of the xy plane, then we can prove the existence of various order moments. And the moment sequence mpq could be uniquely determined by the image f(x,y). At the same time the moment sequence mpq also uniquely determine the image f(x,y). The central moments can be expressed as follows,

upq=x-xcpy-ycqfx,ydxdy (2)

For digital images, the above integral can be replaced by the following formula,

upq=xyx-xcpy-ycqfx,y (3)

The Standardized central moment is ηp,q , which can be expressed mathematically as

follows,

ηp,q=μp,qμ0,0γ, where γ=p+q+22,p+q=2,3,…(4)

Seven common invariant moments are presented. From the above equation (1), (2) we could

get seven Hu moment invariants, which can be expressed as follows,

| Variable | Description | Parameters |
| --- | --- | --- |
| HU invariant moment1 | ϕ1=η20+η02 | N/A |
| HU invariant moment2 | ϕ2=(η20-η02)2+4η112 | N/A |
| HU invariant moment3 | ϕ3=(η30-3η12)2+(3η12-η03)2 | N/A |
| HU invariant moment4 | ϕ4=(η30+η12)2+(η21+η03)2 | N/A |
| HU invariant moment5 | ϕ5=(η30-3η12)η30+η12[(η30+η12)2-3η21+η032] +(3η21-η03)(η21+η03)[3η30+η122-η21+η032] | N/A |
| HU invariant moment6 | ϕ6=(η20-η02)[(η30+η12)2-η21+η032] +4η11(η30+η12)(η21+η03)] | N/A |
| HU invariant moment7 | ϕ7=(3η21-η03)η30+η12[(η30+η12)2-3η21+η032] +(η03-η12)(η21+η03)[3η30+η122-η21+η032] | N/A |

Category: Gray-Level Co-occurrence Matrix features (23 descriptors)

| Variable | Description |
| --- | --- |
| Auto Correlation | N/A |
| Cluster Prominen | N/A |
| Cluster Shade | N/A |
| Cluster Tendendcy | N/A |
| Contrast | For the feature description, refer to the documentation on MATLAB function "gray co-props". |
| Correlation | For the feature description, refer to the documentation on MATLAB function "gray co-props". |
| Difference Entropy | N/A |
| Dissimilarity |  |
| Energy | For the feature description, refer to the documentation on MATLAB function "graycoprops". |
| Entropy |  |
| Homogeneity | 1.This feature is equivalent to Homogeneity1 in Hugo's paper. 2. For the feature description, refer to the documentation on MATLAB function "gray co-props". |
| Homogeneity2 | N/A |
| InformationMeasureCorr1 | N/A |
| InformationMeasureCorr2 | N/A |
| Inverse Diff Moment Norm | N/A |
| Inverse Diff Norm | N/A |
| Inverse Variance | N/A |
| Max Probability | N/A |
| Sum Average | N/A |
| Sum Entropy | N/A |
| Sum Variance | N/A |
| Variance | N/A |

Category: Local  entropy  and entropy features (23 descriptors)

| Variable | Description | Parameters |
| --- | --- | --- |
| Local Entropy Max | 1. First, at each pixel, compute entropy in its neighborhood region. 2. Then, compute the maximum among all the pixel's entropy calculated from 1. | NHood=9; RangeMin=0; RangeMax=4096; |
| Local Entropy Mean | 1. First, at each pixel, compute entropy in its neighborhood region. 2. Then, compute the mean among all the pixel's entropy calculated from 1. | NHood=9; RangeMin=0; RangeMax=4096; |
| Local Entropy Hist | 1. First, at each pixel, compute entropy in its neighborhood region. 2. Then, compute the histogram among all the pixel's entropy calculated from 1. | Scalar=20 |
| Energy | compute entropy in whole image region at each pixel. | N/A |

Category: statistics features (23 descriptors)

| Variable | Description |
| --- | --- |
| stats_Area | The total number of pixels in each region of the ROI |
| stats_MajorAxisLength | The major axis length of an ellipse with the same standard second order central moment as the region (in pixel) |
| stats_MinorAxisLength | The minor axis length of an ellipse with the same standard second-order central moment as the region (in pixel) |
| stats_Eccentricity | foci of the ellipse and to major axis length |
| stats_Orientation | The intersection Angle (degree) of the major axis and the X axis of an ellipse with the same standard second order central moment as the region |
| stats_ConvexArea | The number of pixels in a convex polygon image filled with a region |
| stats_FilledArea | The number of pixels in the image of the filled region |
| stats_EulerNumber | euler number |
| stats_EquivDiameter | Diameter of circle with same area of region |
| stats_Solidity | At the same time in the region and its minimum convex polygon pixel ratio |
| stats_Extent | The ratio of pixels in both the region and its minimum bounding box |
| stats_Perimeter | Perimeter of region |
| stats_PerimeterOld | Perimeter of original region |
| stats_ThinnessRatio | thinness ratio of region |
| stats_AspectRatio | Aspect Ratio of region |
| stats_Extrema1 | One of Eight directional region extremum points |
| stats_Extrema2 | One of Eight directional region extremum points |
| stats_Extrema3 | One of Eight directional region extremum points |
| stats_Extrema4 | One of Eight directional region extremum points |
| stats_Extrema5 | One of Eight directional region extremum points |
| stats_Extrema6 | One of Eight directional region extremum points |
| stats_Extrema7 | One of Eight directional region extremum points |
| stats_Extrema8 | One of Eight directional region extremum points |

Category: Gray Level Run Length Matrix in 4 direction and mean of 4 directions (55 descriptors)

Variable

Gray Level Non uniformity

High Gray Level Run Empha

Long Run Emphasis

Long Run High Gray Level Empha

Long Run Low Gray Level Empha

Low Gray Level Run Empha

Run Length Non uniformity

Run Percentage

Short Run Emphasis

Short Run High Gray Level Empha

Short Run Low Gray Level Empha

Category: Gabor wavelet filter features (12 descriptors)

| Variable | Description | Parameters |
| --- | --- | --- |
| Gabor wavelet | N/A | Scale=2 Orientation=12(θ = π/6, π/3, π/2, 2π/3, 5π/6, π, 7π/6, 4π/3, 3π/2, 5π/3, 11π/6, 2π). |

Category: Hessian matrix  features(23  descriptors)

| Variable | Description | Parameters |
| --- | --- | --- |
| Hessian matrix max | - first compute the Hessian detector on the gray-scale image IM - then compute the max of Hessian matrix. | N/A |
| Hessian matrix mean | 1. first compute the Hessian detector on the gray-scale image IM 2. then compute the mean of Hessian matrix | N/A |
| Hessian matrix hist | 1. first compute the Hessian detector on the gray-scale image IM 2. then compute the histogram among Hessian matrix | Scaler=20 |
| Hessian matrix min | 1. first compute the Hessian detector on the gray-scale image IM 2. then compute the min of Hessian matrix | N/A |

Category: Histogram of Oriented Gradients (HOG) features, (22  descriptors)

| Variable | Description | Parameters |
| --- | --- | --- |
| HOG max | 1.from a grayscale image I and returns the features in a 1-by-N vector. These features encode local shape information from regions 2. then compute the max of HOG matrix | N/A |
| HOG mean | 1.from a grayscale image I and returns the features in a 1-by-N vector. These features encode local shape information from regions 2. then compute the mean of HOG matrix | N/A |
| HOG hist | 1.from a grayscale image I and returns the features in a 1-by-N vector. These features encode local shape information from regions 2.then compute the histogram among HOG matrix | Scaler=20 |

Category: Phase congruency features. (23  descriptors)

| Variable | Description | Parameters |
| --- | --- | --- |
| Phase congruency max | 1. computer the Phase congruency of region 2. then compute the max of Phase congruency matrix | N/A |
| Phase congruency mean | 1.computer the Phase congruency of region 2. then compute the mean of Phase congruency matrix | N/A |
| Phase congruency hist | 1.computer the Phase congruency of region 2. then compute the histogram among Phase congruency matrix | Scaler=20 |
| Phase congruency min | 1.computer the Phase congruency of region 2. then compute the min of Phase congruency matrix | N/A |

References:

[1] Peter Kovesi, "Image Features From Phase Congruency". Videre: A Journal of Computer Vision Research. MIT Press. Volume 1, Number 3, Summer 1999 http://mitpress.mit.edu/e-journals/Videre/001/v13.html

[2] Peter Kovesi, "Phase Congruency Detects Corners and Edges". Proceedings DICTA 2003, Sydney Dec 10-12

Section 4: Parameters of radiomics models based on machine learning methods

SVM:  We utilize the svm method to construct the radiomics model. The optimal hype-parameters were set as follows: kernel function(Gaussian radial basis function,-t 2), gamma value(-g 0.1),  cost value(-c 0.2).

BP: In this study, the sizes of hidden layers was 2, and the nodes were 300, transfer function of the hidden layer chose ‘logsig’, output layer chose ‘purelin’, backprop network training function chose ‘traingdx’, the number of epochs was 500.

DELM: Deep extreme learning machine (DELM) was proposed as a special deep learning, which is constructed by multiple ELM autoencoders.[1] Particle swarm optimization can quickly help to find the optimal  hidden layer nodes to improve the learning ability and efficiency. In this study, we used the Deep extreme learning machine based on the particle swarm optimization method(PSO-DELM) to predict the COVID-19. The number of hidden layers was set to three, the number sizes of nodes were 100 to 1000 in each layer, the transfer function was ‘sig’, and the iteration is 1000 to find the optimal hidden layer nodes. The optimal hidden nodes were set in the  following table:

|  | Transverse Plane | Coronal Plane | Sagittal Plane |
| --- | --- | --- | --- |
| Hidden nodes | [470 303 243] | [545 273 170] | [329 523 160] |

References:

[1]L.L.C. Kasun, H. Zhou, G.-B. Huang, and C. M. Vongc, “Representational learning with ELMs for big data,” IEEE Intell. Syst., vol. 28, no. 6, pp. 31–34, Dec. 2016.