Supplemental Information:

GHG Emissions Affected by Agricultural Drainage Ditch Dredging and Vegetation Brushing

Andrew Schietzsch1, Emilia Craiovan2, Mark Sunohara2, Oliver Blume3, Richard Amos3, Anne-Martine Doucet4, Ian Clark4, David R. Lapen2, David Blowes1, Carol Ptacek1*

1Dept. Earth and Environmental Sciences, University of Waterloo, 200 University Ave W, Waterloo, Canada, N2L 3G1

2Ottawa Research Development Centre, Agriculture and Agri-Food Canada, 960 Carling Ave., Ottawa, Canada, K1A 0C6

3Dept. Earth Sciences, Carleton University, 1125 Colonel By Dr., Ottawa, Canada, K1S 5B6

4Dept. Earth and Environmental Sciences, University of Ottawa, 75 Laurier Ave. E., Ottawa, Canada, K1N 6N5

*corresponding author

GHG data cleaning

GHG concentration time series data typically will form a linear trend. Occasionally one data point was quite divergent from the trend indicated by the other four data points in the time series. This could be due to a contaminated Exetainer vial that was not evacuated properly prior to sampling, poor fastening of a GHG chamber lid on the GHG collar, technician error when inserting the gas sample syringe into the Exetainer vial septa cap, or a leaking seal on the septa cap. Technician error can be significant due to the challenge of sampling with sharp syringe needles at specific time intervals while standing in a drainage ditch with sloped uneven and slippery terrain and flowing water. Another issue is that the first sample (0 min) can sometimes be off the trend of the other samples. The first sample is a measure of ambient GHG concentrations and sampled near the soil at the GHG collars before the GHG chamber lids were placed on top of the collars. The deviance from the other samples can be due to this sample being obtained in an ambient environment which may have effects from wind mixing or possibly the technician exhaling near the sample. To ensure slopes are real trends in GHG concentration, all time series trends were manually inspected with slope and R2 calculation. If four out of five data points showed a strong trend but one data point was subjectively off the trend line, that a single data point was removed to clean the dataset (Supplementary Figure S3).

Supplementary Figure S1 Photographs of ditch and bank brush cutting and ditch dredging.

Top row: a) Control Tree May 2019, b) Brush cutting trees (February 2018), c) Brush Cut Tree (May 2019), d) Brush Cut Tree (Aug 2019).

Bottom row: e) Control Shrub (May 2019), f) ditch dredging (November 2018), g) Brush Cut Shrub (May 2019), h) Brush Cut Shrub (August 2019).

Supplementary Figure S2 Overhead view of microplot site showing layout of instrumentation. Deployed in 2018 above 2019 and 2020 on bottom. Instrumentation type in legend. Ditch zone on right.

Supplementary Figure S3 Comparison of GHG concentration time series data before and after manual data cleaning. Data is for single GHG chamber location on southwest bank collected on July 9, 2019. Line is linear fit where slope is used in subsequent GHG chamber flux data. From top to bottom: N2O, CH4, CO2. From left to right: raw data, manually cleaned data.

Supplementary Figure S4 Temporal changes in Hyporheic zone soil chemical composition for 2019 at 0-15 cm depth for Brush Cut Shrub •, Brush Cut Tree ■, Control Shrub ▲, Control Tree ♦. From top to bottom: NH4-N, NO3-N, N, C, C:N ratio, inorganic carbon, organic carbon, plant available phosphorus.

Supplementary Figure S5 Temporal changes in Bank zone soil chemical composition for 2019 at 0-15 cm depth for Brush Cut Shrub •, Brush Cut Tree ■, Control Shrub ▲, Control Tree ♦. From top to bottom: NH4-N, NO3-N, N, C, C:N ratio, inorganic carbon, organic carbon, plant available phosphorus.

Supplementary Figure S6 Temporal changes in Shoulder zone soil chemical composition for 2019 at 0-15 cm depth for Brush Cut Shrub •, Brush Cut Tree ■, Control Shrub ▲, Control Tree ♦. From top to bottom: NH4-N, NO3-N, N, C, C:N ratio, inorganic carbon, organic carbon, plant available phosphorus.

Supplementary Figure S7 CO2 Flux •, Soil Moisture ■, and Soil T ♦ time series with calculated standard error bars from LI-COR measurements in agricultural drainage ditch Shoulder zone. From top to bottom 2018, 2019, and 2020 sampling seasons. From left to right Control Tree, Brush Cut Tree, Control Shrub, Brush Cut Shrub sites.

Supplementary Figure S8 Time Series of CO2, CH4, N2O Fluxes from May until November 2018 for Brush Cut Shrub •, Brush Cut Tree ■, Control Shrub ▲, Control Tree ♦. Only flux with R2>0.8 included. Note different scale for CH4 flux from Stream Bed. From top to bottom: CO2-C, CH4-C, N2O-N fluxes. From left to right Shoulder, Bank, Instream zones. 2018 data used different chamber layout than 2019 and 2020. Bank includes Bank and Hyporheic zones.

Supplementary Figure S9 Time Series of CO2, CH4, N2O Fluxes from July until November 2020 for Brush Cut Shrub •, Brush Cut Tree ■, Control Shrub ▲, Control Tree ♦. Only flux with R2 >0.8 included. From top to bottom: CO2-C, CH4-C, N2O-N fluxes. From left to right Shoulder, Bank, Hyporheic, Instream zones.

Supplementary Figure S10 LI-COR CO2 flux versus soil moisture content with calculated R2 correlation coefficients. Line is a linear regression to visually indicate correlation. Drainage ditch regions: top row Bank, bottom row Shoulder. Ditch sites from left to right: Brush Cut Shrub, Control Shrub, Brush Cut Tree, Control Tree.

Supplementary Figure S11 LI-COR CO2 flux versus soil T with calculated R2 correlation coefficients. Line is a linear regression to visually indicate correlation. Drainage ditch regions: top row Bank, bottom row Shoulder. Ditch sites from left to right: Brush Cut Shrub, Control Shrub, Brush Cut Tree, Control Tree.

Supplementary Figure S12 LI-COR CO2 flux with calculated standard error versus Vial method CO2 flux during 2019 sampling season for Brush Cut Shrub •, Brush Cut Tree ■, Control Shrub ▲, Control Tree ♦. Black line represents theoretical LI-COR=Vial method. Red line represents linear fit with no intercept given by equation above it.

Supplementary Table S1 Multiple Linear Regressions LI-COR Data, CO2 dependent variable, Soil Moisture and Soil T independent variables

|  |  | Soil M |  | Soil T |  |
| --- | --- | --- | --- | --- | --- |
| Region Name | N | P Value | Significant | P Value | Significant |
| All sites bank and shoulder | 243 | 0.825 | No | <0.001 | Yes |
| Brush Cut Shrub Bank | 33 | 0.060 | No | <0.001 | Yes |
| Brush Cut Shrub Shoulder | 33 | 0.466 | No | 0.062 | No |
| Brush Cut Tree Bank | 28 | 0.690 | No | <0.001 | Yes |
| Brush Cut Tree Shoulder | 28 | 0.731 | No | 0.029 | Yes |
| Control Shrub Bank | 32 | 0.238 | No | 0.016 | Yes |
| Control Shrub Shoulder | 32 | 0.545 | No | 0.137 | No |
| Control Tree Bank | 28 | 0.114 | No | <0.001 | Yes |
| Control Tree Shoulder | 28 | 0.581 | No | 0.745 | No |

Supplementary Table S2 Mean and standard error measurements over three years for parameters at different microplot sites

|  | Brush Cut Shrub (Site 1) |  | Brush Cut Tree (Site 5) |  | Control Shrub (Site 10) |  | Control Tree (Site 11) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Set | Mean | SEM | Mean | SEM | Mean | SEM | Mean | SEM |
| LI-COR Bank CO2 | 52 | 4 | 66 | 3 | 42 | 2 | 39 | 2 |
| Bank Soil M | 0.23 | 0.01 | 0.15 | 0.01 | 0.19 | 0.01 | 0.23 | 0.01 |
| Bank Soil T | 21 | 1 | 22 | 1 | 20 | 1 | 18 | 1 |
| Vial Shoulder CO2 | 47 | 3 | 45 | 4 | 52 | 3 | 33 | 3 |
| Vial Bank CO2 | 38 | 2 | 31 | 2 | 29 | 2 | 22 | 2 |
| Vial Hyporheic CO2 | 19 | 4 | 8 | 2 | 24 | 5 | 8 | 2 |
| Vial Instream CO2 | 26 | 3 | 41 | 5 | 20 | 2 | 37 | 6 |

Supplementary Table S3 One way ANOVA analysis over three years for parameters

| Data Set | N | Normality | P | Equal Variance | P | ANOVA significant (Y/N) | P | Holm Sidak Paired T-Test Results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LI-COR Bank CO2 | 41 | Failed | < 0.050 | Passed | 0.078 | Y | <0.001 | 5 significantly higher than 10, 11 |
| Bank Soil M | 41 | Passed | 0.093 | Passed | 0.656 | Y | <0.001 | 5 significantly lower than 1, 11. 10 significantly lower than 11 |
| Bank Soil T | 41 | Failed | <0.050 | Passed | 0.797 | Y | 0.005 | 11 significantly lower than 1, 5 |
| Vial Shoulder CO2 | 37 | Failed | <0.050 | Passed | 0.165 | Y | <0.001 | 11 significantly lower than 1, 10 |
| Vial Bank CO2 | 37 | Passed | 0.884 | Failed | <0.050 | Y | <0.001 | 11 significantly lower than 1, 5. 10 significantly lower than 1 |
| Vial Hyporheic CO2 | 22 | Passed | 0.577 | Failed | <0.050 | Y | 0.041 | 11 significantly lower than 10 |
| Vial Instream CO2 | 30 | Failed | <0.050 | Failed | <0.050 | Y | 0.002 | 10 significantly lower than 5, 11 |