Multiple potentially toxic elements in urban gardens from a Brazilian industrialized city

Supplementary Material

For results obtained in the soil samples, the risk assessment was divided into non-carcinogenic and carcinogenic risks, considering three exposure routes: ingestion and inhalation and dermal, calculating the chronic daily dose (CDD) in mg kg-1 day-1 (Eq. 2-4) according to the methodology proposed by Gabarrón et al. (2017) and Hu et al. (2017) and using the selected parameters (Table S1). The risk was assessed through the hazard quotient (HQ) that represents the non-carcinogenic risk of a given PTE (Eq.5). The HQ is defined as the ratio between CDD and the reference dose (RfD, mg kg-1 day-1) and is an estimate of the daily exposure to the human population that is not likely to present a considerable risk of deleterious effects during life (USEPA, 2009).

CDDing=Csoil×IngR ×EF ×EDBW ×AT ×10-6  		        (Eq.2)

CDDdermal=Csoil×SL ×SA ×ABS ×EF ×EDBW ×AT ×10-6		(Eq.3)

CDDinh=Csoil×InhR ×EF ×EDPEF ×BW ×AT ×10-6		        (Eq.4)

HQ=CDDRfD 					(Eq.5)

CR=CDD ×SF 	  		 	         (Eq.6)

The RfD values were consulted in the Risk Assessment Information System (RAIS, 2021). The combined risk called hazard index (HI) was calculated through the sum of the risk quotients (USEPA, 2009). If HI<1.0, health risks are not expected to occur for a given PTE; HI>1.0, there may be potential health effects and some monitoring evaluation will be needed and the probability tends to increase as the HI value increases (USEPA, 1989).

For carcinogens, risk was estimated as the incremental probability that an individual will develop cancer over a lifetime as a result of exposure to PTEs (USEPA, 2009). The carcinogenic risk (CR) was estimated according to the USEPA (2009) methodology (Eq.6). Here, the CR is the probability of carcinogenic risk (dimensionless), the TCR is the total probability of carcinogenic risk and the CSF is the carcinogenic slope factor of each element (1 mg-1 kg-1 day-1). The total carcinogenic risk is equal to the sum of the risk from all exposure pathways for all individual PTEs. The HR values for selected EPTs in different exposure pathways (RAIS, 2021). The acceptable range of total risk for regulatory purposes is 10-6 to 10-4 (USEPA, 2009). In regulatory terms, a TRC of 10-4 or greater indicates a potentially large risk.

Table S1 – Exposure parameters of human health risk assessment.

| Parameter | Definition | Adult | Reference |
| --- | --- | --- | --- |
| Csoil | Concentration of PTE in soil (mg kg-1) | The upper limit value of the arithmetic mean at the 95% confidence interval | (USEPA, 1992) |
| IngR | Ingestion rate (mg day-1) | 100 | (USEPA, 2002) |
| EF | Exposure frequence (days year-1) | 122 | (Gabarrón et al., 2017) |
| ED | Exposure duration (years) | 24 | (USEPA, 2002) |
| BW | Body weight (kg) | 70 | (USEPA, 1986) |
| AT | Average time | ED x 365 days for non-carcinogenic; 70 x 365 days for carcinogens | (USEPA, 1989) |
| IR | Air inhalation rate (m3) | 20 | Adult (USEPA, 1986); |
| PEF | Particulate emission factor (m3 kg-1) | 1.32 x 109 | (USEPA, 1996) |
| SL | Skin adherence factor (cm-2) | 5700 | (USEPA, 2004) |
| SA | Exposed skin area (mg cm-2) | 0.07 |  |
| ABS | Dermal absorption factor | 0.001 for all studied elements |  |

Table S2 - Select physico-chemical properties of the soils from urban gardens (0-20 cm).

| Urban Garden | Type of Sample | Total Sand | Coarse sand | Fine sand | Clay | Silt | Classification | pH | OM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (g kg-1) |  |  |  |  |  |  | (g dm-3) |
| Capuava 1 | Bed | 487 | 296 | 191 | 339 | 174 | Sandy clay loam | 6.4 | 53 |
|  | Local | 460 | 312 | 148 | 343 | 197 | Clay | 5.9 | 68 |
|  | Bed | 467 | 308 | 159 | 336 | 197 | Sandy clay loam | 5.3 | 63 |
| Capuava 2 | Bed | 434 | 259 | 175 | 370 | 196 | Sandy clay loam | 7.3 | 28 |
|  | Bed | 477 | 334 | 143 | 333 | 190 | Sandy clay loam | 5.7 | 59 |
|  | Local | 451 | 292 | 159 | 360 | 189 | Clay loam | 6.6 | 46 |
| B. Jardim | Bed | 352 | 215 | 137 | 489 | 159 | Clay | 6.6 | 31 |
|  | Local | 305 | 172 | 133 | 536 | 159 | Clay | 6 | 27 |
|  | Bed | 383 | 246 | 137 | 487 | 130 | Clay | 6.1 | 30 |
|  | Bed | 412 | 275 | 137 | 461 | 127 | Clay | 6.1 | 31 |
| V. Bastos | Bed | 646 | 371 | 275 | 227 | 127 | Sandy clay loam | 6.2 | 36 |
|  | Bed | 502 | 324 | 178 | 314 | 184 | Sandy clay loam | 6.2 | 35 |
|  | Local | 547 | 332 | 215 | 245 | 208 | Sandy clay loam | 7.6 | 21 |
|  | Bed | 548 | 328 | 220 | 321 | 131 | Sandy clay loam | 6.5 | 33 |
| Marajoara | Bed | 353 | 207 | 146 | 423 | 224 | Clay | 7.1 | 46 |
|  | Local | 320 | 144 | 176 | 547 | 133 | Clay | 7 | 29 |
|  | Bed | 355 | 197 | 158 | 408 | 237 | Clay | 7 | 54 |
|  | Bed | 457 | 318 | 139 | 321 | 222 | Sandy loam | 7 | 57 |
| Bela Vista | Bed | 462 | 194 | 268 | 248 | 290 | Sandy clay loam | 6.3 | 41 |
|  | Local | 390 | 121 | 269 | 325 | 285 | Clay loam | 6.4 | 38 |
|  | Bed | 365 | 177 | 218 | 311 | 294 | Clay loam | 6.3 | 49 |
|  | Bed | 426 | 169 | 257 | 224 | 350 | Loam | 6.6 | 35 |
| Urban Garden | Type of Sample | SB | K-Exc | Ca-Exc | Mg-Exc | H+Al | CEC | V (%) | P- Exc |
|  |  | (mmolc dm-3) |  |  |  |  |  |  | (mg dm-3) |
| Capuava 1 | Bed | 156.1 | 4.1 | 125 | 27 | 21 | 177 | 88 | 284 |
|  | Local | 151.9 | 6.9 | 118 | 27 | 31 | 183 | 83 | 159 |
|  | Bed | 146.1 | 3.1 | 123 | 20 | 47 | 193 | 76 | 264 |
| Capuava 2 | Bed | 110.5 | 4.5 | 86 | 20 | 10 | 121 | 91 | 365 |
|  | Bed | 130.6 | 3.6 | 113 | 14 | 31 | 162 | 81 | 382 |
|  | Local | 141.6 | 4.6 | 121 | 16 | 14 | 159 | 91 | 192 |
| B. Jardim | Bed | 104.8 | 4.8 | 88 | 12 | 14 | 119 | 88 | 173 |
|  | Local | 131.4 | 2.1 | 52 | 10 | 16 | 147 | 89 | 18 |
|  | Bed | 83.7 | 3.7 | 69 | 11 | 19 | 103 | 81 | 179 |
|  | Bed | 98.4 | 4.4 | 80 | 14 | 17 | 115 | 86 | 173 |
| V. Bastos | Bed | 105.5 | 4.5 | 87 | 14 | 16 | 122 | 86 | 225 |
|  | Bed | 105.8 | 5.8 | 85 | 15 | 16 | 122 | 87 | 349 |
|  | Local | 181.9 | 0.9 | 172 | 9 | <9 | 190 | 96 | 14 |
|  | Bed | 120.4 | 6.4 | 97 | 17 | 16 | 136 | 89 | 453 |
| Marajoara | Bed | 162.1 | 8.1 | 126 | 28 | 11 | 173 | 94 | 474 |
|  | Local | 106.7 | 5.7 | 86 | 15 | 11 | 118 | 90 | 36 |
|  | Bed | 204.3 | 12.3 | 158 | 34 | 12 | 216 | 95 | 562 |
|  | Bed | 185.3 | 11.3 | 143 | 31 | 12 | 197 | 94 | 547 |
| Bela Vista | Bed | 131.4 | 12.4 | 89 | 30 | 16 | 147 | 89 | 555 |
|  | Local | 92 | 5 | 70 | 17 | 15 | 107 | 86 | 377 |
|  | Bed | 150.6 | 6.6 | 114 | 30 | 17 | 168 | 90 | 453 |
|  | Bed | 159 | 11 | 111 | 34 | 12 | 171 | 93 | 302 |

OM: organic matter; SB: sum of bases. equal to exchangeable Ca. Mg and K; Exc: exchangeable; CEC: cation exchange capacity (millimol of charge per dm3). equal to SB + H+Al; V: base saturation. equal to V(%) = (100 x SB)/CEC.

Table S3 - Results of Pearson's correlation between soil characteristics of urban gardens in Santo André (0-20 cm) and PTEs (p<0.05).

|  | As | Ba | Cd | Co | Cr | Cu | Mo | Ni | Pb | Se | Sb | V | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sand | -0.55 | 0.25 | 0.56 | 0.14 | -0.38 | 0.24 | 0.06 | 0.07 | 0.50 | 0.18 | 0.13 | -0.39 | 0.47 |
| Clay | 0.66 | -0.20 | -0.36 | -0.13 | 0.80 | -0.15 | 0.09 | 0.07 | -0.29 | -0.10 | -0.07 | 0.80 | -0.30 |
| Silt | -0.26 | -0.02 | -0.17 | 0.02 | -0.68 | -0.08 | -0.20 | -0.19 | -0.21 | -0.09 | -0.05 | -0.67 | -0.16 |
| pH | 0.32 | -0.41 | -0.10 | -0.31 | 0.12 | -0.45 | -0.39 | -0.24 | -0.46 | -0.36 | -0.30 | 0.28 | -0.41 |
| MO | -0.57 | 0.65 | 0.23 | 0.75 | -0.11 | 0.69 | 0.63 | 0.67 | 0.43 | 0.73 | 0.66 | -0.42 | 0.66 |
| P-tro | -0.16 | -0.11 | -0.38 | -0.12 | -0.45 | -0.09 | -0.14 | -0.22 | -0.39 | -0.11 | -0.22 | -0.37 | -0.15 |
| K-tro | 0.04 | -0.18 | -0.43 | -0.15 | -0.30 | -0.19 | -0.18 | -0.17 | -0.47 | -0.17 | -0.21 | -0.27 | -0.23 |
| Ca-tro | -0.32 | 0.37 | 0.54 | 0.53 | 0.00 | 0.41 | 0.35 | 0.52 | 0.32 | 0.43 | 0.45 | -0.16 | 0.40 |
| Mg-tro | -0.21 | 0.24 | -0.21 | 0.21 | -0.36 | 0.09 | 0.10 | 0.11 | -0.19 | 0.22 | 0.11 | -0.46 | 0.11 |
| H+Al | -0.24 | 0.34 | 0.85 | 0.38 | 0.05 | 0.41 | 0.29 | 0.39 | 0.64 | 0.28 | 0.44 | -0.06 | 0.36 |
| SB | -0.21 | 0.32 | 0.34 | 0.45 | 0.02 | 0.31 | 0.29 | 0.45 | 0.17 | 0.34 | 0.37 | -0.13 | 0.32 |
| CEC | -0.34 | 0.50 | 0.45 | 0.63 | 0.02 | 0.53 | 0.49 | 0.61 | 0.36 | 0.52 | 0.56 | -0.21 | 0.50 |
| V% | 0.25 | -0.44 | -0.19 | -0.36 | -0.01 | -0.53 | -0.49 | -0.31 | -0.53 | -0.41 | -0.41 | 0.16 | -0.43 |

REFERENCES

Gabarrón, M., Faz, A., & Acosta, J. A. (2017). Soil or dust for health risk assessment studies in urban environment. Archives of environmental contamination and toxicology, 73(3), 442-455. https://doi.org/10.1007/s00244-017-0413-x

RAIS - The Risk Assessment Information System (2022). Disponível em: https://rais.ornl.gov/tools/tox_profiles.html.  Accessed 09 June 2022

USEPA (1986). Superfund public health evaluation manual. Office of Emergency and Remedial Response. U.S. Environmental Protection Agency Washington, 20460. EPA/540/1-86/060.

USEPA (1989). Risk assessment guidance for superfund volume I human health evaluation manual (Part A). Office of Emergency and Remedial Response. U.S. Environmental Protection Agency Washington, 20450. EPA/540/1-89/002.

USEPA (1992). Guidelines for exposure assessment. Risk Assessment Forum. U.S. Environmental Protection Agency Washington, EPA/600/Z-92/001.

USEPA (1996). Soil screening guidance: Technical Background Document. Office of Solid Waste and Emergency Response. Washington, 20460. EPA/540/R95/128.

USEPA (2002). Calculating upper confidence limits for exposure point concentrations at hazardous waste sites. Office of Emergency and Remedial Response U.S. Environmental Protection Agency Washington, 20460. OSWER 9285.6-10.

USEPA (2004). Risk assessment guidance for superfund volume I: human health evaluation manual (Part E, Supplemental Guidance for Dermal Risk Assessment). Office of Superfund Remediation and Technology Innovation U.S. Environmental Protection Agency Washington, EPA/540/R/99/005.

USEPA (2005). Guidelines for Carcinogen Risk Assessment. Forum U.S. Environmental Protection Agency Washington, DC U.S. Environmental Protection Agency Washington.

USEPA (2009). Integrated science assessment for particulate matter. National Center for Environmental Assessment-RTP Division. Office of Research and Development. U.S. Environmental Protection Agency. Research Triangle Park, EPA/600/R-08/139F.