On the stability & phase locking to a system reference of an optoelectronic oscillator with large delay

Mehedi Hasan1*, Charles Nicholls2, Trevor Hall1

1 Photonic Technology LAB, University of Ottawa, 25 Templeton Street, Ottawa, ON, Canada, K1N 6X1

2 NANOWAVE Technologies Inc., 6 Gurdwara Rd, Nepean, ON, Canada, K2E 8A3

*To whom correspondence should be addressed; E-mail: mhasa067@uottawa.ca

Supplementary material

Baseband equivalent transfer function of a bandpass filter

Let the bandpass filter be described by the convolution:

y=f⊗x							(A1)

where f is the real impulse response of the filter;

xt= utexpiω0t						(A2)

yt= vtexpiω0t						(A3)

u, v are the respective complex envelopes of x, y and ω0 is the nominal carrier frequency:

Introducing the Fourier transform pairs:

x,y,f,u,v⟷X,Y,F,U,V					(A4)

it follows that:

Yω=FωXω						(A5)

Xω0+ω=Uω						(A6)

Yω0+ω=Vω						(A7)

Hence:

Vω=Yω0+ω=Fω0+ωXω0+ω=Hω Uω		(A8)

where:

Hω= Fω0+ω						(A9)

is the baseband equivalent transfer function of the bandpass filter.

The transfer function of a two-pole bandpass filter is described by:

Fω=11+iQ1+ω-ω0ω0-11+ω-ω0ω0					(A10)

where Q=ω0Δω is the quality factor, ω0 is the passband centre frequency, and Δω is the -3dB bandwidth.

Consequently, the baseband equivalent transfer function is given by:

Hω=11+iQ1+ωω0-11+ωω0						(A11)

Noting the offset frequencies ω of interest are small ωω0~10-6-10-4 one may apply the binomial expansion to first order:

11+ωω0=1-ωω0+…						(A12)

to yield a highly accurate single pole baseband equivalent transfer function:

Hω≈11+iωτR;τR=2Qω0=1Δω2				(A13)

Discussion on optoelectronic oscillator phase noise performance and prospects for integration for an optical delay (τD) comparable to the bandpass filter delay (τR)

In the case of an intra-loop phase fluctuation spectral density independent of the fibre length, the close to carrier (ωτD<1) phase noise spectral density is proportional to 1ωτD2. The theoretical description remains valid as τD→τR but the character of the oscillator progressively evolves from a multimode time delay oscillator with properties dominated by the delay line to a classical single mode oscillator with properties dominated by the RF bandpass filter (resonator). In terms of the phase noise spectral density, the close-in phase noise of the time delay oscillator is low ~1ωτD2 but features spurious sidemode resonances whereas the close-in phase noise of the classical oscillator is high ~1ωτR2 but is free spurious sidemode resonances.

In the authors’ experiments, the on-resonance group delay of the RF-bandpass filter is τR~100 ns which equates to a waveguide delay line ~20 m in length. Implementation as an integrated waveguide (or as a compact ring resonator with the same on-resonance group delay) would require waveguide losses lower than ~0.20 dB m-1. Currently commercially accessible low-loss photonic integration platforms feature waveguide losses two orders of magnitude higher ~0.20 dB cm-1. Losses of ~0.1 dB m-1 have been demonstrated in the laboratory using specialised processes. The demonstration of fully integrated OEOs is an important milestone and there are projections that suggest that processes featuring waveguide losses of ~0.01 dB m-1 are feasible. However, the integrated OEOs demonstrated to date fail to leverage the delay and their phase noise performance is poor.

Optoelectronic oscillator under proportional control: location of system poles

The poles are located at the roots of the characteristic function (Eq. (17) main manuscript)

1+Fs-11+τRsexp-τDs=0				(A14)

Simple proportional control corresponds to setting F=κ. Following the method for the free oscillator described in the main text, the Lambert equation for the roots is then:

ρ+τDsexpρ+τDs=ρexpρ1+κ			(A15)

which implies:

skτD= wkρexpρ-log1+κ- ρ				(A16)

Eq. (A16) demonstrates that for ρ≫log1+κ the proportional control displaces the poles further into the left half-plane substantially by the same amount ~-κ for small proportional gain κ≪1 (see Fig. A1).

Fig. 1A. The predicted location of the poles of the system function of a proportionally controlled time delay oscillator in the phased locked loop configuration (τDτR=300; κ=0.05). All poles have been displaced further along the negative real axis substantially by the proportional gain.

Controlled optoelectronic oscillator experiment: Bode plots & outer pole locations

The simulated open loop bode plot for the two different sets of loop filter is presented in the main manuscript. The corresponding system pole locations of the phase locked oscillator is presented herein.

Fig. 2A. Phase locked oscillator outer system pole locations; (a) Case-1; modulational instability arises due to the small positive real part of the poles associated to the first adjacent sidemodes; (b) Case-2; all pole locations have a negative real part albeit slight. Modulational instability is suppressed.

Equations used to calculate the values listed in Table 1(b):

κ=τ2τ11NKVCOτDICPR19		(A17)

τ1=R19C25+C28		(A18)

τ2=R21C25			(A19)

τ3=R21C25C28C25+C28		(A20)

τ4=R19C17			(A21)

τ5=R14C5			(A22)