Supplementary Information

Novel green fluorescent probe stem from carbon quantum dots for specific recognition of tyrosinase in serum and living cells

Wenxian Wang#, Dan Yang#, Yufei, Zhou, Yuding Zhang, Lijiao Guan, Xianfen Zhang, Weiming Xue*, Saipeng Huang*

# These authors contributed equally to this work and should be considered co-first authors.

*Corresponding authors.

School of Chemical Engineering, Northwest University, Xi’an, PR China

1. Instrumentation and characterization methods

Transmission electron microscopy (TEM) was performed by a copper grid coated with thin films of carbon on a Tecnai G2 F20 microscope (FEI Co. U.S.A) at an accelerating voltage of 200 kV. Fourier-transform infrared spectra (FT-IR) were obtained for KBr pellets on Frontier-PerkinElmer FT-IR spectrometer from 4000 to 400 cm−1 of wavenumber. UV−vis absorption spectra were obtained on a UV-3600 UV-VIS-NIR spectrophotometer (Shimadzu, Japan) by a quartz cuvette with 1 cm path length.

Fluorescence excitation and emission spectra were recorded on a Hitachi F-4600 fluorescence spectrophotometer with a scan rate of 240 nm·min-1, which utilized a Xenon lamp of 1/5 nm slit width and equipped by a quartz cuvette with an optical path length of 1 mm, the excitation (Ex) /emission wavelengths (Em) were 465 nm and 550 nm, respectively. Fluorescence imaging experiments were performed on an FV 1000-IX81confocal laser scanning microscope (Olympus, Japan) with FV5-LAMAR for excitation at 465 nm and a variable bandpass emission filter set to 500-550 nm through a 100 × 1.4 NA objective. Fluorescent photographs were taken by a SONY DSC-W800 with ZF-20D hand-held UV lamp (Shanghai, China) under excitation at 465 nm.

X-ray photoelectron spectroscopy (XPS) survey data were obtained by a Thermo Scientific ESCALAB 250Xi equipped with an Al Kα X-ray source (1487.2 eV), the analysis chamber was 8×10−8 mbar and the X-ray spot was 650 μm. Zeta potential of the SeO3-CQDs aqueous solution was measured by a Zetasizer Nano ZS series (Malvern Instruments, UK) with a laser wavelength of 633 nm and a detection angle of 173° at 25 °C.

2. Experimental

2.1 Quantum yield calculations of the TYR-CQDs

The quantum yield (QY) of the TYR-CQDs was calculated by a reported formula utilized quinine sulfate in 0.1 mol/L H2SO4 as the reference sample. The QY of quinine sulfate was 54% at 365 nm excitation, while the QY of TYR-CQDs was calculated according to the following equation:

where ϕ represents the QY of testing sample, F represents the fluorescence intensity of testing sample, A stands for the absorbance, n stands for the refractive index. The subscript “1” and “2” refer to the TYR-CQDs and quinine sulfate, respectively. In an identical solvent, n2/n1 = 1.

2.2 Stability of TYR-CQDs

The photostability and chemical stability of TYR-CQDs were mainly investigated under different conditions, exemplified by photobleaching, inorganic salt ions and different acid-base environments. Firstly, the photostability was investigated by continuously stimulating TYR-CQDs aqueous solution (70 μg/mL) under a 150 W xenon lamp for 60 min, and the fluorescence spectra (Ex=465 nm, Em=550nm) were measured at different time. Meanwhile, the resisted inorganic salt ions ability and pH stability were evaluated by the fluorescence changes of TYR-CQDs (70 μg/mL) incubated with a series concentration of NaCl solution (10-6, 10-5, 10-4, 10-3, 10-2, 10-1 and 1.0 mol/L) or a series buffer solutions with pH 4.0, 5.0, 6.0, 7.0, 8.0, 9.0 and 10.0.

2.3 Selectivity of TYR-CQDs to TYR

Based on the excellent optical properties of TYR-CQDs, the selectivity and anti-interference ability were investigated by the fluorescence fluctuation of TYR-CQDs (70 μg/mL) incubated with different amino acids (1 mmol/L His, Ser, Ala, Thr, Tyr, Cys, Cys-Cys), various proteins (3 mg/mL SPI, PPI, OVA, BSA) and enzymes (100 KU/L GOX, LYS, ALP). The fluorescence intensity of TYR-CQDs in the absence (F0) or presence (F) of interfering components was recorded accordingly.

2.4 Detection of TYR inhuman serum samples

Human serum samples containing four different concentrations of TYR were added to the TYR-CQDs solution, the final concentration of TYR-CQDs was 70 μg/mL, and the standard concentration of TYR was 50, 200, 600, and 1000 U/L, respectively. The mixtures were incubated at 37 °C for 1 h, and the fluorescence intensity were recorded at an excitation wavelength of 465 nm. According to the linear fitting equation of TYR, the measured concentration and subsequent recovery rate of three samples could be calculated. Each experiment was performed three times.

2.5 Cytotoxicity measurements

In order to evaluate cytotoxicity of TYR-CQDs and intracellular TYR analysis, Hela cells were cultured in DMEM with 10% fetal bovine serum and seeded in 96-well plates at 37℃ for 24 h in a humidification incubator containing 5% CO2. Different concentrations of TYR-CQDs (0，0.5 , 1.0 , 5.0 , 10.0 , 25.0, 50.0 and 100.0 μg/mL ) with 10% fetal bovine serum were added and cultured for 24 h. After washing three times with PBS and adding MTT solution for 4 h at 37 ◦C, the medium was carefully removed and 150 μL DMSO was added to dissolve the purple formazan crystals. The absorbance optical density at 490 nm was measured to calculate the cytotoxicity of TYR-CQDs. Each experiment was performed at least 3 times.

3. Figure

Fig. S1 Zeta potential data of TYR-CQDs

Fig. S2 The fluorescence intensity variation of TYR-CQDs under continuous irradiation with xenon lamp for 60 min

Fig. S3 The fluorescence intensity variation of TYR-CQDs under different concentrations of NaCl

Fig. S4 The fluorescence intensity variation of TYR-CQDs under different pH

Fig. S5 The mechanism research of specific reaction between TYR-CQDs and TYR. The infrared spectra of p-aminophenol (PAP), carbon quantum dots prepared by p-aminophenol (PAP-CQDs), PAP-CQDs + TYR (PAP-CQDs cultured with TYR), TYR-CQDs + TYR (TYR-CQDs cultured with TYR)