### 1. Materials and Methods

1.1. Satellite data

The high-resolution spatial-temporal analysis in this study was based on examining QuickBird multispectral satellite images (ground pixel resolution of 2.4 m) and three bands (blue, green, red), obtained in Nov/2004, Oct/2007, Nov/2012, and Jan/2015. Images were processed in the Global Mapper version 22.1.2. Each satellite was orthorectified according to ground control points (GCPs) identifiable in satellite images. The planimetric analysis was finalized with drone images (2.6 cm/pixel) obtained in Oct/2017, Nov/2019, Nov/2021 and Jul/2022 during the low tide.

1.2. Lidar data

The LIDAR data were obtained in 2002 (NOAA/NASA/USGS) and 2013 (USGS), with a vertical and horizontal accuracy of 10-15 cm and 73-100 cm, respectively, downloaded from the Atlas website at Louisiana State University (https://atlas.ga.lsu.edu/) and the National Oceanic and Atmospheric Administration website (https://coast.noaa.gov/dataviewer/#/). These planialtimetric data were used to evaluate the beach barrier and wetlands dynamics.

1.3. Drone images

A Drone DJI Phantom 4 Advanced (Oct/2017, Nov/2019 and Nov/2021)  and a Phantom 4 Real-Time Kinematic (Jul/2022) with a digital 4K/20MP (RGB) camera were used to obtain high spatial resolution (2.6 cm) images. A D–RTK 2 Mobile Station and differential corrections transmitted in real-time from a network allowed georeferencing each obtained photo by the Phantom 4 RTK drone in Jul/2022. The missions were carried out autonomously (90º camera angle, 90% frontal, and 75% lateral overlay, at 100 m altitude).  A total of ~13000 images were recorded to cover ~940 ha in Oct/2017, Nov/2019, Nov/2021 and Jul/2022. A dense point cloud, processed using the Agisoft Metashape Professional version 1.6.3, was generated to obtain digital models of the terrain- and vegetation-based photogrammetric analysis of drone images. The difference in spectral characteristics and elevation of point clouds allowed us to identify points indicating the vegetation, muddy flats and the sandy barrier. More details about this processing can be obtained in Cohen et al. (2021a and 2021b).

1.4. Ground control points

The planialtimetric positions of the Ground Control Points (GCP, 89 points), identified on the ground as black rubber mats (1 m2) with yellow cross, were obtained by an Antenna Trimble Catalyst with a differential Global Navigation Satellite System (GNSS) (accuracy ± 1 cm with the Real-Time Kinematic correction) and an electronic Self Leveling Horizontal Rotary Laser during field trips in Oct/2017, Nov/2019, Nov/2021 and Jul/2022 (Table 1, supplementary material).

1.5. Image classification

Mangrove, saltmarsh, beach barrier, water and anthropogenic interventions were manually individualized and classified by photointerpretation using the Global Mapper 22.1.2 software. Multispectral digital numbers, physical, and geometric characteristics were identified for each class. Panoramic aerial photos and fieldwork with 89 GCPs were also employed to support this classification. This cross-validation data allow us to identify and establish limits for those classes with high precision. Erosion zones were automatically recorded by the comparative analysis between the digitalized limits of the mangrove, saltmarsh, and beach barrier classes identified in Nov/2019, Nov/2021 and Jul/2022.

1.6. 3D models

The drone data were processed through the Agisoft Metashape Professional 1.6.2. to produce a 3D spatial model and orthomosaics. Orthomosaic images of Oct/2017, Nov/2019,  Nov/2021 and Jul/2022 were utilized for the spatial-temporal analysis. A high-resolution, dense point cloud was executed according to point clouds with point spacing between 4 and 5 cm. The dense point clouds were analyzed to identify points representing the terrain elevation to get a Digital Terrain Model (DTM). The DTM corresponds to the ground devoid of vegetation. Another procedure produced a digital surface model (DSM) indicating the vegetation cover, ground, houses and streets (Agisoft, 2018). The DTM for the mangrove and marsh substrate was based on drone survey from 2018 and previously processed and published by Cohen et al. (2021) (Fig. 3d). The DTMs from the beach barrier were obtained in Nov/2019 (Cohen et al., 2021), Nov/2021 and Jul/2022 (Fig. 4). The Avicennia tree density was previously obtained by Cohen et al. (2021) and processed by the Global Mapper based on drone images (2018) and compared with fieldwork data (10 ×10 m in 36 visited sites) (Fig. 3c).

The elevation grid for the DTM was based on the mean dense point cloud to minimize the effects of ripples of the beach barrier on the drone surveys. Elevations were referenced to the Geoid Model EGM2008-5. The quantitative analysis of these models was based on the vertical divergences between GCP and the DTM, according to equation 1 (Cohen et al., 2018):

(2)	Zdif = ZDEM - Zgrd

where Zdif = the vertical differences, ZDEM = the Z value of the 3D dense point cloud, and Zgrd = the Z value of the GCPs. The Zdif values were <10 cm, suggesting a vertical error of ± 10 cm for the 3D models (Table 1, supplementary material).

The higher tidal level was used to reference the shoreline position, while the dune crest was defined as the maximum surface elevation of the beach barrier, as shown in the cross-shore profiles (Fig. 4). The mean sea level (0 m) was used as a baseline for sediment volume calculations. Such a procedure was developed in the Global Mapper version 22.1.2 software to calculate cut-and-fill volumes within a selected area. The volume was calculated by breaking the area and assessed into many small squares, and by using the formula: Volume = Height * Pixel Size. Fences along the beach barrier were identified in the drone images as a common reference to support the assessment of beach barrier retreat between 2019 and 2021 (Fig. 4b). In addition, Global Mapper allows for generating cross-shore profiles in a specified path based on loaded planialtimetric datasets (“Global Mapper User’s Manual,” 2020). A spatial and temporal sequence of these planialtimetric profiles allows us to identify changes in the beach barrier morphology (Fig. 4)

1.7. Spatial-temporal analysis

Planialtimetric positions of infrastructure (e.g. houses, boat ramps, poles) and GCP were used as references for time series analysis. The higher tidal level in the intertidal zone was used to assess the shoreline dynamics. Dimensions were calculated by Global Mapper version 22.1.2 software on drone images. Considering the maximum planimetric differences between the 3D model and the GCP data of 89 cm (X and Y, table 1, supplementary material), an error margin of 90 cm was admitted for the planimetric data obtained from these images.

Table 1 - Ground Control Points of the study area with longitude, latitude, elevation and differences (m) between latitudes longitudes and elevation obtained by photogrammetry and those obtained in the field by a topographic survey.

| GCP | X/Longitude | Y/Latitude | Z/Elevation | Xdif | Ydif | Zdif |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | -90.169318 | 29.117012 | 0.78 | -0.014713 | 0.000967 | 0.04 |
| 2 | -90.166061 | 29.119106 | 0.72 | 0.004428 | 0.004298 | 0.01 |
| 3 | -90.167760 | 29.117485 | 0.45 | -0.003994 | 0.000603 | 0.04 |
| 4 | -90.154767 | 29.127904 | 0 | 0.066736 | 0.03 | 0.08 |
| 5 | -90.154317 | 29.127531 | 0.45 | 0.088765 | 0.02 | 0.10 |
| 6 | -90.153269 | 29.126278 | 0 | 0.088842 | 0.003677 | 0.038 |
| 7 | -90.157144 | 29.124695 | 1.13 | 0.065826 | 0.009959 | 0.03 |
| 8 | -90.166814 | 29.119035 | 0.6 | 0.034 | 0.303264 | 0.09 |
| 9 | -90.164600 | 29.120375 | 1.18 | 0.018295 | 0.006667 | 0.03 |
| 10 | -90.161136 | 29.122167 | 1.03 | 0.039914 | 0.006835 | 0.03 |
| 11 | -90.168978 | 29.118352 | 0.27 | -0.008730 | 0.008301 | 0.03 |
| 12 | -90.168500 | 29.118745 | 0.21 | -0.005413 | 0.009216 | 0.03 |
| 13 | -90.168125 | 29.117093 | 0.20 | -0.008472 | -0.000703 | 0.02 |
| 14 | -90.170295 | 29.117766 | 0.3 | 0.014279 | -0.005867 | -0.01 |
| 15 | -90.171358 | 29.115810 | 1.0 | 0.002286 | 0.003633 | -0.02 |
| 16 | -90.172034 | 29.116723 | 0.61 | 0.003032 | 0.004055 | 0.003 |
| 17 | -90.170875 | 29.115429 | 0.26 | -0.000194 | 0.005523 | -0.001 |
| 18 | -90.173615 | 29.114195 | 0.75 | 0.002720 | 0.011527 | -0.04 |
| 19 | -90.176311 | 29.112635 | 0.78 | 0.006339 | 0.016190 | 0.003 |
| 20 | -90.173905 | 29.115531 | 0.87 | -0.022920 | -0.040214 | -0.002 |
| 21 | -90.178845 | 29.110735 | 1.15 | 0.042 | 0.046 | -0.02 |
| 22 | -90.181842 | 29.108411 | 0.78 | -0.636392 | 0.054 | -0.04 |
| 23 | -90.181485 | 29.108824 | 0.54 | 0.19 | 0.053 | 0.02 |
| 24 | -90.169279 | 29.117025 | 0.78 | 0.008734 | -0.000714 | 0.03 |
| 25 | -90.176196 | 29.114711 | 0 | -0.000529 | -0.003827 | 0.05 |
| 26 | -90.181666 | 29.109986 | 0 | 0.001820 | -0.002098 | 0.08 |
| 27 | -90.182765 | 29.111802 | 0 | -0.006467 | -0.008261 | 0.07 |
| 28 | -90.182708 | 29.111594 | 0 | 0.005128 | 0.006270 | 0.03 |
| 29 | -90.180144 | 29.111834 | 0.78 | -0.130523 | -0.783544 | 0.05 |
| 30 | -90.181499 | 29.109870 | 0.58 | 0.769515 | -0.681291 | 0.08 |
| 31 | -90.19069677 | 29.11536351 | 0.80 | -0.023 | 0.001 | 0.08 |
| 32 | -90.19016652 | 29.11431639 | 0.82 | 0.025 | 0.002 | 0.08 |
| 33 | -90.27262901 | 29.40220097 | 0.08 | -0.01 | 0.001 | 0.07 |
| 34 | -90.17542554 | 29.15725036 | 2.09 | 0.037 | 0.02 | 0.09 |
| 35 | -90.18374607 | 29.11241007 | 0.20 | 0.023 | 0.010 | 0.1 |
| 36 | -90.18330629 | 29.11229771 | 0.31 | 0.044 | 0.013 | 0.06 |
| 37 | -90.18279381 | 29.11217180 | 0.31 | 0.049 | 0.012 | 0.06 |
| 38 | -90.18282984 | 29.11196535 | 0.46 | 0.037 | 0.323 | 0.04 |
| 39 | -90.1834419 | 29.11183377 | 0.30 | 0.032 | 0.056 | 0.1 |
| 40 | -90.18342126 | 29.11122964 | 0.42 | 0.025 | 0.043 | -0.02 |
| 41 | -90.18442454 | 29.11149069 | 0.41 | -0.045 | 0.018 | -0.04 |
| 42 | -90.18511817 | 29.11275833 | 0.22 | -0.008 | 0.012 | 0.08 |
| 43 | -90.18360577 | 29.11592754 | 0.19 | -0.008 | -0.021 | 0.08 |
| 44 | -90.18249316 | 29.11581338 | 0.19 | 0.014 | -0.015 | -0.01 |
| 45 | -90.18229348 | 29.11597191 | 0.20 | 0.025 | 0.033 | 0.05 |
| 46 | -90.18256824 | 29.11750025 | 0.19 | 0.027 | 0.006 | 0.04 |
| 47 | -90.18092084 | 29.11767480 | 0.40 | -0.020 | 0.004 | 0 |
| 48 | -90.18065388 | 29.11726114 | 0.30 | 0.013 | 0.012 | 0.1 |
| 49 | -90.18029993 | 29.11778024 | 0.38 | 0.012 | 0.022 | 0.02 |
| 50 | -90.18157424 | 29.11834611 | 0.20 | -0.029 | -0.021 | 0.10 |
| 51 | -90.17979119 | 29.11950689 | 0.19 | 0.043 | 0.033 | 0.06 |
| 52 | -90.17871277 | 29.11907442 | 0.26 | -0.032 | 0.042 | 0.10 |
| 53 | -90.18069169 | 29.12028496 | 0.13 | 0.034 | 0.074 | -0.03 |
| 54 | -90.18066797 | 29.12046102 | 1.11 | 0.012 | -0.054 | 0.08 |
| 55 | -90.18044134 | 29.12074042 | 0.30 | -0.004 | -0.021 | 0.03 |
| 56 | -90.1779868 | 29.12066791 | 0.30 | 0.043 | -0.031 | 0.10 |
| 57 | -90.17629609 | 29.1216794 | 0.30 | -0.026 | -0.034 | 0.04 |
| 58 | -90.17247803 | 29.12269903 | 0.44 | 0.015 | 0.012 | -0.07 |
| 59 | -90.17396613 | 29.12483293 | 0.22 | -0.030 | -0.041 | 0.07 |
| 60 | -90.17380542 | 29.12708151 | 0.70 | 0.069 | -0.036 | 0.10 |
| 61 | -90.17325054 | 29.12730165 | 0.35 | 0.012 | 0.032 | 0.03 |
| 62 | -90.1752577 | 29.13295352 | 0.06 | 0.024 | 0.012 | 0.02 |
| 63 | -90.17067997 | 29.13297452 | 0.05 | -0.032 | -0.021 | 0.05 |
| 64 | -90.17382294 | 29.13579115 | 0.10 | 0.033 | 0.014 | -0.05 |
| 65 | -90.17399032 | 29.13560896 | 0.03 | 0.017 | 0.023 | 0.09 |
| 66 | -90.17123578 | 29.13816305 | 0.12 | -0.032 | -0.054 | -0.02 |
| 67 | -90.17055395 | 29.12313720 | 0.30 | 0.075 | 0.013 | -0.01 |
| 68 | -90.16858257 | 29.12393073 | 0.8 | -0.047 | 0.042 | 0 |
| 69 | -90.16787752 | 29.12416421 | 0.69 | 0.043 | 0.046 | -0.1 |
| 70 | -90.16700951 | 29.12455065 | 0.39 | 0.021 | 0.026 | 0.06 |
| 71 | -90.16418490 | 29.12573747 | 0.45 | -0.023 | -0.021 | 0.01 |
| 72 | -90.16211471 | 29.12696577 | 0.38 | 0.023 | 0.031 | -0.07 |
| 73 | -90.1625257 | 29.12575242 | 0.42 | -0.014 | -0.044 | 0.08 |
| 74 | -90.16321299 | 29.12530798 | 1.29 | 0.019 | 0.022 | 0.05 |
| 75 | -90.16347944 | 29.12516513 | 0.46 | -0.068 | -0.065 | 0.01 |
| 76 | -90.16686088 | 29.1238981 | 0.42 | 0.034 | 0.034 | 0.08 |
| 77 | -90.16940172 | 29.12113039 | 0.21 | 0.012 | -0.042 | -0.01 |
| 78 | -90.16904086 | 29.12049661 | 0.45 | 0.034 | 0.032 | -0.05 |
| 79 | -90.17279388 | 29.12001534 | 0.26 | 0.021 | 0.026 | 0.01 |
| 80 | -90.17526493 | 29.12021927 | 0.30 | -0.089 | -0.025 | 0.1 |
| 81 | -90.18179969 | 29.11550258 | 0.22 | 0.021 | 0.014 | -0.09 |
| 82 | -90.18260242 | 29.11311032 | 0.3 | 0.017 | 0.028 | 0 |
| 83 | -90.18150111 | 29.11200832 | 1.49 | 0.023 | 0.031 | 0.02 |
| 84 | -90.18100701 | 29.11134545 | 1.77 | 0.015 | 0.018 | -0.10 |
| 85 | -90.18365622 | 29.11015654 | 1.86 | 0.011 | 0.024 | -0.10 |
| 86 | -90.17724454 | 29.11476665 | 1.48 | 0.017 | 0.028 | 0.02 |
| 87 | -90.17531057 | 29.11601451 | 1.50 | 0.015 | 0.017 | 0.00 |
| 88 | -90.17340744 | 29.117274 | 1.33 | 0.024 | 0.015 | 0.05 |
| 89 | -90.17150143 | 29.11851633 | 1.38 | 0.007 | -0.008 | 0.04 |

2. References

Cohen, M., de Souza, A. V., Liu, K. biu, Rodrigues, E., Yao, Q., Ryu, J., Dietz, M., Pessenda, L.C.R., Rossetti, D., 2021. Effects of the 2017–2018 winter freeze on the northern limit of the American mangroves, Mississippi River delta plain, Geomorphology. Elsevier. https://doi.org/10.1016/J.GEOMORPH.2021.107968

Cohen, M.C.L., Souza, A.V. de, Liu, K.-B., Rodrigues, E., Yao, Q., Pessenda, L.C.R., Rossetti, D., Ryu, J., Dietz, M., 2021. Effects of Beach Nourishment Project on Coastal Geomorphology and Mangrove Dynamics in Southern Louisiana, USA. Remote Sens. 13, 2688. https://doi.org/10.3390/RS13142688

Global Mapper User’s Manual [WWW Document], 2020. URL http://www.globalmapper.it/helpv11/Help_Main.html (accessed 4.1.20).