Supplementary information

Light-Sheet Laser Speckle Imaging

Kai Long1, Keertana Vinod Ram1, Shuhao Shen2, E Du3, Ziheng Ren4, Zhiyuan Gong4, and Nanguang Chen1, 5, *

1Department of Biomedical Engineering, National University of Singapore, 4 Engineering Drive 3, S117583, Singapore

2Xidian University, Guangzhou Institute of Technology, Guangzhou 510555, China

3School of Microelectronics, Shenzhen Institute of Information Technology, Shenzhen 518172, China

4 Department of Biological Sciences, National University of Singapore, 16 Science Drive 4, S117558, Singapore

5 NUS (Suzhou) Research Institute, No. 377 Linquan Street, Suzhou Industrial Park, Suzhou, Jiangsu, China

Corresponding author

*Correspondence to: Nanguang Chen，biecng@nus.edu.sg

List of supplementary information

NOTE S1. Laser speckle signal characteristics (Page 2)

NOTE S1. Laser speckle signal characteristics

Here we provide additional analysis of raw laser speckle signals obtained from the trunk region of a 3-dpf zebrafish larva (Fig. 3c). Shown in Fig. S1a is a representative temporal speckle signal picked from one pixel in the artery (DA). Its autocorrelation function and frequency domain power spectrum are plotted in Fig. S1b and Fig. S1c, respectively. In a smaller delay time window (Fig. S1d), two different exponential decay patterns are visible in the experimental data (blue dots) and the curve can be best fitted to

g2τ=1+β0.602e-2τ0.0206+0.426e-2τ20.003662. 			(1)

Here the first exponential term (green dashed) corresponds to single scattering unordered motion (n=1) with a relatively long decorrelation time of 20.5 milliseconds. It is associated with the slow background random motion of blood cells as well as the entire animal at an effective velocity of 39.7 μm/s. The second exponential term (light blue) is linked to single scattering ordered motion (n=2) with a much smaller decorrelation time of 3.66 milliseconds, translating to a flow velocity of 223 μm/s.

An example speckle signal from the vein (PCV) is plotted in Fig. S1e, which is identical to that in Fig. 3c2. One can see in its power spectrum (Fig. S1e) a strong side lobe, which is concentrated in a small frequency range and appears as a sharp peak. In comparison, the side lobe in the DA signal power spectrum (Fig. S1c) is much weaker and broadly distributed. Consequently, the DA autocorrelation function contains negligible fast ripples while a strong, periodic modulating pattern can be seen in the PCV g2τ, which may lead to additional challenges in model fitting.

Figure S1| DA and PCV laser speckle signal characteristics. (a) Raw laser speckle signal from a pixel in the DA, (b) its autocorrelation function, and (c) frequency domain power spectrum. (d) Fitting of the DA autocorrelation function (blue dots) with the sum (red) of one background motion term (green dashed) and one flow velocity term (light blue). (e) Raw laser speckle signal from a pixel in the PCV and (f) its frequency domain power spectrum.