Supplementary Information

Hydrothermal-alkaline conversion of sewage sludge: Focus on optimization of process parameters and characterization of humic acid

List of supplementary information

Text S1 Water content measuring method.

Text S2 The determination method of the content of HA in unit mass of dry sludge

Table S1 Analysis of variance of the response surface quadratic model for sludge dewatering performance

Table S2 Analysis of variance of the response surface quadratic model on extraction rate of humic acid

Fig. S1 Normal plots of residuals showing the influences of date of sludge dewatering performance.

Fig. S2 Normal plots of residuals showing the influences of date of HA extraction rate.

Text S1 Water content measuring method

In this experiment, the moisture tester (MB 23, Ohaus Instruments GmbH) was used to determine the water content of sludge. The method comprises the following steps of: (1) Removing the mass of a sample disc. (2) Uniformly dispersing 0.6 g of sludge sample on the sample disc, weighing, covering a heating cover, heating at 105 °C. (3) Directly reading the water content of the sludge after measurement.

Text S2 The determination method of the content of HA in unit mass of dry sludge [1, 2]

The quantitative method of sludge humic acid refers to the method of determination of total humic acid content in mineral sources in GBT34766-2017. The method comprises the following steps of: (1) Extracting a raw material with sodium pyrophosphate alkali solution to obtain an extracting solution. (2) Precipitating under an acidic condition, separating and removing substances soluble under the acidic condition such as fulvic acid and the like, precipitating with a dilute alkali solution. (3) Oxidizing the extracted humic acid with potassium dichromate under a strong acid condition, titrating excessive potassium dichromate with ferrous ammonium sulfate standard solution. (4) Calculating the result according to the consumption of the ferrous ammonium sulfate standard solution. The results are expressed as total humic acid carbon content. (5) Finally calculate the content of humic acid by using the carbon-containing coefficient of the biochemical humic acid.

Table S1 Analysis of variance of the response surface

quadratic model for sludge dewatering performance

| Sources of variation | Sum of squares | Degrees of freedom | Mean square | F-value | p-value | comment |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 1353.92 | 9 | 150.44 | 69.91 | ＜0.0001 | Significant |
| X1 | 0.44 | 1 | 0.44 | 0.21 | 0.6642 | Insignificant |
| X2 | 573.08 | 1 | 573.08 | 266.34 | ＜0.0001 | Significant |
| X3 | 508.96 | 1 | 508.96 | 236.54 | ＜0.0001 | Significant |
| X1X2 | 2.61 | 1 | 2.61 | 1.21 | 0.3073 | Insignificant |
| X1X3 | 2.81 | 1 | 2.81 | 1.30 | 0.2911 | Insignificant |
| X2X3 | 34.46 | 1 | 34.46 | 16.01 | 0.0052 | Insignificant |
| X12 | 179.05 | 1 | 179.05 | 83.21 | ＜0.0001 | Significant |
| X22 | 42.27 | 1 | 42.27 | 19.65 | 0.0030 | Significant |
| X32 | 0.19 | 1 | 0.19 | 0.088 | 0.7759 | Insignificant |
| Residual | 15.06 | 7 | 2.15 |  |  |  |
| Lack of Fit | 3.32 | 3 | 1.11 | 0.38 | 0.7753 | Insignificant |
| Pure Error | 11.74 | 4 | 2.93 |  |  |  |
| Total | 1168.99 | 16 |  |  |  |  |
| Std. Dev. | R-Squared | C.V% | Adj R-Squared |  | Adequacy Precision |  |
| 1.47 | 0.9890 | 2.53 | 0.9749 |  | 29.226 |  |

Table S2 Analysis of variance of the response surface

quadratic model on extraction rate of humic acid

| Sources of variation | Sum of squares | Degrees of freedom | Mean square | F-value | p-value | comment |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 4191.82 | 9 | 465.76 | 90.19 | ＜0.0001 | Significant |
| X1 | 208.18 | 1 | 208.18 | 40.31 | 0.0004 | Insignificant |
| X2 | 588.25 | 1 | 588.25 | 113.91 | ＜0.0001 | Significant |
| X3 | 2103.09 | 1 | 2103.09 | 407.26 | ＜0.0001 | Significant |
| X1X2 | 0.31 | 1 | 0.31 | 0.061 | 0.8124 | Insignificant |
| X1X3 | 442.05 | 1 | 442.05 | 85.60 | ＜0.0001 | Significant |
| X2X3 | 43.03 | 1 | 43.03 | 8.33 | 0.0234 | Insignificant |
| X12 | 0.19 | 1 | 0.19 | 0.037 | 0.8524 | Insignificant |
| X22 | 0.0048 | 1 | 0.0048 | 0.0009 | 0.9765 | Insignificant |
| X32 | 803.16 | 1 | 803.16 | 155.53 | ＜0.0001 | Significant |
| Residual | 36.15 | 7 | 5.16 |  |  |  |
| Lack of Fit | 5.87 | 3 | 1.96 | 0.26 | 0.8524 | Insignificant |
| Pure Error | 30.28 | 4 | 7.57 |  |  |  |
| Total | 4227.97 | 16 |  |  |  |  |
| Std. Dev. | R-Squared | C.V% | Adj R-Squared |  | Adequacy Precision |  |
| 2.27 | 0.9915 | 3.38 | 0.9805 |  | 30.744 |  |

Table S3 Sludge dewatering method and dewatering effect in this study

and its comparison with previous report

| Sludge dewatering method | Water content after dewatering |
| --- | --- |
| Thermal-alkali treatment | 46.7% (centrifugation) |
| One-step acidﬁcation sludge dewatering method by using oxalic acid as a conditioner [3] | 60% (membrane ﬁlter press) |
| Thermal hydrolysis pre-treatment [4] | 55.38 (membrane ﬁlter press) |
| alkaline hydrothermal pretreatment [5] | 60.5% (centrifugation) 31 % (ﬁlter press) |

Fig. S1 Normal plots of residuals showing the influences of date of sludge dewatering performance on the (a) Internally Studentized Residuals, (b) Externally Studentized Residuals,

(c) Leverage, (d) DEFITS, (e) DFBETAS, (f) Cook’s Distance.

Fig. S2 Normal plots of residuals showing the influences of date of HA extraction rate on the (a) Internally Studentized Residuals, (b) Externally Studentized Residuals,

(c) Leverage, (d) DEFITS, (e) DFBETAS, (f) Cook’s Distance.

References

[1].	GBT 34766-2017 Determination of total humic acid content of mineral sources.

[2] Li et al., 2009 H. Li, Y. Jin, Y. Nie

Application of alkaline treatment for sludge decrement and humic acid recovery

Bioresour. Technol., 100 (24) (2009), pp. 6278-6283

[3] Chen et al., (2020) N. Chen, S. Tao, K. Xiao, S. Liang, J. Yang, L. Zhang

A one-step acidification strategy for sewage sludge dewatering with oxalic acid

Chemosphere, 238 (2020), p. 124598

[4] Li et al., 2017 C. Li, X. Wang, G. Zhang, G. Yu, J. Lin, Y. Wang

Hydrothermal and alkaline hydrothermal pretreatments plus anaerobic digestion of sewage sludge for dewatering and biogas production: Bench-scale research and pilot-scale verification

Water Res., 117 (2017), pp. 49-57

[5]	Kim et al., 2020 H. Kim, K. Chon, Y. Lee, Y. Kim, A. Jang

Enhanced mechanical deep dewatering of dewatered sludge by a thermal hydrolysis pre-treatment: Effects of temperature and retention time

Environ. Res., 188 (2020), p. 109746