# Additional file 1:

# MRI and CT scanning parameters and image scoring protocol

Table S1: Computed tomography acquisition parameters

|  | Peak voltage (kV) | Tube current (mAs) | FOV (mm) | Matrix | Scan slice thickness (mm) | Recon Increment (mm) | Pitch | Recon kernel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First | 130 | 60 | 1464 x 762 | 512 x 512 | 2.0 | 1.0 | 2.0 | B80s very sharp |
| Second | 130 | 60 | 1464 x 762 | 512 x 512 | 1.0 | 0.5 | 2.0 | B80s very sharp |

Abbreviations: kV, kilovolt; mAs, miliampere second; FOV, field of view.

Table S2: Magnetic resonance imaging acquisition parameters

|  | TR (ms) | TE (ms) | Fat sat | Slice Thickness (mm) | FOV (mm) | Matrix | NEX |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sagittal T1 | 750 | 26 | - | 6 | 1809 x 942 | 256 x 256 | 4 |
| Axial T1 | 860 | 26 | - | 6 | 1809 x 942 | 256 x 256 | 3 |
| Coronal T1 | 850 | 26 | - | 5 | 1809 x 942 | 256 x 256 | 4 |
| Sagittal T2 | 2800 | 90 | - | 6 | 1809 x 942 | 256 x 256 | 1 |
| Axial T2 | 2800 | 90 | - | 6 | 1809 x 942 | 256 x 256 | 1 |
| Coronal T2 | 2800 | 90 | - | 5 | 1809 x 942 | 256 x 256 | 1 |

Abbreviations: TR, repetition time; TE, echo time; Fat sat, fat saturation; FOV, field of view; NEX, number of excitations.

Table S3: Guidelines for scoring of MRI and CT images

|  | CT/MRI | Description |  | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCJ | MRI | Synovial effusion | ﻿﻿Joint effusion is judged based on the largest distance between the bone and the fibrous joint capsule at the most distended part of the middle carpal joint viewed on T2 sagittal MRI images. The distance is measured from the dorso-distal edge of any of the bones in the proximal row of carpal bones perpendicular to the long axis of this bone. | < 2 mm | 2 – 3.9 mm. | ≥ 4 mm |  |  |
|  | MR | Synovial proliferation | Synovial proliferation was recognized as areas of reduced signal intensity protruding into the hyperintense synovial fluid. This was judged on T2W MRI and subjectively graded as: | No enlargement of synovium in dorsal aspect of the middle carpal joint. | Localized areas of reduced signal intensity (enlarged synovium) estimated as less than 50% of synovial fluid volume in the dorsal aspect of the middle carpal joint. | Localized areas of reduced signal intensity (enlarged synovium) estimated as greater than or equal to 50% of synovial fluid volume in the dorsal aspect of the middle carpal joint. |  |  |
| Carpal bone areas | CT | Osteophytes/ enthesophytes | Based on number of osteophytes and the size of the largest osteophyte in each subregion | none | 1 - 2 and largest less than 2mm | more than 2 or largest larger than 2 mm |  |  |
|  | CT | Subchondral sclerosis (subjective) | Subchondral sclerosis is evaluated on both CT and MRI T1W images. Sclerosis is defined as areas of reduced signal on T1 MRI or high density on CT. The axial slice with the largest lesion is evaluated. A ROI is selected (on T1W MRI) and the lesion size/area is compared with the entire sub-region and classified as: | no abnormalities | < 25% of area affected | 26 - 50 % of area affected | More than 50% of area affected |  |
|  | CT | Subchondral sclerosis (objective) | Subchondral sclerosis is defined as before, but regions are quantitatively measured, and a grade was assigned based on sclerosis versus normal ratio: | 0 | 0.1 - 0.25 | 0.26 - 0.5 | >0.5 |  |
|  | MRI | Bone edema-like signal | Defined as poorly marginated areas of increased signal intensity in the normally hypointense trabecular or subchondral bone on MRI T2W images. The axial slice with the largest lesion is evaluated. A ROI is selected, and the lesion area is compared with the entire sub-region and classified as | no abnormalities | <25% of area affected | more than 25% of area affected |  |  |
|  | MRI/CT | Articular cartilage | ﻿Cartilage is evaluated on MRI images and the subchondral bone is evaluated on both MRI and CT images. Abnormalities are scored based on changes in signal and morphology and graded as | no abnormalities, smooth cartilage lining, smooth and regular chondro-osseous junction | irregular cartilage lining, partial- and full thickness lesions with no involvement of the subchondral bone plate. | full thickness cartilage lesions with involvement of the subchondral bone plate. |  |  |
| RCB | CT | Fragment healing | Evidence of healing was assessed by grading new bone formation between the fragment and parent bone. | no evidence of healing | incomplete healing | Complete healing, continuity of fragment with parent bone. |  |  |
|  | CT | Fragment size | Fragment area was calculated by the maximal oblique diameter and its orthogonal dimension, cross-sectional area on transverse plane CT images | No fragment | 1 - 20mm2 | 21 - 50 mm2 | 51 - 100 mm2 | > 1 cm2 |
|  | CT | Fragment score | The fragment size was assigned to a grade: | No fragment | 1 - 20mm2 | 21 - 50 mm2 | 51 - 100 mm2 | > 1 cm2 |

Abbreviations: MCJ, middle carpal joint; RCB, radial carpal bone; MRI, magnetic resonance imaging; CT, computed tomography.