Appendix A

Table A1 Data used for the calculation of cost benefit ratio and incremental cost

| Number | Item | Formula | Nomenclature |
| --- | --- | --- | --- |
| 1 | Total processing cost in AM | CTotal= CMaterial+ CSetup+COperation+ CRemoval | CMaterial – Material cost (USD) CSetup – Setup cost (USD) COperation– Operational cost (USD) CRemoval – Part removal cost (USD) |
| 2 | Operation cost | COperation= TBuild(CAM+CGas) | TBuild – Build time for the part (h) CAM – Hourly rate of AM machine (USD/h) CGas – Hourly rate of inert gas consumed (USD/h) |
| 3 | Build Time | TBuild= TBuildrate (Vp+Vs)+ Trecoat | TBuildrate – Time taken to build unit cm3 of material (h/cm3) Vp – Volume of the part (cm3) Vs – Volume of support structure (cm3) Trecoat – Recoat time (h) |
| 4 | Recoat time | Trecoat= (Trecoat rate×No. of layers)3600 | Trecoat rate – Average time taken to spread one layer of powder (s/layer) |
| 5 | Setup cost | CSetup= TSetup(CAM+Coper) | TSetup – Setup time for machine Coper – Hourly rate of operator (USD/h) |
| 6 | Material cost | CMaterial=M×Cmu | M – Mass of the material consumed Cmu – Unit cost of material (Ti6Al4V) |
| 7 | Mass of the material consumed | M=1.4 ρw(Vp+Vs)+ 0.25 ρtVs | ρw – wrought density of material (g/cm3) ρt – Tap density of material (g/cm3) |
| 8 | Part removal cost | CRemoval= TRem(CAM+Coper) | TRem – Time required to remove the part (h) |
| 9 | Processing cost of support structure | CS=CD+CM+CP | CD – Design cost of support structure (USD) CM – Manufacturing cost of support structure (USD) CP – Post processing cost of support structure (USD) |
| 10 | Design cost of support structure | CD=CO,D+CE,D+CR,D | CO,D - Operator cost for design (USD) CE,D – Equipment cost for design (USD) CR,D – Office space cost for design (USD) |
| 11 | Operator cost for design | CO,D= tod Cwgd+ Cwid | tod – operators time for designing (h) Cwgd – Hourly wage of designer (USD/h) Cwid – Average incidental age (USD/h) |
| 12 | Equipment cost for design | CE,D= tEDuswCsw+ tEDuHW CIHWtaHW | tED – Equipment usage time for designing (h) usw – Annual utilization of design software (h/a) Csw - License cost of design software (USD/a) uHW – Annual utilization of computational power (h/a) CIHW – Investment cost of computer hardware (USD) taHW – Amortization period of computer hardware (year) |
| 13 | Office space cost for design | CR,D= tEDuOAO(CRRO+CROO) | uO - Annual utilization of office workspace (h/a) AO - Standard area office space used (m2) CRRO - Average rent of office space (USD/m2) CROO - Average operation cost of office space (USD/m2) |
| 14 | Manufacturing cost of support structure | CM = CM,M+CP,M,M+CP,F,M+ CP,G,M+ CP,E,M | CM,M – Material cost of support structure CP,M,M – Production cost of support structure CP,F,M Cost due to gas filter CP,G,M – Cost of post processing gas. CP,E,M – Cost of energy for production |
| 15 | Material cost | CM,M= ρVsCmat- Cscrap | ρ – Density of the support material (kg/m3) Vs – Volume of support structure (m3) Cmat – material cost of support structure (USD/kg) Cscrap Scrap price of solid titanium alloy USD/kg) |
| 16 | Cost of production | CP,M,M= tBuMCIMtaM+ AMCRRM+ CROM+ CMSer | tB – Build time (h) uM – Annual utilization of L-PBF machine (h/a) CIM - Investment cost of L-PBF machine (USD) taM - Amortization period of L-PBF machine (year) AM – space for L-PBF machine (m2) CRRM - Average rent for machine space (USD/m2) CROM - Average operation cost of machine space (USD/m2) CMSer - Annual service cost of L-PBF machine (USD/a) |
| 17 | Build time | tB= VsM+ ∆hth (tp+tDel) | M – melt rate of support structure ∆h - Offset height of support structure (mm) th - Height from the build plate (mm) tp - Recoating time for L-PBF machine (s) tDel - Delay time for L-PBF machine (s) |
| 18 | Melt rate | M= vsdhthnL | vs – Scan speed (mm/s) dh - Hatch spacing (mm) th - Layer thickness (mm) nL – Number of layers |
| 19 | Cost of gas filter | CP,F,M= tBuMCIFtaF | CIF – Investment cost of filter (USD) taF - Amortization period of L-PBF machine gas filter (year) |
| 20 | Cost of post processing gas | CP,G,M= tBCG | CG - Cost of gas (USD) |
| 21 | Cost of energy for post processing | CP,E,M= tBCE | CE - Cost energy supplied to the L-PBF machine (USD) |
| 22 | Post processing cost of support structure | CP=CSubstrate+CPostp | CSubstrate – Cost of substrate post processing (USD) CPostp – Cost of post processing (USD) |
| 23 | Cost of substrate post processing | CSubstrate= CStress+ CEDM | CStress - Cost for stress relieving the build plate (USD) CEDM - Cost of separating parts from the build plate |
| 24 | Cost of post processing the part | CPostp= TPostpCop+ Ctools | TPostp - Post processing time (h) Cop – Operators hourly rate (USD/h) Ctools - Cost of tools used for post processing (USD/h) |