Supplementary Materials

Supplementary Table 1. The 5-point Likert scoring for the subjective assessment of PET image quality.

|  | Overall quality of MIP images | Organ boundaries | Image noise | Brain delineation |
| --- | --- | --- | --- | --- |
| 5 | state-of-the-art quality | well-defined boundaries of liver, spleen, and bone | near-imperceptible noise of liver, spleen, and bone | well-defined sulci gyri and basal ganglia |
| 4 | superior to the average | fairly defined | lower than the regular image of daily practice | fairly defined |
| 3* | regular quality of daily practice | hazy, recognizable | similar to the regular image of daily practice | hazy, recognizable |
| 2 | barely diagnostic | ill-defined, impairing spatial resolution of organs | increased noise, slightly worse than the regular image of daily practice | ill-defined, impairing spatial resolution of organs |
| 1 | nondiagnostic | unrecognizable | excessive noise | unrecognizable |

* MIP: Maximum intensity projection

Supplementary Table 2. Objective image quality parameters of background in total-body PET/CT (n = 31).

|  | Group | 20 min | 12 min | 10 min | 8 min | 6 min | 5 min | 4 min | 3 min | 2 min | 1.5 min | 1 min | 0.5 min |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR_Liver | 1 (n=6) | 23.2 ± 5.2 | 23.1 ± 5.6 | 23.0 ± 5.7 | 20.2 ± 4.3 | 18.6 ± 3.2 | 16.7 ± 1.6 | 16.5 ± 1.9 | 14.3 ± 1.5 | 11.5 ± 1.3 | 10.2 ± 1.2 | 8.3 ± 0.9 | 6.6 ± 1.5 |
|  | 1/2 (n=10) | 19.9 ± 4.2 | 19.5 ± 3.8 | 18.8 ± 4.0 | 15.6 ± 3.2 | 15.6 ± 3.4 | 12.9 ± 2.4 | 12.6 ± 2.3 | 11.1 ± 2.5 | 9.4 ± 1.3 | 8.0 ± 1.2 | 6.7 ± 1.2 | 4.6 ± 0.9 |
|  | 1/3 (n=8) | 21.6 ± 3.3 | 19.6 ± 2.9 | 17.1 ± 4.0 | 14.0 ± 2.2 | 13.2 ± 1.5 | 11.3 ± 1.0 | 11.2 ± 0.9 | 10.5 ± 0.9 | 8.7 ± 1.0 | 7.3 ± 0.8 | 6.1 ± 1.0 | 4.3 ± 0.6 |
|  | 1/4 (n=7) | 20.4 ± 4.4 | 17.7 ± 3.5 | 15.2 ± 3.3 | 12.2 ± 2.8 | 11.9 ± 2.4 | 10.2 ± 1.8 | 9.4 ± 1.6 | 8.2 ± 1.5 | 6.9 ± 1.2 | 5.9 ± 0.8 | 5.0 ± 0.7 | 3.4 ± 0.6 |
| Liver SUVmax | 1 (n=6) | 1.5 ± 0.2 | 1.5 ± 0.2 | 1.8 ± 0.3 | 1.5 ± 0.2 | 1.5 ± 0.2 | 1.6 ± 0.3 | 1.9 ± 0.3 | 1.7 ± 0.3 | 2.1 ± 0.6 | 1.7 ± 0.3 | 2.1 ± 0.4 | 1.6 ± 0.2 |
|  | 1/2 (n=10) | 1.7 ± 0.4 | 1.7 ± 0.4 | 2.1 ± 0.5 | 1.8 ± 0.4 | 1.7 ± 0.4 | 1.9 ± 0.4 | 2.3 ± 0.5 | 2.0 ± 0.5 | 2.6 ± 0.7 | 1.9 ± 0.4 | 2.4 ± 0.6 | 1.8 ± 0.4 |
|  | 1/3 (n=8) | 2.1 ± 0.6 | 2.1 ± 0.6 | 2.6 ± 0.9 | 2.2 ± 0.7 | 2.1 ± 0.6 | 2.3 ± 0.7 | 2.8 ± 0.9 | 2.5 ± 0.8 | 3.3 ± 1.1 | 2.4 ± 0.7 | 3.2 ± 1.2 | 2.3 ± 0.7 |
|  | 1/4 (n=7) | 1.9 ± 0.5 | 1.9 ± 0.6 | 2.5 ± 0.7 | 2.0 ± 0.5 | 1.9 ± 0.5 | 2.2 ± 0.6 | 2.7 ± 0.7 | 2.4 ± 0.7 | 3.4 ± 0.9 | 2.3 ± 0.6 | 3.0 ± 0.9 | 2.0 ± 0.5 |
| Liver SUVmean | 1 (n=6) | 1.3 ± 0.2 | 1.3 ± 0.2 | 1.4 ± 0.2 | 1.4 ± 0.2 | 1.4 ± 0.2 | 1.4 ± 0.2 | 1.2 ± 0.5 | 1.4 ± 0.2 | 1.4 ± 0.2 | 1.4 ± 0.2 | 1.4 ± 0.2 | 1.4 ± 0.2 |
|  | 1/2 (n=10) | 1.5 ± 0.3 | 1.5 ± 0.3 | 1.5 ± 0.3 | 1.5 ± 0.3 | 1.5 ± 0.3 | 1.56± 0.3 | 1.6 ± 0.3 | 1.5 ± 0.4 | 1.6 ± 0.3 | 1.6 ± 0.3 | 1.6 ± 0.3 | 1.6 ± 0.3 |
|  | 1/3 (n=8) | 1.8 ± 0.5 | 1.8 ± 0.5 | 1.8 ± 0.5 | 1.9 ± 0.5 | 1.9 ± 0.5 | 1.9 ± 0.5 | 1.9 ± 0.5 | 1.9 ± 0.5 | 1.9 ± 0.6 | 1.9 ± 0.6 | 1.9 ± 0.6 | 1.9 ± 0.6 |
|  | 1/4 (n=7) | 1.6 ± 0.4 | 1.6 ± 0.4 | 1.6 ± 0.5 | 1.6 ± 0.4 | 1.7 ± 0.5 | 1.7 ± 0.5 | 1.7 ± 0.5 | 1.7 ± 0.5 | 1.7 ± 0.5 | 1.7 ± 0.5 | 1.7 ± 0.5 | 1.7 ± 0.5 |
| Liver SD | 1 (n=6) | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.2 ± 0.0 | 0.2 ± 0.1 |
|  | 1/2 (n=10) | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.2 ± 0.0 | 0.2 ± 0.0 | 0.2 ± 0.1 | 0.3 ± 0.1 |
|  | 1/3 (n=8) | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.2 ± 0.1 | 0.2 ± 0.0 | 0.2 ± 0.1 | 0.2 ± 0.1 | 0.3 ± 0.1 | 0.3 ± 0.1 | 0.5 ± 0.2 |
|  | 1/4 (n=7) | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.0 | 0.1 ± 0.1 | 0.2 ± 0.1 | 0.2 ± 0.1 | 0.2 ± 0.1 | 0.3 ± 0.1 | 0.3 ± 0.1 | 0.5 ± 0.1 |

*All values are presented as the mean ± SD.

*The signal-to-noise ratio (SNR) of the liver was calculated by dividing the liver SUVmean in the VOI by its SD.

Supplementary Table 3. Objective image quality parameters of major lesions in total-body PET/CT (n = 34).

|  | Group | 20 min | 12 min | 10 min | 8 min | 6 min | 5 min | 4 min | 3 min | 2 min | 1.5 min | 1 min | 0.5 min |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBR_Liver | 1 (n=8) | 6.5 ± 4.8 | 6.6 ± 4.9 | 6.2 ± 4.2 | 6.4 ± 4.8 | 6.1 ± 4.2 | 6.4 ± 4.8 | 6.5 ± 3.7 | 6.2 ± 4.2 | 6.3 ± 4.2 | 6.1 ± 4.0 | 6.1 ± 4.0 | 6.4 ± 4.3 |
|  | 1/2 (n=13) | 6.9 ± 5.3 | 6.8 ± 5.3 | 7.1 ± 5.5 | 6.6 ± 5.2 | 7.1 ± 5.6 | 6.7 ± 5.3 | 7.2 ± 5.7 | 7.4 ± 5.7 | 7.2 ± 5.7 | 7.1 ± 5.5 | 7.0 ± 5.5 | 7.4 ± 5.8 |
|  | 1/3 (n=6) | 7.6 ± 7.4 | 7.5 ± 7.5 | 7.5 ± 9.2 | 7.2 ± 7.1 | 7.4 ± 9.0 | 7.2 ± 7.1 | 7.1 ± 8.5 | 7.1 ± 8.6 | 7.1 ± 8.6 | 7.0 ± 8.8 | 7.1 ± 9.2 | 7.0 ± 9.2 |
|  | 1/4 (n=7) | 3.4 ± 1.6 | 3.2 ± 1.5 | 3.5 ± 2.0 | 3.3 ± 1.5 | 3.4 ± 2.0 | 3.2 ± 1.5 | 3.5 ± 1.9 | 3.4 ± 1.9 | 3.4 ± 1.9 | 3.5 ± 2.0 | 3.6 ± 2.0 | 3.7 ± 2.1 |
| Lesion SUVmax | 1 (n=8) | 8.4 ± 5.3 | 8.4 ± 5.4 | 8.4 ± 5.6 | 8.4 ± 5.7 | 8.4 ± 5.7 | 8.5 ± 5.8 | 8.3 ± 5.6 | 8.6 ± 5.8 | 8.8 ± 5.9 | 8.7 ± 5.7 | 8.7 ± 5.8 | 9.2 ± 6.2 |
|  | 1/2 (n=13) | 9.3 ± 6.5 | 9.3 ± 6.7 | 9.4 ± 6.7 | 9.4 ± 6.9 | 9.3 ± 6.8 | 9.5 ± 6.9 | 9.6 ± 6.9 | 9.5 ± 6.8 | 9.5 ± 6.7 | 9.6 ± 6.8 | 9.5 ± 6.7 | 9.9 ± 7.0 |
|  | 1/3 (n=6) | 14.5 ± 17.8 | 14.5 ± 18.1 | 14.8 ± 18.2 | 14.8 ± 17.9 | 14.9 ± 18.1 | 14.5 ± 17.9 | 14.5 ± 17.5 | 14.7 ± 17.9 | 14.6 ± 17.9 | 14.6 ± 18.3 | 14.7 ± 19.1 | 14.7 ± 19.3 |
|  | 1/4 (n=7) | 5.3 ± 3.2 | 5.2 ± 3.1 | 5.3 ± 3.2 | 5.1 ± 3.2 | 5.1 ± 3.2 | 5.3 ± 3.1 | 5.3 ± 3.1 | 5.2 ± 3.0 | 5.2 ± 3.1 | 5.4 ± 3.2 | 5.4 ± 3.3 | 5.7 ± 3.5 |

*All values are presented as the mean ± SD.

*The tumour-to-background ratio (TBR) was calculated by dividing the SUVmax of the major lesion by the SUVmean of the liver.

Supplementary Table 4. Subjective image quality was assessed by a 5-point Likert scale.

|  | Group | 20 min | 12 min | 10 min | 8 min | 6 min | 5 min | 4 min | 3 min | 2 min | 1.5 min | 1 min | 0.5 min |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall quality | 1 (n=6) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.2 ± 0.4 | 3.5 ± 0.5 | 2.0 ± 0.9 |
|  | 1/2 (n=10) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.6 ± 0.5 | 3.8 ± 0.8 | 2.8 ± 0.8 | 2.7 ± 0.9 | 1.6 ± 0.5 |
|  | 1/3 (n=8) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.9 ± 0.4 | 4.2 ± 0.5 | 3.5 ± 0.9 | 2.5 ± 0.9 | 2.4 ± 1.1 | 1.2 ± 0.5 |
|  | 1/4 (n=7) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.6 ± 0.5 | 4.4 ± 0.5 | 3.9 ± 0.8 | 3.3 ± 0.5 | 2.6 ± 0.5 | 2.0 ± 0.0 | 1.7 ± 0.5 | 1.1 ± 0.4 |
| Organ boundaries | 1 (n=6) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.2 ± 0.4 | 3.2 ± 0.4 | 3.0 ± 0.6 | 1.5 ± 0.5 |
|  | 1/2 (n=10) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.5 ± 0.5 | 4.0 ± 0.5 | 3.1 ± 0.3 | 2.1 ± 0.3 | 2.0 ± 0.5 | 1.3 ± 0.5 |
|  | 1/3 (n=8) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.5 ± 0.5 | 4.0 ± 0.5 | 3.6 ± 0.7 | 3.1 ± 0.4 | 2.5 ± 0.5 | 1.8 ± 0.5 | 1.2 ± 0.5 | 1.0 ± 0.0 |
|  | 1/4 (n=7) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.4 ± 0.5 | 3.6 ± 0.5 | 3.3 ± 0.5 | 3.0 ± 0.0 | 2.3 ± 0.5 | 2.0 ± 0.0 | 2.0 ± 0.0 | 1.3 ± 0.5 | 1.1 ± 0.4 |
| Image noise | 1 (n=6) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.2 ± 0.4 | 3.2 ± 0.4 | 3.0 ± 0.6 | 1.5 ± 0.5 |
|  | 1/2 (n=10) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.9 ± 0.3 | 4.2 ± 0.4 | 3.8 ± 0.4 | 3.1 ± 0.3 | 2.1 ± 0.3 | 2.0 ± 0.5 | 1.3 ± 0.5 |
|  | 1/3 (n=8) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.5 ± 0.5 | 3.9 ± 0.4 | 3.5 ± 0.5 | 3.0 ± 0.0 | 2.5 ± 0.5 | 1.8 ± 0.5 | 1.1 ± 0.4 | 1.0 ± 0.0 |
|  | 1/4 (n=7) | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.7 ± 0.5 | 4.3 ± 0.5 | 3.4 ± 0.5 | 3.1 ± 0.4 | 2.7 ± 0.5 | 2.1 ± 0.4 | 2.0 ± 0.0 | 1.9 ± 0.4 | 1.3 ± 0.5 | 1.0 ± 0.0 |
| Brain recognition | 1 (n=6) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 3.2 ± 0.4 |
|  | 1/2 (n=10) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.0 ± 0.0 | 4.0 ± 0.0 | 2.0 ± 0.0 |
|  | 1/3 (n=8) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.9 ± 0.4 | 4.1 ± 0.4 | 3.2 ± 0.7 | 2.4 ± 0.5 | 1.6 ± 0.5 |
|  | 1/4 (n=7) | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 5.0 ± 0.0 | 4.5 ± 0.5 | 4.0 ± 0.0 | 3.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 1.5 ± 0.5 |

*All scores are presented as the mean value ± SD.

*Dark blue, mean score = 5.0; Light blue, mean score = 4.0; Grey, mean score = 3.0, regular quality of daily practice; White, mean score < 3.0.

Supplementary Table 5. Lesion detectability and diagnostic confidence assessed by a 3-point Likert scale

|  | Group | 20 min | 12 min | 10 min | 8 min | 6 min | 5 min | 4 min | 3 min | 2 min | 1.5 min | 1 min | 0.5 min |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Major-lesion detectability (%) | 1 (n=8) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 50 |
|  | 1/2 (n=13) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 85 |
|  | 1/3 (n=6) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 50 |
|  | 1/4 (n=7) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 80 | 60 |
| Microlesion detectability (%) | 1 (n=7) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 14 |
|  | 1/2 (n=13) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 85 | 85 | 31 |
|  | 1/3 (n=11) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 91 | 91 | 36 |
|  | 1/4 (n=25) | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 84 | 52 | 52 | 36 |
| Diagnostic confidence of PET image alone | 1 (n=6) | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.4 | 1.7 ± 0.8 | 1.0 ± 0.6 | 0.0 ± 0.0 |
|  | 1/2 (n=10) | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 1.8 ± 0.4 | 1.4 ± 0.8 | 0.9 ± 0.9 | 0.9 ± 0.9 | 0.2 ± 0.4 |
|  | 1/3 (n=8) | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 1.9 ± 0.4 | 1.5 ± 0.8 | 0.6 ± 0.5 | 0.6 ± 0.5 | 0.0 ± 0.0 |
|  | 1/4 (n=7) | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 1.8 ± 0.4 | 1.2 ± 0.6 | 0.7 ± 0.9 | 0.3 ± 0.5 | 0.3 ± 0.5 | 0.0 ± 0.0 |
| Diagnostic confidence of PET/CT image | 1 (n=6) | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 1.3 ± 0.5 |
|  | 1/2 (n=10) | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.4 | 1.6 ± 0.7 | 1.0 ± 0.7 |
|  | 1/3 (n=8) | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 1.6 ± 0.5 | 1.1 ± 0.8 | 0.6 ± 0.7 |
|  | 1/4 (n=7) | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.0 | 2.0 ± 0.4 | 1.7 ± 0.5 | 1.6 ± 0.5 | 0.9 ± 0.7 | 0.7 ± 0.5 |

*All values are the lesion detection rate (LDR) in PET images (presented as a percentage) and the patients’ clinical diagnostic confidence scores of fused PET/CT images (presented as the mean value ± SD). 3-point Likert scale of diagnostic confidence: LOW (0, 50%–75% confidence), MODERATE (1, 76%–90% confidence), and HIGH (2, >90% confidence).

*Dark blue, 100% of lesions detected or high diagnostic confidence (i.e., >90% confidence) with mean score = 2.0.