Supplemental Text 1

MR imaging protocol

MRI was performed using a 3.0 Tesla (T) whole-body MRI system (Intera Achieva 3.0 T, Philips Healthcare, Best, The Netherlands) with a 16-channel phased array coil. All images were obtained in the axial plane, after 4-6 h of fasting. The contrast agent used was gadoxetic acid disodium (Gd-EOB-DTPA, Primovist; Bayer Schering, Pharma, Berlin, Germany) and a dose of 0.025 mmol/kg body weight was injected intravenously as a rapid bolus at a rate of 2 ml/s via a power injector and flushed with 20 ml NaCl. Before contrast agent administration, we acquired unenhanced MR images including fast spoiled dual-echo T1-weighted gradient-recalled-echo images (in-phase and out-phase), fat-suppressed fast spin-echo T2-weighted images (T2WI). We acquired enhanced MR images after Gd-EOB-DTPA administration: arterial phase (20–35 s, AP), portal phase (60 s, PP), transitional phase (3 min, TP), and hepatobiliary phase (20 min, HBP). The MRI parameters were detailed in the supplementary table1.

Supplementary Table 1. MRI scan sequence and parameters

| Sequence |  | Orientation | Category | TR | TE | FOV | Matrix | FA | Band | Seam thickness (mm) | Fat | Hold breath |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (ms) | (ms) | (mm) |  |  | width(HZ) |  | saturation |  |  |
| Philips Healthcare, Intera Achieva 3.0 T, with a 16-channel phased array coil.(training sets) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T2WI |  | TRA/ COR | TSE | 2000 | 70 | 325 × 392 | 232 × 202 | 90° | 513 | 7 | Yes | No |  |
| T1WI IN/OUT PHASE |  | TRA | T1-FFE | 150 | 2.3/1.15 | 375 × 319 | 236 × 200 | 55° | 1235 | 7 | Yes/No | Yes |  |
| CONTRAST- Enhanced T1WI |  | TRA/ COR | THRIVE | 3 | 1.5 | 385 × 332 | 248×212 | 10° | 723.7 | 2.5 | Yes | Yes |  |
| Siemens MAGNETOM Verio (testing sets) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T2WI |  | COR | TSE | 1200 | 83 | 400 | 256 × 256 | 160° | 501 | 6 | no | yes | haste |
| T2WI |  | TRA | TSE | 2200 | 97 | 380×380 | 320 × 320 | 134° | 326 | 6 | Yes | No |  |
| T1WI IN/OUT PHASE |  | TRA | T1-vibe | 4 | 1.23/2.46 | 380 × 380 | 288 × 288 | 9° | 1020 | 5 | Yes/No | Yes |  |
| CONTRAST- Enhanced T1WI |  | TRA/ COR | T1-vibe | 3 | 1.3 | 380 × 332 | 320×320 | 9° | 450 | 3 | Yes | Yes |  |

Notes: TRA, transverse axial; COR, coronal; TSE, turbo spin echo; T1-FFE, T1 fast field echo; T1-vibe,T1 volume interpolate bady examination; THRIVE, T1 high resolution isotropic volume examination.