Supplementary Material

Spatial Optimization of Photovoltaic-Based Hydrogen-Electricity Supply Network through an Integrated Geographical Information System and Mathematical Modelling Approach

Angel Xin Yee, Mah a, b, Wai Shin, Ho a, b, *, Mimi H., Hassim a, c, Haslenda, Hashim a, b, Zarina Ab, Muis a, b, Gabriel Hoh Teck, Ling d, Chin Siong, Ho d

a. School of Chemical and Energy Engineering, Faculty of Engineering, Universiti Teknologi Malaysia (UTM), 81310 UTM Johor Bahru, Johor, Malaysia

b. Process Systems Engineering Centre (PROSPECT), Research Institute of Sustainable Environment (RISE), Universiti Teknologi Malaysia (UTM), 81310 UTM Johor Bahru, Johor, Malaysia

c. Centre of Hydrogen Energy, Institute of Future Energy, Universiti Teknologi Malaysia (UTM), 81310 UTM Johor Bahru, Johor, Malaysia

d. UTM-Low Carbon Asia Research Centre, Program of Urban and Regional Planning, Faculty of Built Environment and Surveying, Universiti Teknologi Malaysia (UTM), Malaysia

*Corresponding author, tel: +60164401186, email: hwshin@utm.my, postal address: Universiti Teknologi Malaysia (UTM), 81310 UTM Johor Bahru, Johor, Malaysia

Nomenclature

| Sets |  |  |
| --- | --- | --- |
| c | Set of hydrogen in different storage and transportation forms |  |
| d | Set of pipelines with different diameters |  |
| j | Set of study regions |  |
| k | Set of study regions, identical to set j |  |
| Parameters |  |  |
| AjFeasible | Feasible area for construction at region j | m2 |
| BcCOMP | Binary defining whether hydrogen needs compression to be converted into form c |  |
| BcFC | Binary defining the form of hydrogen c that can be used for fuel cell |  |
| BcHYD | Binary defining whether hydrogen needs hydrogenation to be converted into form c |  |
| BcLIQ | Binary defining whether hydrogen needs liquefaction to be converted into form c |  |
| BcRF | Binary defining the form of hydrogen c that can be used in refuelling station |  |
| BcPipe | Binary defining the form of hydrogen c that can be transported with pipeline |  |
| Bj,kTrans | Binary defining whether the product can be transported from region j to region k |  |
| BcTruck | Binary defining the form of hydrogen c that can be transported with trailer |  |
| BATDOD | Battery depth of discharge |  |
| BATucapex | Battery unit capital cost | USD/kWh |
| BATuopex | Battery unit operating cost | USD/kWh/y |
| BSEff | Battery storage efficiency |  |
| Cableucapex | Power cable unit capital cost | USD/km |
| CableUL | Maximum capacity of power cable | MW |
| Cableuopex | Power cable unit operating cost | USD/km/y |
| Comptcrange,UL | Maximum limit of compressor with capacity level tc | kW |
| Comptcucapex | Unit capital cost of compressor tc | USD |
| Comptcuopex | Unit operating cost of compressor tc | USD/y |
| Compcupower | Unit power requirement to compress hydrogen to form c | kW/kg/h |
| CONVEff | Converter efficiency |  |
| CONVucapex | Converter unit capital cost | USD/kW |
| CONVuopex | Converter unit operating cost | USD/kW/y |
| CRF | Capital recovery factor |  |
| Dj,k | Transportation distance between region j and region k | km |
| DAutonomy | Days of autonomy | d |
| DHW | Hourly wage for truck driver | USD/h |
| Dieselcost | Diesel cost | USD/L |
| EjDemand | Electricity demand in region j | kWh/d |
| Elosses | Electricity losses during transmission and distribution |  |
| ELEff | Electrolyzer efficiency |  |
| ELucapex | Electrolyzer unit capital cost | USD/kW |
| ELuopex | Electrolyzer unit operating cost | USD/kW/y |
| Eleccost | Grid electricity cost | USD/kWh |
| fLand | Land-use factor based on the area of solar panel |  |
| FCEff | Fuel cell efficiency |  |
| FCucapex | Fuel cell unit capital cost | USD/kW |
| FCuopex | Fuel cell unit operating cost | USD/kW/y |
| FCVFE | Fuel economy of fuel cell vehicle | kg/km |
| HjDemand | Hydrogen demand in region j | kg/d |
| HE | Hydrogen energy content | kWh/kg |
| HPL | Ratio of daily load to peak load |  |
| HScucapex | Hydrogen storage unit capital cost | USD/kg |
| HScuopex | Hydrogen storage unit operating cost | USD/kg/y |
| Hydthrange,UL | Maximum throughput of hydrogenation equipment th | kg/d |
| Hydthucapex | Unit capital cost of hydrogenation equipment th | USD |
| Hydthuopex | Unit operating cost of hydrogenation equipment th | USD/y |
| i | Interest rate |  |
| Liqtlrange,UL | Maximum throughput of liquefaction equipment tl | kg/d |
| Liqtlucapex | Unit capital cost of liquefaction equipment tl | USD |
| Liqtluopex | Unit operating cost of liquefaction equipment tl | USD/y |
| LTc | Time for loading and unloading of hydrogen form c from trailers | h |
| Petrolcost | Petrol cost | USD/L |
| Pipedrange,UL | Maximum allowable throughput of pipe with diameter d | kg/d |
| Pipeducapex | Unit capital cost of pipeline with diameter d | USD/km |
| Pipeduopex | Unit operating cost of pipeline with diameter d | USD/km/y |
| PRcLosses | Hydrogen losses when being processed to convert into form c |  |
| PRVFE | Fuel economy of petrol vehicle | L/km |
| PVEff | Solar panel efficiency |  |
| PVucapex | Solar panel unit capital cost | USD/kWp |
| PVuopex | Solar panel unit operating cost | USD/kWp/y |
| RFcLosses | Losses of hydrogen form c during unloading and handling at refuelling station |  |
| RFcucapex | Refuelling station unit capital cost | USD |
| RFcUL | Maximum capacity of refuelling station receiving hydrogen in form c | kg/d |
| RFcuopex | Refuelling station unit operating cost | USD/y |
| SHj | Average daily sun hours in region j | h |
| SRj | Solar irradiance at region j | kWh/m2/d |
| Tj,k,cTrip | Time required to transport hydrogen form c from region j to region k using trucks | h |
| TA | Operation time of truck in a day | h |
| TCc | Capacity of truck carrying hydrogen in form c | kg |
| TruckFE | Fuel economy of truck | L/km |
| Truckcucapex | Truck unit capital cost | USD |
| Truckcuopex | Truck unit operating cost | USD/y |
| TS | Average speed of truck | km/h |
| YBAT | Lifespan of battery | y |
| YCOMP | Lifespan of compressor | y |
| YCONV | Lifespan of converter | y |
| YEL | Lifespan of electrolyzer | y |
| YFC | Lifespan of fuel cell | y |
| YRF | Lifespan of refuelling station | y |
| YTruck | Lifespan of truck | y |
| Integer Variables |  |  |
| Nj,kCable | Number of power cables transmitting electricity from region j to region k |  |
| Nj,c,tcComp | Number of compressor at capacity level tc in region j |  |
| Nj,thHyd | Number of hydrogenation equipment at capacity level th in region j |  |
| Nj,tlLiq | Number of liquefaction equipment at capacity level tl in region j |  |
| Nj,k,c,dpipe | Number of pipelines with diameter d transporting hydrogen in form c from region j to region k |  |
| Nj,cRF | Number of refuelling stations receiving hydrogen in form c in region j |  |
| Nj,k,cTrip | Number of trips required to transport hydrogen in form c from region j to region k using truck |  |
| Nj,k,cTruck | Number of trucks required to transport hydrogen in form c from region j to region k |  |
| Continuous Variables |  |  |
| BATacapex | Battery annualized capital cost | USD/y |
| BATarep | Battery annualized replacement cost | USD/y |
| BATjCap | Battery capacity in region j | kWh |
| BATopex | Battery annual operating cost | USD/y |
| Cableacapex | Power cable annualized capital cost | USD/y |
| Cableopex | Power cable annual operating cost | USD/y |
| Compacapex | Compressor annualized capital cost | USD/y |
| Comparep | Compressor annualized replacement cost | USD/y |
| Compopex | Compressor annual operating cost | USD/y |
| Compj,cpower | Compressor power requirement in region j | kW |
| Compj,c,tcrange | Throughput of compressor at capacity level tc in region j | kW |
| CONVacapex | Converter annualized capital cost | USD/y |
| CONVarep | Converter annualized replacement cost | USD/y |
| CONVjCap | Converter capacity in region j | kW |
| CONVopex | Converter annual operating cost | USD/y |
| EjImporttoH | Electricity imported to region j used for hydrogen demand | kWh/d |
| EjtoE | PV electricity produced in region j utilized in the form of electricity | kWh/d |
| Ej,ktoExport | PV electricity exported from region j to region k | kWh/d |
| EjtoH | PV electricity produced in region j used to synthesize hydrogen | kWh/d |
| EjS | Energy stored in battery | kWh |
| ELacapex | Electrolyzer annualized capital cost | USD/y |
| ELarep | Electrolyzer annualized replacement cost | USD/y |
| ELjCap | Electrolyzer capacity in region j | kW |
| ELopex | Electrolyzer annual operating cost | USD/y |
| FCacapex | Fuel cell annualized capital cost | USD/y |
| FCarep | Fuel cell annualized replacement cost | USD/y |
| FCjCap | Fuel cell capacity in region j | kW |
| FCopex | Fuel cell annual operating cost | USD/y |
| Hj,cFC | Hydrogen reacted in fuel cell in form c in region j | kg/d |
| Hj,k,cP | Hydrogen transported using pipeline in form c from region j to region k | kg/d |
| Hj,cPout,EIH | Hydrogen produced using electricity imported to region j converted into form c | kg/d |
| Hj,cPout,LC | Hydrogen produced using local electricity in region j converted into form c | kg/d |
| Hj,cRF | Hydrogen received at refuelling stations in form c in region j | kg/d |
| Hj,cS | Amount of hydrogen stored in form c in region j | kg/d |
| Hj,k,cT | Hydrogen transported using truck in form c from region j to region k | kg/d |
| HSacapex | Hydrogen storage annualized capital cost | USD/y |
| HSopex | Hydrogen storage annual operating cost | USD/y |
| Hydacapex | Hydrogenation annualized capital cost | USD/y |
| Hydopex | Hydrogenation annual operating cost | USD/y |
| Hydj,thrange | Throughput of hydrogenation equipment at capacity level th in region j | kg/d |
| Liqacapex | Liquefaction annualized capital cost | USD/y |
| Liqopex | Liquefaction annual operating cost | USD/y |
| Liqj,tlrange | Throughput of liquefaction equipment at capacity level tl in region j | kg/d |
| Pipeacapex | Pipeline annualized capital cost | USD/y |
| Pipeopex | Pipeline annual operating cost | USD/y |
| Pipej,k,c,drange | Throughput of pipe with diameter d transporting hydrogen in form c from region j to region k | kg/d |
| PVacapex | Solar panel annualized capital cost | USD/y |
| PVjCap | Capacity of solar panel at region j | kWp |
| PVopex | Solar panel annual operating cost | USD/y |
| RFacapex | Refuelling station annualized capital cost | USD/y |
| RFarep | Refuelling station annualized replacement cost | USD/y |
| RFopex | Refuelling station annual operating cost | USD/y |
| TCC | Annualized capital cost of supply chain | USD/y |
| TOC | Annual operating cost of supply chain | USD/y |
| TRC | Annualized replacement cost of supply chain | USD/y |
| Truckacapex | Truck annualized capital cost | USD/y |
| Truckarep | Truck annualized replacement cost | USD/y |
| TruckDW | Truck annual driver wage | USD/y |
| TruckFC | Truck annual fuel cost | USD/y |
| Truckopex | Truck annual operating cost | USD/y |

# Data Input

## Data Obtained from Spatial Analysis

Table 1: Processed spatial data for the study regions

| Region, j | Solar irradiation, SRj (kWh/m2/d) | Available area, AjFeasible (m2) | Hydrogen Demand, HjDemand (kg/d) | Electricity Demand, EjDemand (kWh/d) |
| --- | --- | --- | --- | --- |
| 1 | 4.6021 | 1,371,000,000 | 29,596 | 2,184,524 |
| 2 | 4.6483 | 567,000,000 | 10,680 | 788,298 |
| 3 | 4.6148 | 721,000,000 | 20,207 | 1,491,475 |
| 4 | 4.6611 | 895,000,000 | 44,792 | 3,306,114 |
| 5 | 4.5624 | 1,594,000,000 | 51,517 | 3,802,530 |
| 6 | 4.5870 | 1,626,000,000 | 75,029 | 5,537,967 |
| 7 | 4.4688 | 450,000,000 | 60,389 | 4,457,391 |
| 8 | 4.5505 | 1,673,000,000 | 28,289 | 2,088,041 |
| 9 | 4.5554 | 753,000,000 | 23,631 | 1,744,247 |
| 10 | 4.4489 | 472,000,000 | 297,203 | 21,936,760 |

Table 2: Transportation distance between the study regions

|  | Distance, Dj,k (km) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Region, j | Region, k |  |  |  |  |  |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1 | 0 | 205 | 52 | 79 | 109 | 96 | 145 | 189 | 149 | 171 |
| 2 | 205 | 0 | 205 | 198 | 96 | 149 | 130 | 88 | 174 | 120 |
| 3 | 52 | 205 | 0 | 27 | 109 | 77 | 145 | 189 | 150 | 171 |
| 4 | 79 | 198 | 27 | 0 | 102 | 52 | 139 | 183 | 143 | 165 |
| 5 | 109 | 96 | 109 | 102 | 0 | 53 | 70 | 114 | 78 | 100 |
| 6 | 96 | 149 | 77 | 52 | 53 | 0 | 89 | 133 | 93 | 115 |
| 7 | 145 | 130 | 145 | 139 | 70 | 89 | 0 | 44 | 54 | 31 |
| 8 | 189 | 88 | 189 | 183 | 114 | 133 | 44 | 0 | 88 | 35 |
| 9 | 149 | 174 | 150 | 143 | 78 | 93 | 54 | 88 | 0 | 53 |
| 10 | 171 | 120 | 171 | 165 | 100 | 115 | 31 | 35 | 53 | 0 |

## Data Obtained from Literature

Table 3: System information

| Parameter | Value |
| --- | --- |
| Network operating period (d/y) | 365 |
| System lifespan, YSL (y) | 20 |
| Interest rate, i (%) | 5 |

Table 4: Techno-economic parameters of solar panel, converter, electrolyzer, fuel cell and battery

| Parameter | Value | Unit | Ref |
| --- | --- | --- | --- |
| Peak load ratio, HPL | 10 |  |  |
| Days of autonomy, DAutonomy | 2 | d |  |
| Solar Panel |  |  |  |
| Solar panel efficiency, PVEff | 15.8 | % | (Maleki 2018) |
| Solar panel capital cost, PVucapex | 1,806 | USD/kWp | (Maleki 2018) |
| Solar panel operating cost, PVuopex | 54 | USD/kWp/y | (Maleki 2018) |
| Solar panel lifespan | 20 | y | (Maleki 2018) |
| Land-use factor, fLand | 4 |  | (Ong et al. 2013) |
| Converter |  |  |  |
| Converter efficiency, CONVEff | 95 | % | (Maleki 2018) |
| Converter capital cost, CONVucapex | 528 | USD/kW | (Maleki 2018) |
| Converter operating cost, CONVuopex | 5 | USD/kW/y | (Maleki 2018) |
| Converter lifespan, YCONV | 10 | y | (Maleki 2018) |
| Electrolyzer |  |  |  |
| Electrolyzer efficiency, ELEff | 65 | % | (Fonseca et al. 2020) |
| Electrolyzer capital cost, ELucapex | 1,920 | USD/kW | (Fonseca et al. 2020) |
| Electrolyzer operating cost, ELuopex | 38.4 | USD/kW/y | (Fonseca et al. 2020) |
| Electrolyzer lifespan, YEL | 8 | y | (Fonseca et al. 2020) |
| Fuel cell |  |  |  |
| Fuel cell efficiency, FCEff | 55 | % | (Fonseca et al. 2020) |
| Fuel cell capital cost, FCucapex | 3,240 | USD/kW | (Fonseca et al. 2020) |
| Fuel cell operating cost, FCuopex | 226.8 | USD/kW/y | (Fonseca et al. 2020) |
| Fuel cell lifespan, YFC | 8 | y | (Fonseca et al. 2020) |
| Hydrogen energy content, HE | 33.3 | kWh/kg |  |
| Battery |  |  |  |
| Battery capital cost, BATucapex | 147.62 | USD/kWh | (Maleki 2018) |
| Battery operating cost, BATuopex | 4.76 | USD/kWh/y | (Maleki 2018) |
| Battery lifespan, YBAT | 5 | y | (Maleki 2018) |
| Battery storage efficiency, BSEff | 85 | % | (Maleki 2018) |
| Maximum depth of discharge, BATDOD | 80 | % | (Maleki 2018) |

Table 5: Hydrogen storage parameters (Reuß et al. 2017)

| Form of hydrogen, c | Storage capital cost, HScucapex (USD/kg) | Storage capital cost, HScuopex (USD/kg/y) |
| --- | --- | --- |
| GH2 (250 bar) | 600 | 12 |
| LH2 | 30 | 0.6 |
| LOHC | 60 | 1.2 |
| GH2 (100 bar) | 600 | 12 |
| GH2 (30 bar) | 600 | 12 |

Table 6: Hydrogen conversion parameters

| Form of hydrogen, c | Conversion Technology | Processing loses, PRcLosses (%) a | Compression power requirement, Compcupower (kW/kg/h) |
| --- | --- | --- | --- |
| GH2 (250 bar) | Compression | 0.50 | 1.41 |
| LH2 | Liquefaction | 1.65 | - |
| LOHC | Hydrogenation | 3 | - |
| GH2 (100 bar) | Compression | 0.50 | 0.78 |
| GH2 (30 bar) | - | - | - |

a Source: (Reuß et al. 2017)

Table 7: Linearized cost parameters for hydrogen compression (Reuß et al. 2017)

| Capacity level, tc | Upper limit, Comptcrange,UL (kW) | Capital Cost, Comptcucapex (USD) | Operating Cost, Comptcuopex (USD/y) |
| --- | --- | --- | --- |
| tc1 | 100 | 217,393 | 8,696 |
| tc2 | 1,000 | 1,481,651 | 59,266 |
| tc3 | 5,000 | 5,666,793 | 226,672 |
| tc4 | 10,000 | 10,098,244 | 403,930 |
| tc5 | 20,000 | 17,995,104 | 719,804 |

Table 8: Linearized cost parameters for liquefaction process (Reuß et al. 2017)

| Capacity level, tl | Upper limit, Liqtlrange,UL (kg/d) | Capital Cost, Liqtlucapex (USD) | Operating Cost, Liqtluopex (USD/y) |
| --- | --- | --- | --- |
| tl1 | 1,000 | 9,529,066 | 762,325 |
| tl2 | 5,000 | 27,565,796 | 2,205,264 |
| tl3 | 10,000 | 43,556,236 | 3,484,499 |
| tl4 | 20,000 | 68,822,452 | 5,505,796 |
| tl5 | 50,000 | 126,000,000 | 10,080,000 |

Table 9: Linearized cost parameters for hydrogenation process (Reuß et al. 2017)

| Capacity level, th | Upper limit, Hydthrange,UL (kg/d) | Capital Cost, Hydthucapex (USD) | Operating Cost, Hydthuopex (USD/y) |
| --- | --- | --- | --- |
| th1 | 1,000 | 1,566,637 | 46,999 |
| th2 | 5,000 | 4,114,817 | 123,444 |
| th3 | 10,000 | 6,236,896 | 187,107 |
| th4 | 50,000 | 16,381,380 | 491,441 |
| th5 | 100,000 | 24,829,529 | 744,886 |
| th6 | 300,000 | 48,000,000 | 1,440,000 |

Table 10: Hydrogen truck transportation parameters

| Form of hydrogen, c | GH2 (250 bar) | LH2 | LOHC | GH2 (100 bar) | GH2 (30 bar) |
| --- | --- | --- | --- | --- | --- |
| Truck capacity, TCc (kg) a | 670 | 4,300 | 1,800 | - | - |
| Truck capital cost, Truckcucapex (USD) a | 660,000 | 1,032,000 | 180,000 | - | - |
| Truck O&M Cost, Truckcuopex (USD/y) a | 13,200 | 20,640 | 3,600 | - | - |
| Truck loading and unloading time, LTc (USD/y) a | 1.5 | 3 | 1.5 | - | - |
| Lifespan, YTruck (y) a | 12 |  |  | - | - |
| Fuel economy, TruckFE (L/km) a | 0.35 |  |  | - | - |
| Operation time in a day, TA (h) | 8 |  |  | - | - |
| Average speed, TS (km/h) | 60 |  |  | - | - |
| Driver hourly wage, DHW (USD/h) | 6 |  |  | - | - |

a Source: (Reuß et al. 2017)

Table 11: Pipeline parameters

| Pipe diameter, d (mm) | Maximum mass flowrate, Pipedrange,UL (kg/d) | Capital cost, Pipeducapex (USD/km) a | Operating cost, Pipeduopex (USD/km/y) a |
| --- | --- | --- | --- |
| 100 | 15,539 | 334,585 | 8,699 |
| 200 | 89,749 | 528,459 | 13,740 |
| 300 | 250,345 | 734,374 | 19,094 |
| 400 | 518,364 | 962,165 | 25,016 |
| 500 | 911,627 | 1,201,178 | 31,231 |
| 600 | 1,445,924 | 1,460,989 | 37,986 |
| 700 | 2,135,605 | 1,746,334 | 45,405 |
| 800 | 2,993,923 | 2,063,444 | 53,650 |
| 900 | 4,033,263 | 2,420,517 | 62,933 |
| 1000 | 5,265,298 | 2,828,339 | 73,537 |

a Source: Amgad et al. (2015)

Table 12: Refuelling station parameters (Reuß et al. 2017)

| Form of hydrogen, c | Design capacity, RFcUL (kg/d) | Capital Cost, RFcucapex (USD) | O&M Cost, RFcuopex (USD/y) | Losses, RFcLosses (%) | Lifespan, YRF |
| --- | --- | --- | --- | --- | --- |
| GH2 (250 bar) | 850 | 2,040,000 | 204,000 | 0.50 | 10 |
| LH2 | 850 | 2,040,000 | 204,000 | 3 |  |
| LOHC | 850 | 2,400,000 | 240,000 | 0.50 |  |
| GH2 (100 bar) | 850 | 2,400,000 | 240,000 | 0.50 |  |
| GH2 (30 bar) | - | - | - | - | - |

Table 13: Techno-economic parameters of electricity transmission

| Parameter | Value | Unit | Ref |
| --- | --- | --- | --- |
| Capital cost of cable, Cableucapex | 354,000 | USD/km | (Khandelwal and Pachori 2013) |
| Operating cost of cable, Cableuopex | 3,540 | USD/km/y | (Khandelwal and Pachori 2013) |
| Maximum capacity, CableUL | 114.84 | MW | (Khandelwal and Pachori 2013) |
| Transmission and distribution losses, Elosses | 6 | % | (IEA 2018) |

Table 14: Parameters related to conventional energy use

| Parameter | Value | Unit | Ref |
| --- | --- | --- | --- |
| Cost of electricity, Eleccost | 0.06 | USD/kWh | (GlobalPetrolPrices 2021) |
| Cost of petrol, Petrolcost | 0.50 | USD/L | (GlobalPetrolPrices 2021) |
| Cost of diesel, Dieselcost | 0.52 | USD/L | (GlobalPetrolPrices 2021) |
| Fuel economy of petrol car, PRVFE | 0.094 | L/km | (IEA 2020) |
| Fuel economy of fuel cell vehicle, FCVFE | 0.0076 | kg/km | (H2 Mobility 2021) |

Table 15: Binaries

| Form of hydrogen, c | BcCOMP | BcLIQ | BcHYD | BcTruck | BcPipe | BcFC | BcRF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GH2 (250 bar) | 1 | 0 | 0 | 1 | 0 | 1 | 1 |
| LH2 | 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| LOHC | 0 | 0 | 1 | 1 | 0 | 0 | 1 |
| GH2 (100 bar) | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| GH2 (30 bar) | 0 | 0 | 0 | 0 | 0 | 1 | 0 |

Table 16: Regional transportation binary

|  | Bj,kTrans |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Region, j | Region, k |  |  |  |  |  |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 3 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 4 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 5 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 6 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 7 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 9 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 10 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |

# Supply Chain Costing

## Solar PV

The capital and operating costs of solar panel are given by Eqs (1) and (2) respectively.

| PVacapex=jPVjCapPVucapexCRF | (1) |
| --- | --- |
| PVopex=jPVjCapPVuopex | (2) |

## Electrolyzer

There are two possible sources of electricity for electrolyzer, either the local PV electricity or electricity imported from other regions. Thus, the capacity of electrolyzer is dependent on both supply as shown in Eqs (3) and (4). The capital, operating, and replacement costs of electrolyzer can be computed using Eqs (5), (6) and (7) respectively.

| ELjCap≥EjtoHSHj ∀j | (3) |
| --- | --- |
| ELjCap≥EjImporttoHCONVEffHPL ∀j | (4) |
| ELacapex=jELjCapELucapexCRF | (5) |
| ELopex=jELjCapELuopex | (6) |
| ELarep=jELjCapELucapexii+1YEL-1 | (7) |

## Fuel Cell

The capacity of fuel cell can be determined from its maximum throughput as displayed in Eq (8). As fuel cell is only used in region that is not producing PV electricity, its maximum rate of operation is characterized by the peak load in a day, where HPL represents the ratio of daily load to peak hourly load The capital, operating, and replacement costs of fuel cell can be computed using Eqs (9), (10) and (11) respectively.

| FCjCap≥cHj,cFC HE HPL ∀j | (8) |
| --- | --- |
| FCacapex=jFCjCapFCucapexCRF | (9) |
| FCopex=jFCjCapFCuopex | (10) |
| FCarep=jFCjCapFCucapexii+1YFC-1 | (11) |

## Battery and Hydrogen Storage

The capacity of battery system is dependent on the electricity storage requirement and the battery depth of discharge, as shown in Eq (12). The capital, operating, and replacement costs of battery system are given by Eqs (13), (14) and (15). On the other hand, the capital and operating costs of hydrogen storage are given by Eqs (16) and (17).

| BATjCap=EjSBATDOD ∀j | (12) |
| --- | --- |
| BATacapex=jBATjCapBATucapexCRF | (13) |
| BATopex=jBATjCapBATuopex | (14) |
| BATarep=jBATjCap BATucapexii+1YBAT-1 | (15) |
| HSacapex=j,cHj,cS HScucapexCRF | (16) |
| HSopex=j,cHj,cS HScuopex | (17) |

## Converter

For the scope of HSC considered in this study, the conversion between AC and DC occurs three times: conversion of PV electricity into AC for electrical load/electricity export, conversion of fuel cell electricity into AC for electricity load, and the conversion of imported electricity from AC into DC for electrolyzer. Since a dual-converter is employed for this case, both AC/DC and DC/AC conversions can be done in the same device, and thus the capacity of the dual-converter is dependent on all the conversions as shown in Eqs (18) to (20). The capital, operating, and replacement costs of the dual-converter can be computed using Eqs (21), (22) and (23) respectively.

| CONVjCap≥EjtoESHj ∀j | (18) |
| --- | --- |
| CONVjCap≥FCjCapFCEff ∀j | (19) |
| CONVjCap≥ EjImporttoHHPL ∀j | (20) |
| CONVacapex=jCONVjCapCONVucapexCRF | (21) |
| CONVopex=jCONVjCapCONVuopex | (22) |
| CONVarep=jCONVjCapCONVucapexii+1YCONV-1 | (23) |

## Hydrogen Conversion Equipment

As hydrogen can be either produced from PV electricity or imported electricity, the capacity of hydrogen conversion equipment (compression, liquefaction, and hydrogenation) is dependent on both sources, similar to that of the electrolyzer. The compressor power requirement is determined using Eqs (24) and (25), where the binary BcCOMP defines the form of hydrogen that requires compression. As the cost of compression has been classified into several categories based on the capacity of compressor, Eq (26) defines that the sum of compressor power at all capacity levels should fulfil the compression power required. Eq (27) constrains the capacity of compressor to be within the limits. Eqs (28), (29), and (30) show the calculations for compressor capital, operating and replacement costs.

| Compj,cpower=Hj,cPout,LCBcCOMPCompcupowerSHj ∀j,c | (24) |
| --- | --- |
| Compj,cpower=Hj,cPout,EIHBcCOMPCompcupowerHPL ∀j,c | (25) |
| tcCompj,c,tcrange≥Compj,cpower ∀j,c | (26) |
| Compj,c,tcrange≤Comptcrange,ULNj,c,tcComp ∀j,c,tc | (27) |
| Compacapex=j,c,tcNj,c,tcCompComptcucapexCRF | (28) |
| Compopex=j,c,tcNj,c,tcComp Comptcuopex | (29) |
| Comparep=j,c,tcNj,c,tcCompComptcucapexii+1YCOMP-1 | (30) |

As the costing parameters for liquefaction and hydrogenation equipment are given on daily basis, they are sized based on daily throughput instead of the peak load. Eqs (31) and (32) define that the sum of liquefaction equipment at all capacity levels should be greater or equal to the liquefaction throughput. Eq (33) constrains the capacity of liquefaction equipment to be within the limits. Eqs (34) and (35) show the calculations for liquefaction capital and operating costs.

| tlLiqj,tlrange≥Hj,cPout,LCBcLIQ ∀j,c | (31) |
| --- | --- |
| tlLiqj,tlrange≥Hj,cPout,EIHBcLIQ ∀j,c | (32) |
| Liqj,tlrange≤Liqtlrange,ULNj,tlLiq ∀j,tl | (33) |
| Liqacapex=j,tlNj,tlLiq LiqtlucapexCRF | (34) |
| Liqopex=j,tlNj,tlLiq Liqtluopex | (35) |

Eqs (36) and (37) define that the sum of hydrogenation equipment at all capacity levels should be sufficient to handle the hydrogenation throughput. Eq (38) constrains the capacity of hydrogenation equipment to be within the limits. Eqs (39) and (40) show the calculations for hydrogenation capital and operating costs.

| thHydj,thrange≥Hj,cPout,LCBcHYD ∀j,c | (36) |
| --- | --- |
| thHydj,thrange≥Hj,cPout,EIHBcHYD ∀j,c | (37) |
| Hydj,thrange≤Hydthrange,ULNj,thHyd ∀j,th | (38) |
| Hydacapex=j,thNj,thHyd HydthucapexCRF | (39) |
| Hydopex=j,thNj,thHyd Hydthuopex | (40) |

## Hydrogen Transportation Costs

The number of trips required to transport hydrogen from one region to another is dependent on the truck capacity as displayed in Eq (41), and the time required for each trip can be estimated using Eq (42). Eq (43) is used to determine the number of trucks required for transportation.

| Nj,k,cTrip≥Hj,k,cTTCc ∀j,k,c | (41) |
| --- | --- |
| Tj,k,cTrip≥2×Dj,k TS+LTc ∀j,k,c | (42) |
| Nj,k,cTruck≥Nj,k,cTrip Tj,k,cTripTA ∀j,k,c | (43) |

Eq (44) can be used to compute the capital investment of truck. Considering the lifespan of truck, the annualized replacement cost can be calculated using Eq (45). On the other hand, the operating and maintenance cost of the truck can be determined via Eq (46), where the fuel cost and driver wage are computed using Eqs (47) and (48).

| Truckacapex=j,k,cNj,k,cTruck×Truckcucapex×CRF | (44) |
| --- | --- |
| Truckarep=j,k,cNj,k,cTruck×Truckcucapexii+1YTruck-1 | (45) |
| Truckopex=j,k,cNj,k,cTruck×Truckcuopex +TruckFC+TruckDW | (46) |
| TruckFC=j,k,c2×Nj,k,cTrip×Dj,k×TruckFE×Dieselcost ×365 | (47) |
| TruckDW=j,k,cNj,k,cTruck×TA ×365×DHW | (48) |

For hydrogen transportation via pipeline, Eq (49) defines that hydrogen can be transported in pipelines with different diameters, but the sum of hydrogen transported should be equal to the targeted amount. Meanwhile, Eq (50) constrains the mass flowrates in pipelines so that they do not exceed the maximum allowable limits. Eqs (51) and (52) show the calculations for pipeline capital and operating costs.

| dPipej,k,c,drange=Hj,k,cP ∀j,k,c | (49) |
| --- | --- |
| Pipej,k,c,drange≤Pipedrange,ULNj,k,c,dpipe ∀j,k,c,d | (50) |
| Pipeacapex=j,k,c,dNj,k,c,dpipe Dj,k PipeducapexCRF | (51) |
| Pipeopex=j,k,c,dNj,k,c,dpipe Dj,k Pipeduopex | (52) |

## Electricity Transmission Cost

Eq (53) constrains the electricity transmission in power cables to not exceed the maximum allowable limits. Eqs (54) and (55) show the calculations for power cable capital and operating costs.

| Ej,ktoExportHPL ≤Cable,ULNj,kCable ∀j,k | (53) |
| --- | --- |
| Cableacapex=j,kNj,kCable Dj,k CableucapexCRF | (54) |
| Cableopex=j,kNj,kCable Dj,k Cableuopex | (55) |

## Refuelling Station Costs

Eq (56) computes the number of refuelling stations required in each region. Eqs (57), (58) and (59) show the calculations for capital, operating and replacements cost of the refuelling stations.

| Nj,cRF≥Hj,cRFRFcUL ∀k,c | (56) |
| --- | --- |
| RFacapex=j,cNj,cRFRFcucapexCRF | (57) |
| RFopex=j,cNj,cRFRFuopex | (58) |
| RFarep=j,cNj,cRFRFucapexii+1YRF-1 | (59) |

## Total Cost

The annualized capital, operating and replacement costs of the entire supply chain can be computed using Eqs (60), (61) and (62) respectively.

| TCC=PVacapex+INVacapex+ELacapex+FCacapex+BATacapex+HSacapex+Compacapex+Liqacapex+Hydacapex+Truckacapex+Pipeacapex+Cableacapex+RFacapex | (60) |
| --- | --- |
| TOC=PVopex+INVopex+ELopex+FCopex+BATopex+HSopex+Compopex+Liqopex+Hydopex+Truckopex+Pipeopex+Cableopex+RFopex | (61) |
| TRC=CONVarep+ELarep+FCarep+BATarep+Comparep+Truckrep+RFarep | (62) |

# References

Amgad E, Reddi K, Mintz M, Brown D (2015) H2A Delivery Scenario Analysis Model Version 3.0 (HDSAM 3.0) User's Manual.

Fonseca JD, Commenge J-M, Camargo M, Falk L, Gil ID (2020) Multi-criteria optimization for the design and operation of distributed energy systems considering sustainability dimensions. Energy 214: 118989. https://doi.org/10.1016/j.energy.2020.118989.

GlobalPetrolPrices (2021) Gasoline and diesel prices by country.  https://www.globalpetrolprices.com/. Accessed 7 April 2021.

H2 Mobility (2021) Toyota MIRAI.  https://h2.live/en/wasserstoffautos/toyota-mirai-ii. Accessed 7 April 2021.

IEA (2018) Electric power transmission and distribution losses (% of output) - Malaysia.  https://data.worldbank.org/indicator/EG.ELC.LOSS.ZS?locations=MY. Accessed 4 April 2021.

IEA (2020) Fuel Consumption of Cars and Vans.  https://www.iea.org/reports/fuel-consumption-of-cars-and-vans. Accessed 15 April 2021.

Khandelwal P, Pachori A (2013) Variation of life cycle cost of overhead transmission line and underground transmission cable. International Journal of Enhanced Research in Science Technology & Engineering 2: 55-64.

Maleki A (2018) Design and optimization of autonomous solar-wind-reverse osmosis desalination systems coupling battery and hydrogen energy storage by an improved bee algorithm. Desalination 435: 221-234. https://doi.org/10.1016/j.desal.2017.05.034.

Ong S, Campbell C, Denholm P, Margolis R, Heath G (2013) Land-use requirements for solar power plants in the United States. National Renewable Energy Lab.(NREL), Golden, CO (United States).

Reuß M, Grube T, Robinius M, Preuster P, Wasserscheid P, Stolten D (2017) Seasonal storage and alternative carriers: A flexible hydrogen supply chain model. Applied energy 200: 290-302.