Table 5. Main statistical parameters of K estimated using different methodologies and evaluation of variations in K with depth in the shallowest 10 m of the aquifer (see Figure S14).

| Method |  | Na | Sampled depth (m) | Range (m/d) | Min. (m/d) | Max. (m/d) | G. mean (m/d) | Median (m/d) | StDb (m/d) | COVc (-) | Does K decrease with depth in the shallowest 10 m? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSA | Hazen | 20 | 0 - 9 | 15.93 | 11.27 | 27.20 | 20.14 | 21.98 | 4.59 | 0.23 | Yes (~2 m/d/m) |
|  | Kozeny | 20 | 0 - 9 | 11.77 | 8.32 | 20.09 | 14.88 | 16.24 | 3.39 | 0.23 | Yes (~1.5m/d/m) |
|  | Slitcher | 20 | 0 - 9 | 4.43 | 3.13 | 7.57 | 5.61 | 6.12 | 1.28 | 0.23 | Yes (~0.6 m/d/m) |
|  | Terzaghi | 20 | 0 - 9 | 7.57 | 5.35 | 12.93 | 9.57 | 10.45 | 2.18 | 0.23 | Yes (≈1 m/d/m) |
|  | Breyer | 20 | 0 - 9 | 21.91 | 14.68 | 36.59 | 26.85 | 29.23 | 6.28 | 0.23 | Yes (~2.8 m/d/m) |
|  | USBR | 20 | 0 - 9 | 4.69 | 4.08 | 8.78 | 6.42 | 6.91 | 1.50 | 0.23 | Yes (~0.6 m/d/m) |
|  | Alyamani & Sen | 20 | 0 - 9 | 25.08 | 14.40 | 39.48 | 28.28 | 30.58 | 7.07 | 0.25 | Yes (~3.1 m/d/m) |
| Cone penetration test |  | 12588 | 0 -18.5 | 403.01 | 0.01 | 403.01 | 23.07 | 16.10 | 24.30 | 1.05 | Yes (~3.8 m/d/m) |
| HPT | McCall & Christy | 2970 | 0 - 13 | 24.57 | 0.01 | 24.57 | 9.52 | 8.73 | 5.20 | 0.55 | Yes (~0.4 m/d/m) |
|  | Borden et al. | 2970 | 0 - 13 | 26.21 | 0.38 | 26.59 | 3.64 | 2.15 | 4.36 | 1.20 | Yes (~0.1 m/d/m) |
| Dilution test | Classical method | 6 | 0 - 8 | 17.52 | 5.12 | 22.64 | 11.05 | 10.61 | 6.06 | 0.55 | No |
|  | CXTFIT model | 6 | 0 - 8 | 19.65 | 10.13 | 29.77 | 17.12 | 16.75 | 7.18 | 0.42 | No |
| Slug test | Bouwer-Rice | 24 | 0 - 8 | 13.47 | 3.78 | 17.25 | 8.09 | 8.33 | 2.75 | 0.34 | No |
|  | Hvorslev | 24 | 0 - 8 | 12.53 | 4.72 | 17.25 | 9.42 | 9.64 | 2.78 | 0.30 | No |
|  | Dagan | 24 | 0 - 8 | 10.13 | 3.60 | 13.73 | 7.41 | 7.69 | 2.25 | 0.30 | No |
|  | KGS | 24 | 0 - 8 | 9.24 | 5.53 | 14.77 | 9.20 | 9.34 | 2.10 | 0.23 | No |
| Pumping test | Constant-rate | 12 | 0 - 8 | 2.07 | 4.89 | 6.96 | 5.95 | 5.92 | 0.69 | 0.12 | No |
|  | Variable-rate | 23 | 0 - 8 | 5.34 | 5.09 | 10.43 | 7.87 | 8.03 | 1.20 | 0.15 | No |
| Tide-Aquifer iteration technique | Time lag | 33 | 0 - 8 | 75.98 | 27.60 | 103.58 | 58.30 | 55.97 | 17.33 | 0.30 | No |
|  | Tidal efficiency | 33 | 0 - 8 | 14.18 | 3.26 | 17.44 | 5.73 | 5.24 | 2.91 | 0.51 | No |
|  | An. solution on sloping beaches | 33 | 0-8 | 9.98 | 5.66 | 15.64 | 7.85 | 7.73 | 4.79 | 0.61 | No |
| Numerical model |  | 15 | 0 - 18.5 | 15.00 | 5.00 | 20.00 | 7.95 | 7.75 | 4.79 | 0.60 | Yes (~0.6 m/d/m) |

a N: Number of estimates; b StD: standard deviation; c COV: Coefficient of variation