# Supplementary Information For: Where do they come from, where do they go? Emissions and fate of OPEs in global megacities

Timothy F. M. Rodgers1, Amanda Giang1*, Miriam L. Diamond2,3, Emma Gillies1, Amandeep Saini4*

1Institute for Resources, Environment and Sustainability, University of British Columbia, Vancouver Canada V6T 1Z4

2Department of Earth Sciences, University of Toronto, Toronto, Canada M5S 3B1

3School of the Environment, University of Toronto, Toronto, Canada M5S 3B1

4Air Quality Processes Research Section, Environment and Climate Change Canada, Toronto, Canada M3H5T4

# Model Evaluation

The surface water concentration predicted by the model, representing rivers in the modeled cities, was within the range of measured values reported in the literature, for 86% of predictions, and within a factor of 100 for 78% of predictions. Some of this model underestimation of water concentrations could be due to neglecting direct water emissions.1 In the soil, 68% of predictions were within the range of measured values, and 79% within a factor of 100. The model also appeared to under-predict measured soil concentration, although this may be because many of the soil measurements were taken from areas where higher concentrations would be expected such as on roadsides2,3 or near e-waste recycling facilities4 compared with the model considering “average” conditions.

Emissions fluxes for each city calculated with the different system boundaries were within a factor of 0.51-1.98 (Table S1). This is well within the expected ~order of magnitude uncertainty of the model,1 indicating that fate processes were similar across a city’s area, regardless of how the boundary was defined. The largest fluxes were estimated for the smallest (5km radius) boundary and the smallest for the larger 15 km buffer. The administrative areas, which tended to be larger than either, generally fell between the two buffer areas. Across all cities the average ∑10OPEs varied by an average of <10% between boundaries, driven by the low sensitivity of TCEP and TCPP, which had the largest emissions.

Table S1: Summary statistics showing the sensitivity of modeled emissions fluxes to the choice of model boundary definition.

| Boundaries | Min | Max | Mean | Median |
| --- | --- | --- | --- | --- |
| 5k/admin | 0.64 | 1.98 | 1.16 | 1.10 |
| 15k/admin | 0.51 | 1.38 | 0.95 | 0.99 |
| 5k/15k | 0.80 | 1.60 | 1.23 | 1.25 |

The 2018 ∑10OPEs estimate presented here for Toronto of 1,800 kg yr-1 was lower than the 3,300 kg yr-1 mean value presented in Rodgers et al.1 for 2010 using the same model and boundary area. This near halving of emissions was caused by lower measured air concentrations; using the 2010 air concentrations with the 2018 parameterization gave an estimate of ~3,100 kg yr-1. Emissions in 2010 and 2018 were dominated by TCEP which was designated as “harmful to human health” under the Canadian Chemicals Management Plan in 2013,5 although the formal restrictions introduced on its use were limited to products intended for children under the age of three, which would likely not be sufficient to cause such a decrease.

He et al.6 estimated OPE emissions across China using production data and activity-specific emissions factors. They estimated that in 2018 the province of Beijing emitted ~130,000 kg yr-1 or 7.3 µg m-2, vs our estimate of ~5,100 kg yr-1 or 3.8 µg m-2 for the Beijing municipal area. Thus, we estimated ~2x lower emissions on an area-normalized basis based on similar measured air concentrations, despite the fact that our boundary contained a larger proportion of the urban area likely responsible for emissions. Thus, the difference in emissions intensity was likely caused by different estimations of chemical fate within the modeled domain.

# Elemental Effects Sensitivity

We conducted a global “Elemental Effects” sensitivity analysis7 to elucidate the processes governing modeled emissions, represented by the emissions flux (kg m-2 yr-1), and chemical fate represented by the proportion of emissions advected from the system (%) in outdoor urban environments (Figure ED 4). As expected, modeled emissions flux was most sensitive to the target air concentration, while compound fate was not (Figure ED 4, left side). Both outputs were least sensitive to “generic” parameters, such as mass-transport coefficients that remained the same across cities and chemicals (Figure ED 4, right side).

Overall, the sensitivity analysis showed that chemical fate in the urban environment varied substantially across cities depending on interactions between the built environment, a chemical’s physicochemical properties, and the city’s climate. The strong influence of the target air concentration supports the contention that emissions of OPEs are driven by the intensity of local emissions and not “transport” factors, such as increases in vapor pressure at higher temperatures that would cause greater emissions.8,9 However, the measured air concentration is itself known to vary substantial in the course of a year;8,10 our results suggest that this is due to the sensitivity of the modeled emissions to chemical fate, with higher emissions necessary to achieve an observed air concentration in some urban environments vs others.

For the built environment, chemicals fate was most sensitive to the area of the city’s footprint, and to the area-factors (Aj/Acity, m² m-2) of vegetation and film within that footprint. For physicochemical properties, the most important parameter was the air-water partition coefficient (log KAW), followed by three solute descriptors used in calculating partition coefficients from ppLFERs (L, V and B). The hexadecane-air partition coefficient (L) and the molar volume (V) represent cavitation and other non-polar forces in partitioning systems, while the hydrogen bonding basicity (B) represents polar forces for OPEs (where the hydrogen-bonding acidity is generally negligible). This indicates that, as expected, differences between specific chemical fates in the same urban environment are influenced by the “three solubilities” of water, organics and air; and could be well-described by comparing the fate of PMT vs PBT compounds.

Finally, for climate, the most important parameters were the rain rate (RR); the rates of advection in the upper and lower air compartments (QLA and QUA), respectively, which are determined by the wind speed; the air temperature; and the height of the upper air compartment, which is modeled as the planetary boundary layer height.

Certain parameters had non-normal rank sensitivity distributions, indicating parameter interactions. For instance, the 5th most sensitive factor for emissions was the rain rate (RR), with the parameter’s rank sensitivity showing a bimodal distribution indicating that emissions of more soluble species or those that were entirely particle-bound were very sensitive to the rain rate, while more volatile, less soluble chemicals were less so. This highlights the need for a global sensitivity analysis for even relatively simple multimedia models.

The interaction between which parameters increased or decreased either emissions, total (air and water) advection, or both simultaneously, highlights the complex, interdependent nature of chemical fate in the urban environment.  Although an increased flow rate in the upper and lower air directly increased both emissions and advection, for many other parameters, higher values increased emissions while decreasing advection, or vice-versa. For instance, the area factor of vegetation (AFveg) had a large positive impact on emissions and a negative impact on total advection, indicating that larger emissions are driven by atmospheric deposition and subsequent transformation in the plant compartment. By contrast, increasing the area of the urban film (AFfilm) or soil (AFsoil) increased both emissions and advection. In this case, increasing the area of the film or soil compartment increased atmospheric deposition and subsequent washoff to and advection from the water compartment. These parameters have bimodal sensitivity rank distributions for the emissions flux, similarly to the rain rate, because volatile lower-solubility compounds were less susceptible to washoff.

# Additional Methods Information

## Compound Identifiers

Table S2: Identifiers for compounds used in this study.

| Compound Name | CAS Number | IUPAC Name | SMILES Identifier |
| --- | --- | --- | --- |
| TEP | 78-40-0 | triethyl phosphate | CCOP(=O)(OCC)OCC |
| TPrP | 513-08-6 | tripropyl phosphate | CCCOP(=O)(OCCC)OCCC |
| TnBP | 126-73-8 | tributyl phosphate | CCCCOP(=O)(OCCCC)OCCCC |
| TCEP | 115-96-8 | tris(2-chloroethyl)phosphate | C(CCl)OP(=O)(OCCCl)OCCCl |
| TCiPP | 13674-84-5 | tris(chloroisopropyl)phosphate | CC(CCl)OP(=O)(OC(C)CCl)OC(C)CCl |
| TDCiPP | 13674-87-8 | tris(1,3-dichloroisopropyl)phosphate | C(C(CCl)OP(=O)(OC(CCl)CCl)OC(CCl)CCl)Cl |
| TPhP | 115-86-6 | triphenyl phosphate | C1=CC=C(C=C1)OP(=O)(OC2=CC=CC=C2)OC3=CC=CC=C3 |
| TmCP | 563-04-2 | tris(3-methylphenyl) phosphate | CC1=CC(=CC=C1)OP(=O)(OC2=CC=CC(=C2)C)OC3=CC=CC(=C3)C |
| TBOEP | 78-51-3 | tris(2-butoxyethyl)phosphate | CCCCOCCOP(=O)(OCCOCCCC)OCCOCCCC |
| TEHP | 78-42-2 | tris(2-ethylhexyl)phosphate | CCCCC(CC)COP(=O)(OCC(CC)CCCC)OCC(CC)CCCC |

## Impervious Surface Index

Figure S1: Relationship between the impervious surfaces index and population density in the built-up area (as described in the Methods). The brackets indicate the model boundary parameterization represented in each point.

# Additional Results

## S4.1 Emissions Regressions

We also found that emissions were mostly controlled by the magnitude of local sources rather than by transport limitations or environmental factors. Correlations Figure ED SI Table S3) between the estimated emissions and the factors that our sensitivity analysis indicated were controls of contaminant fate (Figure S1, SI Section S2) were generally not significant. The main exceptions were the relative humidity, (p<0.05 for TBOEP, TCIPP, TmCP, and ∑10OPEs; r² from 0.17 – 0.30) and the precipitation rate (p<0.05 for TBOEP and TmCP; r² = 0.20 and 0.23, respectively) of the cities. Mechanically, increases in these variables wash contaminants out of the air, lowering the fugacity in the air compartment, which could lead to more emissions due to the larger fugacity gradient between the air and any products containing OPEs. We note that this effect would likely be small, and so this correlation may not imply causation.

Table S3: Regressions between emissions proxies (∑GDP, Gross Domestic Product (GDP) at 2015 purchasing power parity, GDP per capita at 2015 purchasing power parity, ∑CO2 Emissions, anthropogenic emissions of greenhouse gases as CO2 equivalents, and the proportion of anthropogenic CO2 equivalent emissions from the Solvents and Products, Energy for Buildings, Solid Waste Incineration, Non-Metallic Minerals Production, and Combustion for Manufacturing sectors, as described by EDGAR31), and transport controls (relative humidity %, precipitation rate mm/hr, area of vegetation m²) vs the log10 transformed emissions flux (kg m-2 yr-1) for the specified OPE.

| Regression | Independent Variable | m1 | b | p_m1 | p_b | r² | adjr² | p_reg |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ∑OPEs | ∑GDP | 2.46E-12 | -5.98 | 0.00 | 1.94E-17 | 0.40 | 0.36 | 0.00 |  |
| TCIPP | %CO2 Solvents and Products | 3.06E+01 | -6.29 | 0.00 | 7.00E-18 | 0.38 | 0.35 | 0.00 |  |
| ∑OPEs | GDP per Capita | 2.00E-05 | -5.91 | 0.01 | 1.73E-17 | 0.35 | 0.31 | 0.01 |  |
| TmCP | ∑GDP | 2.84E-12 | -8.04 | 0.01 | 8.80E-18 | 0.35 | 0.31 | 0.01 |  |
| TnBP | ∑GDP | 1.68E-12 | -7.80 | 0.01 | 2.61E-21 | 0.34 | 0.30 | 0.01 |  |
| TCIPP | %CO2 Energy for Buildings | 2.89E+00 | -6.50 | 0.01 | 2.23E-15 | 0.32 | 0.28 | 0.01 |  |
| TBOEP | RH | 2.97E-02 | -9.01 | 0.01 | 2.44E-09 | 0.36 | 0.32 | 0.01 |
| TCEP | GDP per Capita | 1.90E-05 | -6.42 | 0.02 | 1.32E-17 | 0.30 | 0.26 | 0.02 |  |
| TCEP | ∑GDP | 2.19E-12 | -6.45 | 0.02 | 3.31E-17 | 0.30 | 0.26 | 0.02 |  |
| ∑OPEs | RH | 2.08E-02 | -6.97 | 0.02 | 5.16E-10 | 0.30 | 0.25 | 0.02 |  |
| TmCP | GDP per Capita | 2.22E-05 | -7.95 | 0.02 | 9.05E-18 | 0.28 | 0.24 | 0.02 |  |
| TCIPP | RH | 2.35E-02 | -7.54 | 0.02 | 1.97E-09 | 0.28 | 0.23 | 0.02 |  |
| TCIPP | ∑GDP | 2.37E-12 | -6.34 | 0.02 | 4.57E-16 | 0.27 | 0.23 | 0.02 |  |
| TmCP | Precipitation Rate | 3.72E+00 | -7.96 | 0.02 | 1.91E-17 | 0.27 | 0.23 | 0.02 |  |
| ∑OPEs | %CO2 Energy for Buildings | 2.21E+00 | -5.97 | 0.03 | 1.57E-15 | 0.26 | 0.21 | 0.03 |  |
| TBOEP | GDP per Capita | 2.35E-05 | -7.46 | 0.03 | 9.88E-15 | 0.28 | 0.23 | 0.03 |  |
| TPrP | Area of Vegetation | -1.10E-10 | -7.41 | 0.03 | 3.42E-22 | 0.25 | 0.21 | 0.03 |  |
| TEHP | %CO2 Solid Waste Incineration | -1.41E+02 | -8.40 | 0.03 | 3.57E-17 | 0.29 | 0.24 | 0.03 |  |
| TCIPP | ∑CO2 Emissions | -1.68E+00 | -5.82 | 0.03 | 2.39E-19 | 0.25 | 0.20 | 0.03 |  |
| TCEP | ∑CO2 Emissions | 7.49E+04 | -6.38 | 0.03 | 2.94E-17 | 0.24 | 0.19 | 0.03 |  |
| TDCIPP | ∑CO2 Emissions | 9.46E+04 | -7.81 | 0.03 | 2.35E-16 | 0.25 | 0.21 | 0.03 |  |
| TnBP | ∑CO2 Emissions | 5.29E+04 | -7.72 | 0.04 | 4.30E-21 | 0.23 | 0.19 | 0.04 |  |
| TCIPP | GDP per Capita | 1.88E-05 | -6.27 | 0.04 | 3.48E-16 | 0.23 | 0.18 | 0.04 |  |
| ∑OPEs | ∑GDP | 2.02E+01 | -5.78 | 0.04 | 1.53E-17 | 0.23 | 0.18 | 0.04 |  |
| TDCIPP | ∑CO2 Emissions | 2.40E-12 | -7.84 | 0.04 | 6.71E-16 | 0.24 | 0.19 | 0.04 |  |
| ∑OPEs | ∑CO2 Emissions | 7.01E+04 | -5.83 | 0.04 | 9.76E-17 | 0.22 | 0.18 | 0.04 |  |
| TCIPP | %CO2 Non-Metallic Minerals Production | -6.28E+00 | -5.53 | 0.04 | 2.74E-15 | 0.22 | 0.17 | 0.04 |  |
| TmCP | RH | 2.21E-02 | -9.05 | 0.04 | 5.21E-10 | 0.22 | 0.17 | 0.04 |  |
| TmCP | ∑CO2 Emissions | 8.56E+04 | -7.89 | 0.04 | 2.14E-17 | 0.22 | 0.17 | 0.04 |  |
| TEP | %CO2 Combustion for Manufacturing | -2.22E+00 | -7.23 | 0.05 | 1.98E-16 | 0.22 | 0.17 | 0.05 |  |
| TPhP | ∑GDP | 1.95E-12 | -7.38 | 0.05 | 2.44E-17 | 0.21 | 0.16 | 0.05 |  |

Figure S2: TBOEP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding. Blanks show cities where no TBOEP was emitted.

Figure S3: TCEP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding.

Figure S4: TCIPP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding.

Figure S5: TDCIPP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding. Blanks show cities where no TDCIPP was emitted.

Figure S6: TEHP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding. Blanks show cities where no TEHP was emitted.

Figure S7: TEP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding.

Figure S8: TmCP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding.

Figure S9: TnBP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding.

Figure S10: TPhP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding.

Figure S11: TPrP emissions and fate for each of the modeled cities using the 2018 annual average administrative area baseline model fate parameterization. Advective processes leaving the city’s modeled boundary are shown with one-way transport arrows at the right-hand side of the figure for the upper air, lower air, soil (to groundwater), water, and sediment (burial) compartments. Figures may not sum exactly to 100 due to rounding. Blanks show cities where no TEHP was emitted.