Appendix A

Table A1. Internal heat gains generated by occupants (Presented by Metabolic rate by Person [MR/P]).

|  | Weekdays |  |  |  | Weekend |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zones | Schedule | Nb of occupants | MR/P |  | Schedule | Nb of occupants | MR / P |
| Zone 1 Living & Dining room | 07h – 08h 13h –14h 19h – 20h 12h – 13h 18h –19h 08h – 11h 14h –17h 19h – 22h | 4 4 4 3 3 1 1 1 | 170 W 170 W 170 W 100 W 100 W 100 W 100 W 100 W |  | 09h – 10h 13h – 14h 20 h – 23h 15h – 18h | 4 4 4 1 | 170 W 170 W 170 W 100 W |
| Zone 2 Bedroom 1 | 22h – 23h 23h – 07h | 2 2 | 100 W 83W |  | 08h – 09h 14h – 17h 23h – 08h | 2 1 2 | 100 W 100 W 83 W |
| Zone 3 Office | 20h – 22h | 1 | 150 W |  | 10h – 13h | 1 | 150 W |
| Zone 4 Bedroom 2 | 20h – 22h 22h – 23h 23h – 07h | 2 2 2 | 150 W 100 W 83 W |  | 10h –13h 14h – 17h 23h – 09h | 2 2 2 | 150 W 100 W 83 W |
| Zone 6 Kitchen | 11h – 13h 17h – 19h | 1 1 | 209 W 209 W |  | 08h – 09h 10h – 13h | 1 1 | 209 W 209 W |

Table A2. Electrical equipment operating conditions

|  | Weekdays |  |  |  | Weekend |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zones | Schedule | Type of gains | Power |  | Schedule | Type of gains | Power |
| Zone 1 Living & Dining room | 12h – 13h 18h – 22h 18h – 22h | TV TV Light | 120 W 120 W 5 W/m² |  | 13h – 14h 20h – 23h 18h - 23h | TV TV Light | 120 W 120 W 5 W/m² |
| Zone 2 Bedroom 1 | 22h – 23h | Light | 5 W/m² |  |  |  |  |
| Zone 3 Office | 20h – 22h 20h – 22h | Light PC | 5 W/m² 200 W |  | 10h –12h | PC | 200 W |
| Zone 4 Bedroom 2 | 20h – 22h 20h – 22h | Light PC | 5 W/m² 200 W |  | 10h – 13h | PC | 200 W |
| Zone 6 Kitchen | 00h – 00h 10h – 11h 12h – 13h 18h – 19h 18h – 19h | Refrigerator Washing Kitchen Kitchen Light | 100 W 2000 W 800 W 800 W 5 W/m² |  | 00h – 00h 10h – 11h 11h – 13h | Refrigerator Washing Kitchen | 100 W 2000 W 800 W |

Appendix B

Table 1B. The scenarios components of exterior wall and roof.

| Building component | Scenario | Description | Components and Thickness ( cm) | U-value (W / m². °C) |
| --- | --- | --- | --- | --- |
|  | WALL 1 | Medium massive /exterior isolation | Solid brick (3 cm) Moroccan cork (5 cm) Stone (15 cm) Gypsum plaster (1,3 cm) | 0,63 |
|  | WALL 2 | Medium massive / interior isolation | Solid brick (3 cm) Stone (15 cm) Moroccan cork (5 cm) Gypsumplaster (1,3 cm) | 0,63 |
| Wall | WALL 3 | High massive/exterior insulation | Solid brick (3 cm) Moroccan cork (5 cm) Stone (30 cm) Gypsumplaster (1,3 cm) | 0,60 |
|  | WALL 4 | High massive/interior isolation | Solid brick (3 cm) Stone (30 cm) Moroccan cork (5 cm) Gypsumplaster (1,3 cm) | 0,60 |
|  | ROOF 1 | Medium massive /exterior isolation | Ceramic (2 cm) Mortar (2 cm) Moroccan cork (5 cm) Solid slab (15 cm) Gypsumplaster (1,3 cm) | 0,63 |
| Roof | ROOF 2 | Medium massive /interior isolation | Ceramic (2 cm) Mortar (2 cm) Solid slab (15 cm) Moroccan cork (5 cm) Gypsumplaster (1,3 cm) | 0,63 |
|  | ROOF 3 | High massive/ exterior insulation | Ceramic (2 cm) Mortar (2 cm) Moroccan cork (5 cm) Solid slab (30 cm) Gypsumplaster (1,3 cm) | 0,61 |
|  | ROOF 4 | High massive/ interior insulation | Ceramic (2 cm) Mortar (2 cm) Solid slab (30 cm) Moroccan cork (5 cm) Gypsumplaster (1,3 cm) | 0,61 |

Table 2B. Thermo-physical properties of glazing types (Ameur et al., 2020).

| Type of glazing | Thermal transmittance coefficient (W/m2.°C) | Solar transmittance coefficient | Visible transmittance coefficient |
| --- | --- | --- | --- |
| Simple glazing | 5.75 | 0.87 | 0. 90 |
| Double glazing | 2.95 | 0.77 | 0.81 |
| Triple glazing | 2.00 | 0.70 | 0.74 |