Supplementary Information for

Technology enabled science: investigation of a fin-shaped rock by the Yutu-2 rover on the lunar farside

Liang Ding1*†, Ruyi Zhou1†, Tianyi Yu2†, Huaiguang Yang1, Ximing He2, Haibo Gao1*, Juntao Wang3,4, Ye Yuan1, Jia Wang2, Zhengyin Wang1, Huanan Qi1, Jian Li2*, Wenhao Feng1, Xin Li2, Chuankai Liu2,5, Shaojin Han2, Xiajia Zeng3,4, Yu-Yan Sara Zhao6,4, Guangjun Liu7, Wenhui Wan8, Yuedong Zhang2, Saijin Wang2, Lichun Li2, Zongquan Deng1, Jianzhong Liu3,4*, Guolin Hu2, Rui Zhao2, Kuan Zhang2

1State Key Laboratory of Robotics and System, Harbin Institute of Technology; Harbin 150080, China.

2Beijing Aerospace Control Center; Beijing 100094, China.

3Center for Lunar and Planetary Sciences, Institute of Geochemistry, Chinese Academy of Science; Guiyang 550081, China.

4CAS Center for Excellence in Comparative Planetology; Hefei 230026, China.

5Key Laboratory of Science and Technology on Aerospace Flight Dynamics; Beijing 100049, China.

6International Center for Planetary Science, College of Earth Science, Chengdu University of Technology; Chengdu 610059, China.

7Department of Aerospace Engineering, Ryerson University; Toronto, ON M5B 2K3, Canada.

8State Key Laboratory of Remote Sensing Science, Aerospace Information Research Institute, Chinese Academy of Sciences; Beijing 100101, China.

*Corresponding author. Email: liangding@hit.edu.cn (L.D.), gaohaibo@hit.edu.cn (H.G.), lisirjian@163.com (Jian Li) and liujianzhong@mail.gyig.ac.cn (Jianzhong Liu).

†These authors contributed equally to this work

This PDF file includes:

Section S1

Supplementary Figure 1 to 2

Supplementary Table 1 to 8

Other Supplementary Information for this manuscript includes the following:

Source Data of Fig. 4 (Excel format): The 1 and 2 μm band center positions of rocks investigated in the CE-4 mission with pyroxenes measured by Adams (1974)

Source Data of Extended Data Fig. 8 (Excel format): Spectra data of the six rocks investigated in the CE-4 mission

S1. Nomenclature

| At, Bt, Ct | normal vector of the wheel-terrain contact plane |
| --- | --- |
| A1, A2, A3 | node of the triangular terrain mesh |
| b | wheel width |
| c | cohesion of the soil |
| c1, c2 | coefficients of the wheel-terrain interaction angle |
| c3 | coefficient of the exit angle |
| fDP | resistance force |
| FN | normal force |
| FDP | drawbar pull |
| FL | lateral force |
| Fu | force beneath the wheel due to shear motion |
| Fs | resistance force acting on the side face of the wheel |
| Fe | vector of equivalent forces in the initial frame |
| eFe | vector of external forces acting on the wheel center |
| h | grouser height |
| j | shearing deformation of the soil |
| jx | longitudinal shearing deformation of the soil |
| jy | lateral shearing deformation of the soil |
| kc | cohesive modulus of the soil |
| kφ | frictional modulus of the soil |
| Ks | equivalent stiffness modulus of the soil |
| Kx | longitudinal shearing deformation modulus of the soil |
| Ky | lateral shearing deformation modulus of the soil |
| MR | driving resistance torque |
| MS | steering resistance torque |
| MO | overturning torque |
| Me | vector of equivalent torques in the initial frame |
| eMe | vector of external torques acting on the wheel center |
| n0 | static sinkage exponent |
| n1 | dynamic sinkage exponent |
| N | variable sinkage exponent of the wheel-terrain interaction |
| pc | position of the wheel center |
| P1, P2, P3, Pi | coordinate point |
| r | wheel radius |
| rs | equivalent shearing radius of a grouser wheel |
| Re | rotation matrix of the wheel-terrain interaction frame relative to the initial frame |
| s | slip ratio |
| T | driving torque |
| Tc | homogeneous transform matrix of the wheel-terrain interaction frame relative to the initial frame |
| v | forward linear velocity |
| vx | longitudinal travelling velocity |
| vy | lateral travelling velocity |
| W | wheel load |
| z | wheel sinkage |
| β | side slip angle |
| θ1 | entrance angle |
| θ2 | exit angle |
| θi | solar incidence angle |
| θm | maximum stress angle |
|  | equivalent entrance angle of a grouser wheel |
| λ | wavelength |
| λs | lug shearing coefficient |
| σ | normal stress |
| σm | maximum normal stress |
| τ | shear stress |
| τx | longitudinal shear stress |
| τxm | maximum longitudinal shear stress |
| τy | lateral shear stress |
| τym | maximum lateral shear stress |
| φ | internal friction angle of the soil |
| φy | external friction angle of the soil |
| ω | angular velocity |
| ΣI | inertial frame |
| Σe | wheel-terrain interaction frame |
| Σc | wheel center frame |

Supplementary Fig. 1. Simulation results of the outboard traverse conditioned on φ=25°, Ky=40 mm. Data is collected at 10 Hz. a, Terrain elevation of the outboard traverse. b, Terrain slope of the outboard traverse. c, Slip ratio of six wheels during the outboard traverse. The slip ratio data is further smoothed with 20 points using the Adjacent-Averaging filter. d, Side slip angle of six wheels during the outboard traverse. The side slip angle data is further smoothed with 20 points using the Adjacent-Averaging filter.

Supplementary Fig. 2. Regolith parameter estimation. Parameters in gray box are input of the framework, and they are fixed parameters set according to wheel parameters or obtained by the dynamic solution or external measurements. Equations in the orange boxes are key equations for parameter identification. Parameters in blue box are the output of the framework and are specific to terrain bearing, shearing and lateral parameters.

Supplementary Table 1. The planned path and associated parameters.

| Phase |  | Movement type | Coordinate of the start point (m) | Rover heading direction at the start point | Coordinate of the end point (m) | Rover heading direction at the end point | Curvature (m-1) | Curve length (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Outbound journey | 1 | Ture in place | (-1.442, -17.871) | -90.718° | (-1.442, -17.871) | 156.436° | - |  |
|  | 1-2 | Curve | (-1.442, -17.871) |  | (-5.036, -17.404) |  | 0.154 | 3.674 |
|  | 2-3 | Curve | (-5.036, -17.404) |  | (-9.227, -16.862) |  | -0.132 | 4.282 |
|  | 3-4 | Curve | (-9.227, -16.862) |  | (-10.812, -15.914) |  | -0.138 | 1.852 |
|  | 4 | Ture in place | (-10.812, -15.914) | 141.796° | (-10.812, -15.914) | -136.810° | - |  |
|  | 4-5 | Straight line | Move forward straightly |  |  |  |  |  |
| Return journey | 5 | Turn in place | (-10.847, -15.939) | -139.483° | (-10.847, -15.939) | -40.366° | - | - |
|  | 5-6 | Curve | (-10.847, -15.939) |  | (-5.741, -17.655) |  | 0.138 | 5.518 |
|  | 6-7 | Curve | (-5.741, -17.655) |  | (-1.504, -18.168) |  | -0.084 | 4.292 |
|  | 7 | Turn in place | (-1.504, -18.168) | -12.735° | (-1.5035, -18.168) | 165.000° | - | - |

Note: A positive curvature represents that the rover is moving and turning right, while a negative curvature represents a left turn of the rover. In the 1-2, 2-3, 3-4 sections, the rover moves according to settled constructions, while in the 4-5 section, it’s movement to approach the rock target for investigation is pendent.

Supplementary Table 2. Setting of the terrain mechanical parameters in the simulation.

| Phase | Number of groups | Ks(kPa/mN) | N | c(Pa) | φ(°) | Kx(mm) | φy(°) | Ky(mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Outboard journey | 7 | 827 | 1.0 | 520 | 25-55 | 17.8 | 0.3×φ | 40 |
|  | 7 | 827 | 1.0 | 520 | 40 | 17.8 | 0.3×φ | 15-45 |
| Return journey | 1(unrectified) | 827 | 1.0 | 520 | 42 | 17.8 | 12.6 | 20 |
|  | 1(rectified) | 827 | 0.9 | 520 | 38.95 | 17.8 | 12.6 | 40 |

Supplementary Table 3. Parameters of terrestrial soil samples.

| Terrain | n | kc(kPa/mN-1) | kφ(kPa/mN) | Ks(kPa/mN) | c(kPa) | φ(°) |
| --- | --- | --- | --- | --- | --- | --- |
| Dry sand (Land Locomotion Lab., LLL) | 1.1 | 0.99 | 1528.43 | 1535.03 | 1.04 | 28 |
| Sandy loam (LLL) | 0.7 | 5.27 | 1515.04 | 1550.17 | 1.72 | 29 |
| Sandy loam Michigan (Strong, Buchele) | 0.9 | 52.53 | 1127.97 | 1478.17 | 4.83 | 20 |
| Sandy loam (Hanamoto) | 0.3 | 2.79 | 141.11 | 159.71 | 13.79 | 22 |
| Clayey soil (Thailand) | 0.5 | 13.19 | 692.15 | 780.08 | 4.14 | 13 |
| Lean clay (WES) | 0.2 | 16.43 | 1724.69 | 1834.22 | 68.95 | 20 |
| LETE sand (Wong) | 0.79 | 102 | 5301 | 5981 | 1.3 | 31.1 |
| Upland sandy loam (Wong) | 1.1 | 74.6 | 2080 | 2577 | 3.3 | 33.7 |
| Rubicon sandy loam (Wong) | 0.66 | 6.9 | 752 | 798 | 3.7 | 29.8 |
| North Gower clayey loam (Wong) | 0.73 | 41.6 | 2471 | 2748 | 6.1 | 26.6 |
| Grenville loam (Wong) | 1.01 | 0.06 | 5880 | 5880 | 3.1 | 29.8 |
| Snow (U.S.) | 1.6 | 4.37 | 196.72 | 225.85 | 1.03 | 19.7 |
| Snow (Sweden) | 1.44 | 10.55 | 66.08 | 136.41 | 6 | 20.7 |

Note: n, kc, kφ, c, φ are original data from reference47, while the values of Ks for terrains are derived based on kc and kφ when the wheel width is 0.15 m.

Supplementary Table 4. Parameters of lunar soil obtained in other lunar exploration missions27.

| Mission (year) | Landing site | Identification method | c(kPa) | φ(°) |
| --- | --- | --- | --- | --- |
| Apollo 11 (1969) | Mare Tranquilliatis | Penetrometer | 0.25-0.85 | 38-42 |
| Apollo 12 (1969) | Ocean of Storms | Vacuum Direct Shear | 0-0.7 | 28-35 |
|  |  | Direct Shear (Surveyor 3) | 0.1-3.1 | 13-56 |
|  |  | Triaxial Shear (Surveyor 3) | 0-1 | 51-59 |
| Luna 16 (1970) | Mare Fecunditatis | Direct Shear and Coulomb Device | 3.9-5.9 | 20-25 |
| Luna 20 (1972) | Apollonius Hightlands |  |  |  |
| Lunokhod 2 (1973) | Le Monnier Crater | Estimated | 0.4 | 40 |

Supplementary Table 5. The parameters of the Yutu-2 lunar rover.

| Group | Parameter name | Value |
| --- | --- | --- |
| Rover | Mass m (kg) | 135 |
|  | Size l×w×h (m) | 1.5×1.1×1.0 |
|  | Number of wheels | 6 |
|  | Maximum driving speed vmax (m/h) | 200 |
|  | Maximum climbable slope (°) | 20 |
|  | Climbable obstacle (mm) | 200 |
|  | Track (mm) | 950 |
|  | Wheelbase (mm) | 600 |
|  | Steering offset (mm) | 106 |
| Wheel | Wheel radius r (m) | 0.15 |
|  | Wheel width b (m) | 0.15 |
|  | Height of the wheel grouser h (m) | 0.01 |
|  | Number of wheel grousers | 24 |
|  | Equivalent shearing radius of a wheel rs (m) | 0.1565 |

Note: Six wheels on the Yutu-2 can drive, and only four corner wheels have steering motors. Track is the distance between the left and right wheels. Wheelbase is the distance between the centers of the front wheels and the middle wheels, or the distance between the centers of the middle wheels and the rear wheels. The distance between the centers of the front and middle wheels is the same as the that between the centers of the middle wheels and the rear wheels for Yutu-2 rover. The 24 wheel grousers are arranged in two staggered rows, and the grousers in each row are evenly arranged on the wheel’s outer cylinder surface.

Supplementary Table 6. Major specifications of the Pancam48, Navcam and Hazcam.

| Camera | Parameter | Value |
| --- | --- | --- |
| Pancam (stereo; two modes) | Baseline | 270 mm |
|  | Waveband | red: 640 nm, green: 540 nm, blue: 470 nm @ color mode |
|  |  | 420-700 nm @ panchromatic mode |
|  | Field of view | 19.6°×14.5° |
|  | Image resolution | 2,352×1,728 pixels @ color mode |
|  |  | 1,176×864 pixels @ panchromatic mode |
|  | Resolving power | 0.15 mrad (about 120 pixels/degree) |
| Navcam (stereo) | Baseline | 270 mm |
|  | Focal length | 17.7 mm |
|  | Field of view | 46.6° |
|  | Image resolution | 1024×1024 pixels |
| Hazcam | Field of view | 170° |
| (stereo; fisheye camera) | Image resolution | 1024×1024 pixels |

Note: The Pancam can acquire a 360° panoramic view by taking 56 pairs of images (28 different azimuth angles at 2 elevation angles).

Supplementary Table 7. Major specifications of the visible and near-infrared imaging spectrometer (VNIS)49.

| Description | Specification |  |
| --- | --- | --- |
|  | VIS/NIR | SWIR |
| Spectral range (nm) | 450-950 | 900-2400 |
| Spectral resolution (nm) | 2.4-6.5 | 3.6-9.6 |
| Field of view (°) | 8.5×8.5 | Φ3.58 |
| Bit resolution | 256×256 | 1 |
| Signal to noise ratio (dB) | >33 | >31 |
| Power consumption (W) | 16.95 |  |
| Weight (kg) | 4.65 (probe part) |  |
|  | 0.7 (electronics part) |  |

Supplementary Table 8. Kinematics parameters of Yutu-2 rover.

| Coordinate | Actuated | X axis (m) | Y axis (m) | Z axis (m) | θx | θy | θz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| {∑0}→{∑1} |  | 0 | D9 | -D1 | 0 | θ1 | 0 |
| {∑0}→{∑2} |  | 0 | -D9 | -D1 | 0 | θ2 | 0 |
| {∑1}→{∑3} |  | -D6 | 0 | -D2 | 0 | θ3 | 0 |
| {∑2}→{∑4} |  | -D6 | 0 | -D2 | 0 | θ4 | 0 |
| {∑1}→{∑5} |  | D8 | 0 | -D2 | 0 | 0 | θ5 |
| {∑2}→{∑6} |  | D8 | 0 | -D2 | 0 | 0 | θ6 |
| {∑1}→{∑7} |  | -D7 | 0 | -D2 | 0 | 0 | θ7 |
| {∑2}→{∑8} |  | -D7 | 0 | -D2 | 0 | 0 | θ8 |
| {∑5}→{∑w1} |  | 0 | D4 | -D3+D2 | 0 | θ9 | 0 |
| {∑6}→{∑w2} |  | 0 | -D4 | -D3+D2 | 0 | θ10 | 0 |
| {∑7}→{∑w3} |  | 0 | D4 | -D3+D2 | 0 | θ11 | 0 |
| {∑8}→{∑w4} |  | 0 | -D4 | -D3+D2 | 0 | θ12 | 0 |
| {∑3}→{∑w5} |  | D6-D5 | D4 | -D3+D2 | 0 | θ13 | 0 |
| {∑4}→{∑w6} |  | D6-D5 | -D4 | -D3+D2 | 0 | θ14 | 0 |

Note: x, y, z, x, y, z: transform parameters with respect to the parent frame; i represents the variable of the joint angle.