Source apportionment of heavy metal(loid)s in sediments and the associated risk assessment based on chemical speciation: A case study from drinking-water reservoir in Southwest China

Table S1. The classification of risk assessment code (Igeo).

| Range | Degree |
| --- | --- |
| Igeo ≤ 0 | Unpolluted |
| 0 < Igeo ≤ 1 | Unpolluted to moderately polluted |
| 1 < Igeo ≤ 2 | Moderately polluted |
| 2 < Igeo ≤ 3 | Moderately to heavily polluted |
| 3 <Igeo ≤ 4 | Heavily polluted |
| 4 < Igeo ≤ 5 | Heavily to extremely polluted |
| Igeo > 5 | Extremely polluted |

Table S2. The evaluation criteria of different toxicity levels (STU).

| Range | Degree |
| --- | --- |
| STU＜4 | Low toxicity |
| 4≤STU≤6 | Moderate toxicity |
| STU＞6 | High toxicity |

Table S3 The extraction procedure, reagents and extraction components of European Community Bureau of Reference (BCR) standard method.

| Step | Target phase | Extraction agent and procedures |
| --- | --- | --- |
| 1 | Exchangeable/acid-soluble fraction-F1 | Duplicate samples, 1.0 g+40 mL of 0.11 M CH3COOH, Shaking for 16 h then centrifuge |
| 2 | Reducible fraction-F2 | Step 1 residue sample, 40 mL NH2OH-HCl 0.5 M (pH=1.5), Shaking for 16h then centrifuge |
| 3 | Oxidizable fraction-F3 | Step 2 residue sample, 10 mL H2O2 (8.8 M) heat to 85 oC for 1 h (Repeated once) and then add 50 mL CH3COONH4 1 M (pH=2), Shaking for 16 h then centrifuge |
| 4 | Residual fraction-F4 | Step 3 residue sample (0.2 g), digested by a mixture of acids including 5 mL HNO3, 5 mL HF, and 3 mL HClO4 in Teflon beakers, heating on a hot plate equipment |

Table S4. The classification of pollution levels code (RSP).

| Range | Degree |
| --- | --- |
| ≤1 | Clean |
| ≥1~2 | Light pollution |
| ≥2~3 | Moderate pollution |
| ＞3 | Severe pollution |

Table S5. The classification of risk assessment code (RAC).

| RAC | Degree |
| --- | --- |
| ≤1 | No risk |
| ≥1~10 | Light risk |
| ≥10~30 | Medium risk |
| ≥30~50 | High risk |
| ＞50 | Very high risk |

Table S6. The classification of risk assessment code (MRI).

| Eir | MRI | Degree |
| --- | --- | --- |
| ＜40 | ＜190 | Low risk |
| ≥40~80 | ≥190~380 | Moderate risk |
| ≥80~160 | ≥380~760 | Considerable risk |
| ≥160~320 | ≥760 | High risk |
| ＞320 |  | Extremely high risk |

Fig. S1. Factors profiles and source contributions of HMs from PMF model.

Fig. S2 Spatial distribution patterns of HMs in agricultural soils of the study area.