Supplementary Material

Reclaimed Water in Taiwan: Current Status and Future Prospects

Hai-Hsuan Chenga, Wan-Sheng Yub, Shu-Chuang Tsengb, Yi-Ju Wua, Ching-Lin Hsiehc, Shi-Shuan Lind, Ching-Ping Chue, Yu-De Hunage, Wan-Ru Chen a,f, Tsair-Fuh Lin a,f, Liang-Ming Whanga,f,*

aDepartment of Environmental Engineering, National Cheng Kung University, No. 1, University Road, Tainan 701, Taiwan

bConstruction and Planning Agency, Ministry of Interior, 342, Sec. 2, Bade Rd., Taipei, Taiwan

cAdvanced Semiconductor Engineering Technology Holding, Kaohsiung 811, Taiwan

fSustainable Environment Research Laboratory (SERL), National Cheng Kung dConcord Technology Co., Ltd, No.31, Jingke 5th Rd., Nantun Dist., Taichung 408, Taiwan

dEnvironmental Engineering Research Center, Sinotech Engineering Consultants, Inc., No.280, Xinhu 2nd Rd., Neihu Dist., Taipei 11494, Taiwan

University, No. 1, University Road, Tainan 701, Taiwan

*Corresponding author. Tel.: +886-6-2757575x65837; fax: +886-6-2752790

E-mail address: whang@mail.ncku.edu.tw

List of Supplementary Tables and Figures

Table S1 Water quality standards for reclaimed water

Table S2 Effluent water quality of typical wastewater treatment plant

Table S3 Effluent water quality of typical municipal wastewater treatment plant

Fig. S1 The average water recovery rate for industrial consumers

Fig. S2 RO rejected water recycled using EDR and MVR

Fig. S3 Mechanical vapor recompression (MVR) for achieving zero liquid discharge (ZLD) goal

Table S1 Water quality standards for reclaimed water

| Water quality items | Landscape water | Tap water (Water Resources Agency) | Irrigation water | Ground water recharge |
| --- | --- | --- | --- | --- |
| Conductivity (μS/cm) | -- | -- | 750 | -- |
| Biochemical oxygen demand (BOD5) (mg/L) | <15 | -- | -- | 1 |
| Total Dissolved Solids (TDS)(mg/L) | -- | 800 | -- | 800 |
| F- (mg/L) | -- | 0.8 | -- | 0.8 |
| Cl- (mg/L) | -- | 250 | 175 | 250 |
| SO42- (mg/L) | -- | 250 | 200 | 250 |
| Total hardness (mg/L) | -- | 400 |  | -- |
| E. coli (CFU/100mL) | ND | 6 | -- | 50 |
| Turbidity (NTU) | -- | 2 | -- | -- |
| Total nitrogen (mg/L) | -- | -- | 3.0 | -- |
| NO3-N(mg/L) | -- | 10 | -- | 10 |
| NO2-N (mg/L) | -- | 0.1 | -- | ND |
| NH3-N(mg/L) | -- | 0.5 | -- | 0.1 |
| Total phosphorus (TP) (mg/L) | -- | -- | -- | -- |
| As (mg/L) | -- | 0.05 | 0.05 | 0.05 |
| Cd (mg/L) | -- | 0.005 | 0.01 | 0.005 |
| Cr (mg/L) | -- | 0.05 | 0.1 | 0.05 |
| Cu (mg/L) | -- | 1.0 | 0.2 | 1.0 |
| Ni (mg/L) | -- | -- | 0.2 | 0.1 |
| Anionic surfactant (mg/L) | -- | 0.5 | 5.0 | 0.5 |
| CN- (mg/L) | -- | 0.005 | -- | 0.01 |

Table S2 Effluent water quality of typical wastewater treatment plant

| WTP Items | Industry (Primary) | Industry (Tertiary/ comprehensive) | Industry (Tertiary/ science park) | Industry (Tertiary/ comprehensive) | Industry (Tertiary/ comprehensive) | Industry (Tertiary/ science park) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Eff. of Linyuan Industrial Parks WTP | Eff. of Hsinchu Industrial Park WTP | Eff. of HWA YA Technology Park WTP | Eff. of Waterfront Industrial Area WTP | Eff. of Guishan Industrial Park WTP | Eff. of Central Taiwan Science Park WTP |
| Conductivity | 6680~6860 | 2700~3700 | 870~895 | 3,500~5,420 | 1,538~2,040 | 2630~3020 |
| TDS | 3300~3680 | 2000~2400 | 390~444 | 1,610~3,040 | 867~916 | 1380~1660 |
| F- | 1.12~3.93 | * | 1.96~2.05 | 1.68~4,60 | 0.1~0.15 | 4.2~4.6 |
| Cl- | 210~1190 | 280~480 | 96.9~239 | 950~1260 | 236~688 | 628~654 |
| SO42- | 17.46~943 | 520~880 | 75.23~106 | 44.72~447 | 69.25~200 | 70.5~207 |
| Total hardness | 513~710 | 220~700 | 104~124 | 352~361 | 112~128 | 256~266 |
| Turbidity | 4.0~7.0 | 3.7~6.9 | 3.8~6.3 | 2.8~9.7 | 3.2~7.7 | 2.6~3.2 |
| Total nitrogen | 12.3~12.6 | 16~25 | 21.80~23.2 | 12.9~22.55 | 13.04~18.8 | 32.3~33 |
| NO3-N | 8.1~10.0 | 15~23 | 6.87~16.50 | 0.03~18.20 | 5.45~12.8 | 13.9~15.4 |
| NO2-N | 0.1~0.88 | <0.01 | 0.31~0.48 | 0.04~0.22 | 0.02~3.32 | 0.20~0.28 |
| NH3-N | 1.1~2.5 | 0.08~0.12 | 4.43~15.8 | 3.68~8.64 | 0.07~0.26 | 16.3~18.8 |
| TP | 1.93~6.76 | 3.4~15 | 8.40~8.93 | 4.24~6.56 | 0.22~1.18 | 19.9~54.7 |
| As | ND~0.004 | * | ND~0.024 | ND~0.013 | ND | ND |
| Cd | ND~0.0004 | 0.01 | ND~0.0004 | ND~0.0003 | ND~0.0004 | ND~0.0006 |
| Cr | 0.0038~0.0045 | 0.04 | ND~0.0017 | 0.0041~0.017 | 0.0048~0.009 | 0.0027~0.0028 |
| Cu | 0.01~0.0487 | 0.11 | 0.036~0.0665 | 0.0036~0.017 | 0.057~0.0603 | 0.0044~0.0047 |
| Ni | 0.0268~0.0272 | 0.18 | 0.022~0.0472 | 0.041~0.056 | ND~0.0236 | ND~0.0152 |
| Anionic surfactant | 0.08 | 0.2~0.32 | 0.07~0.11 | 0.08~0.17 | 0.46~0.53 | ND~0.06 |
| CN- | ND~0.009 | 0.008~0.01 | ND~0.025 | ND | ND~0.002 | ND~0.012 |

*No relevant information

Table S3 Effluent water quality of typical municipal wastewater treatment plant

| WTP Items | Municipal (Primary) | Municipal (Secondary) | Municipal (Secondary) | Municipal (Secondary) | Municipal (Secondary) | Municipal (Secondary) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Eff. of Bali WTP | Eff. of Fu tian water resource recycling center | Eff. of Linkou South District WTP | Eff. of Central District WTP | Eff. of Fongshan River WTP | Eff. of Anping WTP |
| Conductivity | 1020 | 427~459 | 787~791 | 722~834 | 371~790 | 1200~2000 |
| TDS | 512 | 216~354 | 348~513 | 349~606 | 371~494 | 944~2410 |
| F- | * | 0.05~0.18 | 0.18~0.2 | 0.2~1.34 | 0.14~0.25 | 0.16~0.41 |
| Cl- | 164 | 16.8~45.3 | 73~232 | 92.9~156 | 41.1~126 | 308~1080 |
| SO42- | 73 | 56~63 | 75.53~141 | 26.9~29.8 | 75.8~88.4 | 80~197 |
| Total hardness | 173 | 81~168 | 182~191 | 57~138 | 232~241 | 293~527 |
| Turbidity | 45 | 1.2~4.3 | 0.12~8.88 | 0.14~2.32 | 0.39~2.8 | 1.2~2.2 |
| Total nitrogen | 22.1 | 17~20 | 17.34~20.3 | 2.26~46.8 | 26.11~31.5 | 10~14 |
| NO3-N | 0.2 | 16.6~19.7 | 15.2~18.8 | 0.16~44.8 | 22.6~23.5 | 9.2~12.9 |
| NO2-N | 0.07 | 0.01~0.09 | 0.01~0.09 | 0.01~0.02 | 1.67~1.71 | 0.01~0.03 |
| NH3-N | 20.6 | 0.05~0.97 | 0.05~0.97 | 0.14~1.11 | 1.08~6.15 | 0.05~0.3 |
| TP | 1.6 | 3.35~7.17 | 3.35~7.17 | 10.98~40.18 | 2.63~7.63 | * |
| As | * | ND | ND~0.002 | ND~O.004 | ND | 0.003~0.007 |
| Cd | 0.004~0.024 | ND~0.001 | ND | ND | ND | ND~0.009 |
| Cr | 0.04 | ND | 0.003~0.02 | ND~0.002 | ND~0.001 | ND |
| Cu | 0.01~0.12 | ND~0.005 | 0.008~0.017 | 0.013~0.119 | 0.004 | ND |
| Ni | 0.18 | ND~0.009 | ND~0.0245 | ND~0.0058 | ND~0.0037 | ND |
| Anionic surfactant | 3.49 | ND | ND~0.05 | ND~0.06 | 0.15~0.18 | ND~0.22 |
| CN- | 0.008 | ND~0.004 | ND | ND~0.002 | ND~0.055 | ND~0.002 |

*No relevant information

Fig. S1 The average water recovery rate for industrial consumers

.  Fig. S2 RO rejected water recycled using EDR and MVR

Fig. S3 Mechanical vapor recompression (MVR) for achieving zero liquid discharge (ZLD) goal