Supporting Information

The impact of scrubber washwater discharge on the water quality in estuaries and ports

Johannes Teuchies1*, Tom Cox2,3, Katrien Van Itterbeeck4, Filip Meysman2, Ronny Blust1

1University of Antwerp, Department of Biology, Systemic Physiological and Ecotoxicological Research, Groenenborgerlaan 171, B-2020 Antwerp, Belgium

2University of Antwerp, Department of Biology, Ecosystem Management Research Group, Universiteitsplein 1C, B-2610 Wilrijk, Belgium

3  NIOZ Royal Netherlands Institute for Sea Research and Utrecht University, Department of Estuarine and Delta Systems	P.O. box 140, 4400 AC Yerseke Netherlands

4Antwerp Port Authority

*Corresponding author:

Johannes Teuchies

University of Antwerp - Campus Groenenborger

Building U, U539

Groenenborgerlaan 171, B-2020 Antwerp, Belgium

johannes.teuchies@uantwerpen.be

+32 3 265 35 33

Number of Pages: 5

Number of Figures: 2

Number of Tables: 2

- Calculation of concentration changes in contaminant concentrations

To estimate the impact of additional contaminant flux to a receiving water body we compare equilibrium concentration in a well-mixed water volume, flushed with constant discharge, with and without additional inputs.

In equilibrium, contaminant concentration C in a perfectly mixed flow-through system will equal the concentration of the incoming water

C_eq^old = C_in

When an additional flux Jc is added to the volume, the dynamical equation read

dC/dt = Jc/V + Q/V x (C_in – C)

where V is the volume of the receiving water body. Thus, in equilibrium (dc/dt=0) C_eq now is

C_eq ^new – C_eq^old = Delta C = Jc/Q

Since significant gradients exists between (contaminant) concentrations in the docks and in the Scheldt, the concentration in the docks is not only determined by freshwater flushing, but also by exchange through the locks, which we approximated to be proportional to the concentration difference. To estimate the dispersion coefficient of this exchange we assumed the salinity in the docks is in equilibrium with both the freshwater input and the exchange over the locks, thus

dS_dock/dt = 0 = -Q/V_dock x S_dock + D/V_dock x (S_Scheldt – S_dock)

=> D = Q x S_dock / (S_Scheldt - S_dock)

With observed salinities S_Scheldt ~ 4.5 PSU and S_dock ~ 4 PSU and with a typical riverine discharge of 16 m³/s we estimate D = 100 m³/s

In equilibrium, contaminant concentration C in a perfectly mixed dock with both flushing and dispersive exchange over the lock will be given by

dC/dt = 0 = Q/V_dock x (C_in – C) + D/V_dock x (C_Scheldt – C)

=> C_eq^old = 1/(Q+D) x (QxC_in + DxC_Scheldt)

With an additional contaminant flux J_c, we have

dC/dt = 0 = Jc/V + Q/V_dock x (C_in – C) + D/V_dock x (C_Scheldt – C)

=> C_eq^new – C_eq^old = Jc/(Q+D)

Figure S1. Scrubber washwater metal concentrations (µg L-1) sampled right before discharge. Open and closed loop data combined. Values below limit of quantification (LOQ) white, values above LOQ black. Y-axis in logarithmic scale. Different LOQ values per parameter are displayed, because reported LOQ values differ among different datasets.

Figure S2. Scrubber washwater PAH concentrations (µg L-1) sampled right before discharge. Open and closed loop data combined. Values below limit of quantification (LOQ) white, values above LOQ black. Y-axis in logarithmic scale. Different LOQ values per parameter are displayed, because reported LOQ values differ among different datasets.

Table S1

See Excel file.

Table S2. The total number of washwater samples (n, open and closed loop combined) and the percentage of values which were reported to be below the limit of quantification (BLOQ). The parameters in bold and underlined are included in further analysis.

| Parameter | n | % BLOQ |
| --- | --- | --- |
| As | 66 | 71 |
| Cd | 65 | 92 |
| Cr | 57 | 56 |
| Cu | 69 | 28 |
| Hg | 33 | 85 |
| Pb | 66 | 70 |
| Mo | 2 | 0 |
| Ni | 67 | 6 |
| Zn | 69 | 30 |
| V | 68 | 0 |
| Ace | 53 | 15 |
| Acy | 53 | 25 |
| Ant | 53 | 25 |
| B(a)A | 53 | 47 |
| B(a)P | 53 | 72 |
| B(b)F | 53 | 58 |
| B(ghi)P | 53 | 75 |
| B(k)F | 52 | 83 |
| Chr | 53 | 28 |
| D(a,h)A | 53 | 92 |
| Fluoran | 53 | 9 |
| Flu | 53 | 2 |
| I(123cd)P | 53 | 87 |
| Naph | 59 | 0 |
| Phe | 53 | 0 |
| Pry | 53 | 13 |
| Total PAH | 61 | 5 |

Table S3. Summarising numbers on scrubber washwater concentrations, fluxes and impact on water quality for scenario low (10% scrubbers).

(1) Average values for total concentrations in the harbour docks

(2) Water Quality Standards (WQS) from the EU WFD (*) or Flanders (dissolved concentrations for metals, total concentrations for PAHs)