Annex - Input Data and Assumption for ‘Beyond the Dams: Combatting Hydropower Over-reliance & Securing Pathways for a Low-carbon Future for Laos' Electricity Sector using OSeMOSYS (Open-Source Energy Modelling System)’

Input Data

Annex 1. The Estimated Installed Capacity for Power Generation. Source: Brinkerink & Deane (2020), Brinkerink et al. (2021), Byers et al. (2018), IRENA (2020) and EDL (2019).

| Power Generation Technology | Year | Estimated Installed Capacity (MW) |
| --- | --- | --- |
| Biomass Power Plant | 2019 | 30 |
| Coal Power Plant | 2019 | 1876.0 |
| Large Hydropower Plant (Dam) (>100MW) | 2019 | 6209 |
| Medium Hydropower Plant (10-100MW) | 2019 | 222 |
| Small Hydropower Plant (<10MW) | 2019 | 19 |
| Residential Solar PV | 2019 | 1.63 |

Annex 2. Technical Renewable Potential of Laos. Source: IES & MKE (2016), United Nations (2019) and Global Solar Atlas (2022).

| Power Generation Technology | Technical Renewable Potential (MW/yr) |
| --- | --- |
| Biomass Power Plant | 2300 |
| Large Hydropower Plant (Dam) (>100MW) | 25270 |
| Medium Hydropower Plant (10-100MW) | 1329 |
| Small Hydropower Plant (<10MW) | 453 |
| Utility Scale Solar PV | 11000 |
| Residential Solar PV | 5500 |
| Onshore Wind Power | 27400 |

Annex 3. Technological Parameters for Laos’s Power Plants. Source: IPCC (n.d.), IRENA & ACE (2016), IRENA (2020), IRENA (2021), Staffell & Pfenninger (2016a), Staffell & Pfenninger (2016b), NREL (2014) and Brinkerink & Deane (2020) from CCG’s SDK.

| Power Generation Technology | Capital Cost ($/kW in 2020) | Fixed Cost ($/kW/yr in 2020) | Operational Life (years) | Efficiency | Average Capacity Factor | Emissions Factor (kg CO2/GJ) |
| --- | --- | --- | --- | --- | --- | --- |
| Biomass Power Plant | 2750.0 | 69.0 | 25 | 0.38 | 0.7 | 100 |
| Coal Power Plant | 1300 | 52.0 | 60 | 0.3 | 0.75 | 94.6 |
| Large Hydropower Plant (Dam) (>100MW) | 1539.0 | 46.17 | 80 | 1.0 | 0.55 | N/A |
| Medium Hydropower Plant (10-100MW) | 1592.86 | 47.79 | 40 | 1.0 | 0.55 | N/A |
| Small Hydropower Plant (<10MW) | 2162.0 | 64.86 | 40 | 1.0 | 0.55 | N/A |
| Utility-scale Solar PV | 1160.0 | 15.08 | 30 | 1.0 | 0.28 | N/A |
| Residential Solar PV | 2130.8 | 42.62 | 24 | 1.0 | 0.28 | N/A |
| Onshore Wind Power | 2220.09 | 88.8 | 30 | 1.0 | 0.08 | N/A |

Annex 4. The Capital Cost, Operational Life and Efficiency of Electricity Transmission and Distribution. Source: Terpilowski-Gil (2020) and Li & Chang (2014).

| Technology | Capital Cost ($/kW in 2020) | Fixed Cost ($/kW in 2020) | Operational Life (years) | Efficiency (2020) | Efficiency (2030) | Efficiency (2050) |
| --- | --- | --- | --- | --- | --- | --- |
| Electricity Transmission | 306.39 | 6.13 | 50 | 0.91 | 0.92 | 0.95 |
| Electricity Distribution | 306.39 | 6.13 | 50 | 0.91 | 0.92 | 0.95 |

Annex 5. Electricity Export Demand. Source: ERIA & MEM (2020).

| Year | Export Demand (PJ) | Year | Export Demand (PJ) | Year | Export Demand (PJ) | Year | Export Demand (PJ) | Year | Export Demand (PJ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2015 | 47.143368 | 2026 | 85.7088687 | 2037 | 124.2943694 | 2048 | 162.8698701 | 2059 | 201.4453708 |
| 2016 | 50.6502317 | 2027 | 89.2157324 | 2038 | 127.8012331 | 2049 | 166.3767338 | 2060 | 204.9522345 |
| 2017 | 54.1570954 | 2028 | 92.7225961 | 2039 | 131.3080968 | 2050 | 169.8835975 | 2061 | 208.4590982 |
| 2018 | 57.6639591 | 2029 | 96.2394598 | 2040 | 134.8149605 | 2051 | 173.3904612 | 2062 | 211.9659619 |
| 2019 | 61.1708228 | 2030 | 99.7463235 | 2041 | 138.3218242 | 2052 | 176.8973249 | 2063 | 215.4728256 |
| 2020 | 64.6776865 | 2031 | 103.2531872 | 2042 | 141.8286879 | 2053 | 180.4041886 | 2064 | 218.9796893 |
| 2021 | 68.1745502 | 2032 | 106.7500509 | 2043 | 145.3355516 | 2054 | 183.9110523 | 2065 | 222.486553 |
| 2022 | 71.6814139 | 2033 | 110.2669146 | 2044 | 148.8424153 | 2055 | 187.417916 |  |  |
| 2023 | 75.1882776 | 2034 | 113.7737783 | 2045 | 152.349279 | 2056 | 190.9247797 |  |  |
| 2024 | 78.6951413 | 2035 | 117.280642 | 2046 | 155.8561427 | 2057 | 194.4316434 |  |  |
| 2025 | 82.202005 | 2036 | 120.7875057 | 2047 | 159.3630064 | 2058 | 197.9385071 |  |  |

Scenario Assumptions

Least Cost Scenario

The Least Cost scenario had no constraints set upon it. This was done to develop a control scenario to act as a comparison for the other scenarios. The reduced time slices were to reduce model runtime. Laos’ electricity imports (PWRTRNIMP) was removed as this analysis focuses on meeting Laos electricity demand locally and for export purposes. Export demand (PWRTRNEXP) was updated with the data seen in Annex 5 based on ERIA & MEM (2020) to reflect its increasing demand. Utility-scale PV with 2 hour storage (PWRSOL001S) and On-shore wind power plant with storage (PWRWND001S) is also removed as storage technology is not considered within this analysis. The operational years of large hydropower (PWRHYD001) was extended to 80 years, to better reflect real-life and conforming to other OSeMOSYS modelling analysis conducted on Laos such as Terpilowski-Gill (2020). The residual capacity of large hydropower (PWRHYD001) was also updated as the numbers of the original model were reported only up to 2015. All of these constraints were applied to all the following models for consistency.

Moderate & Ambitious Scenarios

The Moderate & Ambitious scenarios were set to simulate a curtailment in hydropower development based on ERIA & MEM (2020), stating the amount of projects slated to be finished by years 2025 and 2030, being mapped out in the Ambitious and Moderate scenarios respectively. The Moderate Scenario will have a constraint where no new hydropower developments can grow beyond the year 2030 and none for the Ambitious Scenario beyond the year 2025, assuming all projects are finished by their announced dates.

|  | Number of Installed Capacity Projects | Installed capacity (MW) |
| --- | --- | --- |
| Under Construction Projects and expect to complete construction by 2025 | 35 | 4,147 |
| Expect to complete by 2030 | 38 | 4,434 |

Table [1]: Hydropower projects expected to be complete

Moderate Coal Phase Out & Ambitious Coal Phase Out Scenarios

The two coal phase-out scenarios were constructed as well to understand how the Laos electricity sector can play its part in securing a low-carbon future, through the decommissioning of the Hongsa Coal Power Plant (HCPP). The year 2050 was chosen to allow for the greatest flexibility needed for any necessary infrastructure needed to phase out coal. It also aligns with Laos’ intentions to achieve net-zero by 2050, as the HCPP is currently the biggest single-source of CO2 emissions in Laos.

Moderate Net-Zero & Ambitious Net-Zero

The two Net-Zero scenarios were built out to assess the feasibility and infrastructure needed to realise a fully decarbonised electricity system in Laos. The decision to adopt the year 2050 as Laos’ net-zero point is to align with their published NDC (Lao PDR, n.d.).

Sources

Brinkerink, M. & Deane, P. (2020) PLEXOS-World 2015. Available at: https://doi.org/10.7910/DVN/CBYXBY, Harvard Dataverse, V6, UNF:6:fyT1L5t+sHlvSHolxelaVg== [fileUNF]

Brinkerink, M., Gallachóir, B. and Deane, P. (2021) ‘Building and Calibrating a Country-Level Detailed Global Electricity Model Based on Public Data’. Energy Strategy Review (33). https://doi.org/10.1016/j.esr.2020.100592

Byers, L. et al. (2018) A Global Database of Power Plants. Washington, DC: World Resources Institute.

Economic Research Institute for ASEAN (ERIA) and East Asia and Ministry of Energy and Mines (MEM) (2020) Lao PDR Energy Outlook 2020.

Electricite Du Laos (no date) StatisticYearbook 2019.

Global Solar Atlas (n.d.) Global Photovoltaic  Power Potential by Country. Available at: https://globalsolaratlas.info/global-pv-potential-study

Intelligence Energy Systems (2016) Alternatives for Power Generation in the Greater Mekong Sub- region.

IRENA and the ACE (2016) Renewable Energy Outlook for ASEAN. Available at: https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2016/IRENA_REmap_ASEAN_2016_report.pdf

IRENA (2020) Renewable Power Generation Costs in 2019. Available at: https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2020/Jun/IRENA_Power_Generation_Costs_2019.pdf

IRENA (2020) Renewable Energy Statistics 2020.

IRENA (2021) Planning and Prospects for Renewable Power: Eastern and Southern Africa. Available at: https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2021/Apr/IRENA_Planning_Prospects_Africa_2021.pdf

Lao PDR (no date a) Lao PDR Nationally Determined Contribution (NDC)

Li, Y. and Chang, Y. (2014) Infrastructure Investments for Power Trade and Transmission in ASEAN+2: Costs, Benefits, Long-Term Contracts, and Prioritised Development. Available at: https://www.eria.org/ERIADP-2014-21.pdf

NREL (2014) Global CFDDA-based Onshore and Offshore Wind Potential Supply Curves by Country, Class, and Depth (quantities in GW and PWh). Available at: https://openei.org/doe-opendata/dataset/c186913f-6684-4455- a2f2-f26e152a9b35/resource/4dc4a6fd-3a63-47df-bcbe-e9c83b83b38e/download/nrelcfddawindsc20130603.xlsx

Terpilowski-Gill, E (2020) Decarbonising the Laotian Energy System. Available at: http://hdl.handle.net/10044/1/8667114

United Nations (2019) World Small Hydropower Development Report 2019. Available at: https://www.unido.org/our-focus-safeguardingenvironment- clean-energy-access-productive-use- renewable-energy-focus-areas-small-hydro-power/world-small-hydropowerdevelopment- report

Staffell, I. and Pfenninger, S. (2016a). ‘Using bias-corrected reanalysis to simulate current and future wind power output’, Energy. (114), pp. 1224–39.

Staffell, I. and Pfenninger, S. (2016b). ‘Long-term patterns of European PV output using 30 years of validated hourly reanalysis and satellite data’, Energy. (114), pp.1251–65.