An accessible deep learning tool for voxel-wise classification of brain malignancies from perfusion MRI

Supplementary Material

Appendix A. Patient selection and classification performance of the BERRY tool in the external validation cohorts

Data from 135 patients from two external centres (HT Medica, Jaen, Spain and University of California, San Diego, California, USA) were retrospectively collected. From these, 80 patients met the inclusion criteria (excluded 26 with insufficient imaging data, 4 with artefacts impeding segmentation, 9 with no visible enhancement on the CE-T1W, 16 with lesions smaller than ten millimetres in diameter). The DSC-PWI protocol did not include contrast preload, the acquisition parameters were: TR of 1123-3031 ms, TE of 28-60 ms, FA of 40º-90º and in-plane resolution of 0.9 mm. Imaging was performed on Philips Achieva or Intera 1.5T (n=55), Siemens Essenza (n=4), Toshiba Vantage Elan 1.5T (n=2), GE Signa HDxt 1.5T (n=2), Philips Achieva 3T (n=9), GE Signa Architect 3T (n=5), GE Discovery 3T (n=2) and Philips Panorama HFO 1T (n=1).  The overall 3-way classification accuracy obtained was 0.71 with the corresponding table as follows:

|  |  | Ground truth |  |  |
| --- | --- | --- | --- | --- |
|  |  | GBM | PCNSL | Metastasis |
| Classification | GBM | 28 | 2 | 10 |
|  | PCNSL | 3 | 7 | 4 |
|  | Metastasis | 3 | 1 | 22 |

An additional cohort of 25 patients from the same institution as the development cohort (i.e., Bellvitge University Hospital) but acquired in a scanner with higher magnetic field strength (3T Philips Ingenia) was retrospectively collected, with matching acquisition parameters (Appendix B). The following table summarises the ground truth and the classification results, obtaining an overall accuracy of 0.72:

|  |  | Ground truth |  |  |
| --- | --- | --- | --- | --- |
|  |  | GBM | PCNSL | Metastasis |
| Classification | GBM | 11 | 0 | 0 |
|  | PCNSL | 3 | 1 | 1 |
|  | Metastasis | 3 | 0 | 6 |

A total of 23 patients with brain glioblastoma, DSC-PWI scan and available enhancing tumour segmentation were identified from the IvyGAP public dataset1,2. The DSC-PWI sequence was acquired after administering contrast preload, with 3T General Electric scanners (13 on Signa and 10 on Discovery). The confusion table of the classification results is given below, yielding 0.78 of accuracy:

|  |  | Ground truth |
| --- | --- | --- |
|  |  | GBM |
| Classification | GBM | 18 |
|  | PCNSL | 1 |
|  | Metastasis | 4 |

Appendix B. Imaging acquisition parameters

For the training and internal testing cohort, acquisitions were performed with 1.5T Philips scanners (219 on Ingenia and 221 on Intera). The CE-T1WI main parameters were gradient-echo with repetition time (TR) of 6.58-25 ms, echo time (TE) of 3.17-5.49 ms, flip angle (FA) of 8-30º, and spin-echo with TR of 535-625 ms, TE of 10-15 ms, FA of 70-90º, all with in-plane resolution of 0.9 mm. The DSC-PWI dynamic sequences (gradient-echo echo-planar imaging and PRESTO3) salient parameters were: temporal sampling of 1.26-1.93 s, 40-60 timepoints, TR of 1640 ms (PRESTO 16 ms) TE of 24-40 ms, FA of 75º (PRESTO 7º) and in-plane resolution of 1.7 mm, acquired during a bolus administration of gadolinium contrast agent (gadobutrol 0.1 mmol/kg) without contrast preload.

Appendix C. Conventional DSC-PWI metrics

The T2*-weighted signal S from a DSC-PWI sequence can be expressed as:

S∝e-TET2* ;1T2*=R2*

With R2* representing the relaxation rate. In a dynamic acquisition, a signal S(t) is acquired at n discrete time points t=[0,1,2,...,n] during the contrast bolus passage. The change in relaxation rate ΔR2*(t) is proportional to the contrast concentration C(t), and can be obtained as follows:

Ct∝ΔR2t*=R2t*-R20*=-1TEln StS0

The relative cerebral blood volume (rCBV) is equal to the area under the ΔR2* curve and, as such, it has been related to vascular density with respect to normal-appearing tissue. For absolute estimates, a measured arterial input function can be used as a transfer function to deconvolve the ΔR2* signal4. These estimations rely on the assumption that the contrast agent that comes in eventually comes out of the vessel, but it may not hold true if the tumour breaks the blood-brain barrier and leakage occurs. In that case, leakage correction and approximations of the area under the curve are necessary to obtain reliable results5.

The percentage of signal recovery (PSR) can be defined as the ratio of the recovered DSC-PWI signal over the baseline signal, as follows:

PSR=(Sr-Smin)(Sb-Smin)·100

Where Smin is the minimum peak point reached by the perfusion curve, Sb is the baseline perfusion signal before contrast bolus arrival and Sr is the recovered signal after bolus passage. A signal that does not return to its baseline value can reflect abnormal presence of contrast in either vessels or leaked into surrounding tissue, and therefore PSR has been related to vascular permeability.

Supplementary Tables

Supplementary Table S1. Demographics of the population included in the study, split by tumour type, train and test sets. Age is shown as median and range (minimum-maximum). No significant (p-value>0.05) differences among groups were found. The age differences between train and test sets within a tumour type were tested with Welch’s t-test. A one-way ANOVA was used to test the age differences among tumour types. The difference in the distribution of sex was tested with Fisher’s exact test and the difference among tumour types were tested with a Chi-square test (χ2).

| Tumour type | Group | Number | Age (median, range) | p-value train vs test (Welch’s) | p-value tumour types (ANOVA) | Sex |  | p-value train vs test (Fisher’s) | p-value tumour types (χ2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Male | Female |  |  |
| All |  | 440 | 62 (20-86) | - | - | 272 | 168 | - | - |
|  | Training | 40 | 64 (36-82) | p=0.721 |  | 25 | 15 | p=1.000 |  |
|  | Test | 400 | 62 (20-86) |  |  | 247 | 153 |  |  |
| GBM |  | 300 | 62 (20-86) | - | p=0.242 | 186 | 114 | - | p=0.985 |
|  | Training | 10 | 67 (48-74) | p=0.158 |  | 6 | 4 | p=1.000 |  |
|  | Test | 290 | 62 (20-86) |  |  | 180 | 110 |  |  |
| Metastasis |  | 85 | 61 (24-84) | - |  | 58 | 37 | - |  |
|  | Training | 10 | 60 (44-68) | p=0.413 |  | 6 | 4 | p=1.000 |  |
|  | Test | 75 | 61 (24-84) |  |  | 52 | 33 |  |  |
| PCNSL |  | 45 | 65 (36-82) | - |  | 28 | 17 | - |  |
|  | Training | 20 | 61 (36-82) | p=0.285 |  | 13 | 7 | p=0.767 |  |
|  | Test | 25 | 65 (37-80) |  |  | 15 | 10 |  |  |

Supplementary Table S2. Classification results for binary and three-way classifiers on the test group of patients, with the proposed CNN model and perfusion parameters PSR and rCBV. The results are given as average and 95% confidence intervals between parentheses. CNN: convolutional neural network, PCNSL: primary central nervous system lymphoma, GBM: glioblastoma multiforme, rCBV: relative cerebral blood volume, PSR: percentage of signal recovery.

| Model | Classification | Class | Accuracy | Sensitivity | Specificity |
| --- | --- | --- | --- | --- | --- |
| CNN | PCNSL vs non-PCNSL | PCNSL | 0.94 (0.93-0.94) | 0.94 (0.93-0.95) | 0.84 (0.77-0.91) |
|  |  | Non-PCNSL |  | 0.76 (0.68-0.83) | 0.94 (0.93-0.95) |
|  | GBM vs metastasis | GBM | 0.81 (0.79-0.82) | 0.93 (0.92-0.94) | 0.40 (0.36-0.44) |
|  |  | Metastasis |  | 0.40 (0.36-0.44) | 0.93 (0.92-0.94) |
|  | 3-way | GBM | 0.78 (0.76-0.79) | 0.88 (0.87-0.89) | 0.57 (0.54-0.61) |
|  |  | PCNSL |  | 0.87 (0.82-0.92) | 0.93 (0.92-0.94) |
|  |  | Metastasis |  | 0.35 (0.31-0.40) | 0.94 (0.93-0.95) |
| PSR | PCNSL vs non-PCNSL | PCNSL | 0.84 (0.83-0.85) | 0.84 (0.82-0.85) | 0.90 (0.85-0.96) |
|  |  | Non-PCNSL |  | 0.77 (0.69-0.85) | 0.83 (0.82-0.85) |
|  | GBM vs metastasis | GBM | 0.65 (0.63-0.67) | 0.47 (0.42-0.51) | 0.70 (0.68-0.72) |
|  |  | Metastasis |  | 0.70 (0.68-0.72) | 0.47 (0.42-0.51) |
|  | 3-way | GBM | 0.55 (0.53-0.56) | 0.53 (0.50-0.55) | 0.67 (0.64-0.71) |
|  |  | PCNSL |  | 0.89 (0.85-0.93) | 0.83 (0.82-0.84) |
|  |  | Metastasis |  | 0.78 (0.43-0.52) | 0.74 (0.72-0.76) |
| rCBV | PCNSL vs non-PCNSL | PCNSL | 0.72 (0.70-0.74) | 0.71 (0.69-0.73) | 0.85 (0.78-0.91) |
|  |  | Non-PCNSL |  | 0.71 (0.63-0.80) | 0.71 (0.69-0.73) |
|  | GBM vs metastasis | GBM | 0.69 (0.67-0.71) | 0.58 (0.53-0.63) | 0.73 (0.71-0.75) |
|  |  | Metastasis |  | 0.73 (0.71-0.75) | 0.58 (0.53-0.63) |
|  | 3-way | GBM | 0.59 (0.57-0.60) | 0.71 (0.69-0.73) | 0.64 (0.61-0.67) |
|  |  | PCNSL |  | 0.86 (0.81-0.90) | 0.70 (0.68-0.72) |
|  |  | Metastasis |  | 0.02 (0.01-0.04) | 0.96 (0.95-0.97) |

Supplementary Table S3. Results of the binary and three-way classifications performed on the test data with rCBV and PSR combined into single logistic regressions. Three-way classification was achieved by consecutively coupling the classifiers. The values shown are the average and 95% confidence intervals between parentheses. PCNSL: primary central nervous system lymphoma, GBM: glioblastoma multiforme, rCBV: relative cerebral blood volume, PSR: percentage of signal recovery.

| Model | Classification | Class | Accuracy | Sensitivity | Specificity |
| --- | --- | --- | --- | --- | --- |
| PSR + rCBV regression | PCNSL vs non-PCNSL | PCNSL | 0.80 (0.78-0.81) | 0.71 (0.63-0.80) | 0.80 (0.78-0.81) |
|  |  | Non-PCNSL |  | 0.79 (0.78-0.81) | 0.85 (0.78-0.91) |
|  | GBM vs metastasis | GBM | 0.53 (0.51-0.55) | 0.62 (0.60-0.64) | 0.23 (0.19-0.26) |
|  |  | Metastasis |  | 0.23 (0.19-0.16) | 0.62 (0.60-0.64) |
|  | 3-way | GBM | 0.47 (0.46-0.49) | 0.48 (0.46-0.50) | 0.75 (0.71-0.78) |
|  |  | PCNSL |  | 0.86 (0.81-0.90) | 0.78 (0.76-0.80) |
|  |  | Metastasis |  | 0.29 (0.25-0.33) | 0.69 (0.67-0.70) |

References

1.	Shah N, Feng X, Lankerovich M, Puchalski RB, Keogh B. Data from Ivy Glioblastoma Atlas Project (IvyGAP): The Cancer Imaging Archive; 2016.

2.	Data from the Multi-Institutional Paired Expert Segmentations and Radiomic Features of the Ivy GAP Dataset. The Cancer Imaging Archive (TCIA); 2020. https://wiki.cancerimagingarchive.net/pages/viewpage.action?pageId=70222827.

3.	van Gelderen P, Duyn JH, Ramsey NF, Liu G, Moonen CT. The PRESTO technique for fMRI. Neuroimage. 2012; 62(2):676-681.

4.	Ostergaard L, Weisskoff RM, Chesler DA, Gyldensted C, Rosen BR. High resolution measurement of cerebral blood flow using intravascular tracer bolus passages. Part I: Mathematical approach and statistical analysis. Magn Reson Med. 1996; 36(5):715-725.

5.	Zaharchuk G. Theoretical basis of hemodynamic MR imaging techniques to measure cerebral blood volume, cerebral blood flow, and permeability. AJNR Am J Neuroradiol. 2007; 28(10):1850-1858.