Supplementary Figures

Fig. 1 Image pair of the CEDI images from the mobile blood analyzer and the Wright-Giemsa images from the conventional bright-field microscope (40X, 0.95 NA). The blood cell types for the cell presented in the image centers were marked. P: platelet, NEU: neutrophil, EOS: eosinophil, BAS: basophil, MON: monocyte, LYM: Lymphocyte. Scale bar: 10 μm.

Fig. 2.  The two-step image registration algorithm for the CEDI images by the miniature microscope and the Writer-Giemsa images. The RBC position mask images (binary images generated after performing the RBC segmentation algorithm) were adopted as the ‘fingerprint’ characteristics of the CEDI images and the Writer-Giemsa images. In the image preprocessing stage, the Wright-Giemsa images were cut into small image patches with image sizes of 640 x 640, following a cut path of the ‘z’ shape. Then the RBC segmentation algorithm introduced in the main text was performed to obtain the RBC position mask images of all the image patches (‘fingerprint’ dataset of the Wright-Giemsa images). The input CEDI image with an image size of 128x128 would be digitally magnified with a magnification of 4. The RBC position mask of the CEDI image would be obtained by the RBC segmentation algorithm too. After the image preprocessing, the two-step registration algorithm containing coarse and fine registration steps would be performed. The matched RBC position mask from the ‘fingerprint’ dataset for the RBC position mask image of the CEDI image was searched by the coarse registration algorithm based on template matching, with a similarity criterion of minimum absolute difference (MAD). The fine registration algorithm based on the multimodal registration from MATLAB was used to correct the affine transformation between the Writer-Giemsa image (coarse registration result according to the matched RBC position mask) and the magnified CEDI image. Finally, the matched Wrigher-Giemsa and CEDI images with image sizes of 512x512 were obtained.

Fig. 3.  Confusion matrix. The horizontal axis presents the actual label of the instances and the vertical axis presents the predicted labels of the instances from the trained network. NEU: Neutrophil, LYM: Lymphocyte, EOS: Eosinophil, BAS: Basophil, MON: Monocyte, BG: Background.

Supplementary Tables

Table 1. Statistics of the image gradient for the five WBC populations and the granulocyte.

|  | Median | Interquartile Range | 95% Confidence Interval |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower Limit | Upper Limit |
| Neutrophil | 121.513 | 21.213 | 118.680 | 133.800 |
| Eosinophil | 133.608 | 29.919 | 125.617 | 149.171 |
| Basophil | 106.214 | 18.540 | 103.043 | 114.253 |
| Lymphocyte | 81.885 | 13.192 | 77.739 | 86.032 |
| Monocyte | 134.037 | 21.810 | 125.968 | 142.106 |
| Granulocyte | 126.240 | 21.213 | 118.679 | 133.780 |

Table 2. Statistics of the image area for the lymphocyte, monocyte, and granulocyte.

|  | Median | Interquartile Range | 95% Confidence Interval |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower Limit | Upper Limit |
| Lymphocyte | 6391.450 | 1329 | 5782.540 | 7000.360 |
| Monocyte | 13778.700 | 1382 | 13034.410 | 14522.990 |
| Granulocyte | 10773.100 | 1584 | 10014.690 | 11531.510 |

Table 3. Statistics of the image mean intensity for the three granulocyte populations.

|  | Median | Interquartile Range | 95% Confidence Interval |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower Limit | Upper Limit |
| Neutrophil | 166.245 | 12.177 | 162.822 | 170.693 |
| Eosinophil | 150.308 | 10.721 | 144.177 | 152.438 |
| Basophil | 168.606 | 7.125 | 165.406 | 170.369 |

Table 4. Independence sample t-test for equality of image area, mean intensity, and gradient

| Image feature |  | T-value | Degrees of freedom | P-value | Mean difference | Std. error difference | 95% Confidence interval of the difference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Lower | Upper |
| Image area | Gran Vs. Lym | 7.954 | 77 | 1.248x10-11 | 4225.821 | 531.291 | 3167.887 | 5283.756 |
|  | Gran Vs. Mon | -5.810 | 77 | 1.335x10-7 | -3161.430 | 544.139 | -4244.950 | -2077.910 |
|  | Lym Vs Mon | -16.079 | 38 | 1.529x10-18 | -7387.250 | 459.448 | -8317.85 | -6457.150 |
| Image mean intensity | Eos Vs. Bas | -8.505 | 31.135 | 2.513x10-10 | -19.580 | 2.302 | -24.274 | -14.885 |
|  | Eos Vs. Neu | 6.698 | 36.978 | 7.193x10-8 | 18.670 | 2.7887 | 13.023 | 24.319 |
| Image gradient | Lym Vs. Mon | -12.032 | 38 | 1.574x10-14 | -52.151 | 4.334 | -60.926 | -43.377 |
|  | Bas Vs. Neu | 3.717 | 37 | 0.665x10-3 | 17.018 | 4.579 | 7.741 | 26.295 |

Table 5. Independence sample t-test for equality of RBC hemoglobin means (anemia versus health).

| T-value | Degrees of freedom | P-value | Mean difference | Std. error difference | 95% Confidence interval of the difference |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Lower | Upper |
| -58.625 | 1259.401 | 0.000 | -11.289 | 0.193 | -11.667 | -10.911 |

Table 6. Independence sample t-test for equality of RBC area means (anemia versus health).

| T-value | Degrees of freedom | P-value | Mean difference | Std. error difference | 95% Confidence interval of the difference |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Lower | Upper |
| -17.764 |  | 9.996 x 10-62 | -17.764 | -6.912 | -7.675 | -6.148 |

Table 7. Statistic data of the RBC hemoglobin and area for the anemia sample.

|  | Median | Interquartile Range | 95% Confidence Interval |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower | Upper |
| Hemoglobin | 21.509 | 5.370 | 21.269 | 21.913 |
| Area | 48.254 | 11.356 | 47.196 | 48.593 |

Table 8. Statistic data of the RBC hemoglobin and area for the health sample.

|  | Median | Interquartile Range | 95% Confidence Interval |  |
| --- | --- | --- | --- | --- |
|  |  |  | Lower | Upper |
| Hemoglobin | 33.064 | 4.813 | 32.683 | 33.078 |
| Area | 55.037 | 7.776 | 54.482 | 55.121 |

Table 9. Performance indexes for FWD testing.

|  | Neutrophil | Eosinophil | Basophil | Lymphocyte | Monocyte | Average |
| --- | --- | --- | --- | --- | --- | --- |
| Precision | 0.993 | 0.941 | 0.920 | 0.979 | 0.974 | 0.961 |
| Recall | 0.991 | 0.941 | 0.958 | 1.000 | 0.987 | 0.975 |
| F1 Score | 0.992 | 0.941 | 0.939 | 0.989 | 0.980 | 0.968 |

Table 10.  Sample amount for FWD (n = 30)

| Sample | Image | Total WBC | Neutrophil | Eosinophil | Basophil | Lymphocyte | Monocyte |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 145 | 326 | 273 | 1 | 8 | 28 | 16 |
| 2 | 150 | 242 | 133 | 5 | 2 | 91 | 11 |
| 3 | 160 | 207 | 109 | 60 | 1 | 34 | 3 |
| 4 | 178 | 341 | 287 | 2 | 1 | 43 | 9 |
| 5 | 87 | 118 | 78 | 4 | 0 | 25 | 10 |
| 6 | 164 | 245 | 133 | 2 | 3 | 82 | 25 |
| 7 | 118 | 848 | 825 | 1 | 1 | 12 | 9 |
| 8 | 189 | 290 | 202 | 5 | 1 | 65 | 17 |
| 9 | 174 | 293 | 159 | 4 | 2 | 108 | 20 |
| 10 | 149 | 237 | 167 | 1 | 0 | 65 | 4 |
| 11 | 141 | 210 | 130 | 2 | 0 | 75 | 3 |
| 12 | 147 | 189 | 122 | 2 | 2 | 61 | 2 |
| 13 | 163 | 263 | 178 | 2 | 1 | 79 | 3 |
| 14 | 180 | 290 | 244 | 9 | 2 | 27 | 8 |
| 15 | 89 | 191 | 92 | 10 | 1 | 73 | 15 |
| 16 | 119 | 328 | 180 | 10 | 5 | 67 | 66 |
| 17 | 85 | 184 | 144 | 0 | 0 | 34 | 6 |
| 18 | 131 | 235 | 196 | 1 | 2 | 34 | 2 |
| 19 | 68 | 104 | 75 | 14 | 0 | 12 | 1 |
| 20 | 94 | 190 | 135 | 3 | 0 | 44 | 8 |
| 21 | 154 | 258 | 101 | 11 | 2 | 143 | 2 |
| 22 | 30 | 187 | 181 | 0 | 0 | 3 | 2 |
| 23 | 45 | 330 | 323 | 0 | 0 | 3 | 4 |
| 24 | 50 | 343 | 335 | 1 | 0 | 3 | 4 |
| 25 | 90 | 184 | 155 | 1 | 2 | 23 | 5 |
| 26 | 50 | 92 | 79 | 1 | 1 | 11 | 1 |
| 27 | 90 | 147 | 120 | 6 | 1 | 14 | 6 |
| 28 | 100 | 166 | 137 | 7 | 1 | 18 | 4 |
| 29 | 70 | 152 | 129 | 0 | 6 | 10 | 7 |
| 30 | 90 | 202 | 174 | 1 | 4 | 16 | 7 |