Supplementary materials for: Multidimensional optical tweezers synthetized by rigid-body emulated structured light

Liuhao Zhu,1 Yuping Tai,1 Hehe Li1, Huajie Hu,1 Xinzhong Li1,*, Yangjian Cai2, 3,* and Yijie Shen4, *

1School of Physics and Engineering, Henan University of Science and Technology, Luoyang, 471023, China

2Shandong Provincial Engineering and Technical Center of Light Manipulations and Shandong Provincial Key Laboratory of Optics and Photonic Device, School of Physics and Electronics, Shandong Normal University, Jinan 250014, China

3Photoelectronics Research Centre and Centre for Photonic Metamaterials, University of Southampton, Southampton SO17 1BJ, United Kingdom

*Corresponding Authors: Xinzhong Li, E-mail: xzli@haust.edu.cn, Yangjian Cai, E-mail: yangjian_cai@163.com, and Yijie Shen, E-mail: y.shen@soton.ac.uk

In the supplementary material, we present a method for designing a six-dimensional (6D) optical tweezer beam, a description of the phase mask design, supplementary experimental proofs, and additional optical tweezer parameter measurements.

# 1.	SUPPLEMENTARY OF 6D OPTICAL TWEEZERS THEORY

First, we designed a pair of 2D structured optical tweezers. A structured optical field with an arbitrary curvilinear mode is reported in [1]. In the initial plane, the beam is represented as

(S1)

where |c'0(t)| = [x'0(t)2 + y'0(t)2]1/2 dominates the shape of the beam with t∈[0, T = 2π] and ' denotes the differential operation. Ψ0(x1, y1, t) and ψ0 (x1, y1, t) are terms that represent the phase of the beam.

(S2)

Here, λ is the wavelength of light in the medium and f is the focal length of the lens. z0(t) is the defocusing distance defined along the curve c0(t). The curve is determined by the relations x0(t), y0(t), and z0(t). In this case, a circular curve was selected as the candidate to construct the optical tweezers, and the relationships among these three parameters are as follows:

(S3)

Optical tweezers were constructed using two semicircles by assigning opposing TCs. The interference petal point is the tip and the two semicircles are the arms of the optical tweezers [2,3]. 2D optical tweezers have the advantage of a larger trapping range [4]. To realize optical tweezers using the aforementioned structured optical field, we alter the upper and lower integral limits of these two semicircles on the circle curve described by (S3). Assuming that |m| represents the TC magnitude of the beam (m is set as 30 in this case), and the integral limits satisfy T2 = (T3-T1)/2 + (T3-T1)/2m. Here, the amplitude of the optical tweezers is determined as

(S4)

(S5)

(S6)

where i = 1 and 2; t1∈[T1, T2] and t2∈[T3, T2]. To ensure that the beam has a closed curve, T1 = 0 and T3 = 2π. It's worth noting that if we want to get two focal points, we just need to set T1=-(T3-T1)/2m and T3=2π+(T3-T1)/2m. If need more focal optical tweezers, we can just continue to segment the function.

Intuitively, a pair of real 6D optical tweezers can be obtained via combined translation and rotation operations around the center of the 3D tweezers. After these operations, the tip of the tweezers can theoretically reach an arbitrary position, according to the researcher’s desires. First, to better describe and regulate these three dimensions, it is necessary to use the universal rotation theory of the rigid-body Euler angle. The nutation angle 𝜃 and precession angle ψ can be obtained by modifying beam equations and coordinate transformation techniques. The transformed coordinates can be expressed as follows

(S7)

where (r1, ψ1) are the polar coordinates corresponding to the coordinates of (x1, y1) and (x2, y2) are the coordinates after the transformation. Here, Ψi(x1, y1, ti) in (S5) and (S6) can be rewritten as follows:

(S8)

Substituting (S7) and (S8) into (S4) and (S5), the roll and pitch can be obtained using this rigid-body beam. Yaw (rotation along the z-axis) can then be achieved by changing the relationship between T1 and T2 to T1 = φ and T3 = T1 + 2π. The relation after transformation can be expressed as

(S9)

Finally, we present the creation of tweezers for the translation of the three axes. The translation in the x-y plane surge and sway can be realized by the Fourier phase-shift theorem, whereas the translation along the z-axis heave can be realized using additional spherical waves. The beam was shifted in the image plane by adding a phase shift to the object plane. Hence, all these were achieved by changing the phase masks. Therefore, we add plane and spherical waves to its computer-generated hologram (CGH) to execute the manipulation displacement operation.

(S10)

Where Δx, Δy, and Δz are the displacements of the surge, sway, and heave, respectively, along the x-, y-, and z-axes. Owing to the Fourier displacement theorem, the phase shifts of the plane and spherical waves applied to the hologram become the displacement of the toroidal light, which can be obtained as

(S11)

Six independent DoFs are contained in the total complex amplitude of the 6D structured optical tweezers M(θ,ψ,φ,Δx,Δy,Δz). Assuming that the ring light tip coordinate is p (x0, y0, z0), its coordinates can be expressed as

(S12)

# 2	DIGITAL GENERATION OF 6D OPTICAL TWEEZERS

## 2.1	Phase mask design of generalized 6D optical tweezers and experiment of trap particle via orbital angular momentum

Fig. S1 CHG of phase masks designed and used for Figs. 4 and 5 in the main article, and Fig. S3 hereinafter are shown in sub-figures (a1)–(a5), (b1)–(b5), and (c1)–(c5), respectively.

Up to date, tweezers have been successfully transformed into spatial optical tweezers with six DoFs. Before optical tweezer manipulation, it is necessary to clarify the generation method of 6D optical tweezers. In this study, the CGH method was exploited using an SLM. The phase mask is expressed as follows:

(S13)

where Arg[ ] is the phase extraction function. Several CGH phase masks of the 6D optical tweezers used in the main text are shown in Fig. S1. The phase masks in Fig. 4 in the main text are shown in the top row of Fig. S1. In the experiment, we generated 100 phase masks and refreshed the masks at a rate of 0.5 s for the purpose of particle manipulation. Here, the refresh rate and interval between the values of the masks simultaneously determine the movement speed of the particle. For the same number of masks, the faster the refresh rate, the faster the particle's movement speed. However, the refresh rate is limited by the hardware of the instrument, and the large spacing of the mask also leads to a change in the particle forces, hence, small particles cannot be manipulated. Therefore, it was adjusted by weighing the experimental conditions.

Next, it was necessary to experimentally verify that it can carry out 3D rigid-body rotation and capture particles to the tip using the OAM. The results obtained using the experimental device in the main text are shown in Fig. S2. First, we ensured that the beam and particle were not in the same plane, and the results are shown in Fig. S2 (a1)-(a2). The particle does not move, but only carries out Brownian motion in situ. The beam then gradually rotated its precession angle until it was in the same plane as the particle. As shown in Fig. S2 (a2)–(a5), the particle is gradually captured and rotated using the OAM to the tip (for details, see Visualization 3).

Fig. S2 Yeast was captured by changing the precession angle. (a1)—(a5) Images recorded during the experiment, (b1)—(b5) schematic diagrams of the 3D model.

Fig. S3 Yeast perform a 360-degree flip. (a1)—(a5) Images recorded during the experiment, (b1)—(b5) schematic diagrams of the 3D model.

Finally, we made the particles perform a 360-degree flip to demonstrate that our beam can travel at any angle. In this experiment, we chose the spin angle φ= 0–2π as shown in Fig. S3 (a1)–(a5) (for details, see Visualization 4). The experimental results fully prove large angle the angle between the toroidal structured beam and the x-y plane can be achieved and the ability of generating a toroidal structured beam in the x-z or y-z plane.

## 2.2	Characterization of the performance of the proposed 6D optical tweezers

To characterize the performance of the proposed optical tweezers, the trapping force f and trap stiffness k of the optical tweezers were estimated using Stokes’ drag force test [5] as follows:

(S14)

where η is the dynamic viscosity of the surrounding medium (in this case, for an aqueous solution, η = 0.89 mPa s), Fdrag is the drag force, r is the radius of the particles, and v is the limiting velocity as the particles travel. Considering generality, polystyrene spheres (diameter:3 μm) were selected for the drag test. The power of the beam before entering the microscope objective is dominant. The parameters of the optical tweezers were R0 = 0.5 mm and ψ = 0–2π. The maximum velocity of the polystyrene sphere in the light trap was measured by rotation, as shown in Fig. S4(a1)–(a4).

Furthermore, we determined the optical trap stiffness of the trapped particles with diameters of 1, 3, and 5 μm under different input powers, as shown in Fig. S4(b). The initial power required to drive the particle increased with an increase in the particle size, and the initial powers for moving 1, 3, and 5 μm particles were 2.04, 3.1, and 4.8 mW, respectively. This is because the larger particle has a larger contact surface with the liquid, and therefore, a greater viscous resistance, which requires greater power to provide sufficient drag force.

The trap stiffness of the particle is proportional to the laser power for a specific particle. By fitting the experimental data, the slopes of the 1-, 3, and 5 μm particles were K1=0.0239, K2=0.0355, and K3=0.0323 pN/μm, respectively. If necessary, a stronger optical trap stiffness can be obtained to satisfy the different requirements of various particles by increasing the laser power to avoid overheating. However, the optical trap stiffness of the 3 μm particles increased faster than that of the others. To determine the reasons for this, we measured the tip size of the proposed optical tweezers with a diameter of 2.75 μm, which indicates that the optical trap stiffness is determined by the size match between the tip and trapped particle [6]. Consequently, the performance of the optical tweezers, especially the optical trap stiffness, is dominated by not only the optical tweezers but also the trapped particles, which agrees well with the results in [7]. For practical applications, the optimal tip of the optical tweezers should be carefully designed for different particles. For researchers to manipulate smaller particles to break the diffraction limit, several advanced imaging techniques should be employed, such as dark-field microscopy [1, 6]. Achieving this goal requires extensive effort and should be further studied in the near future. However, most of the biological particles are within the scales of 0.5–10 μm [7], which can be manipulated in 3D space by employing the proposed optical tweezers.

Fig. S4 (a1)–(a4) Stokes’ drag force test to determine the trapping force and the stiffness of the proposed 6D optical tweezers. (b) Optical trap stiffness versus input power. A polystyrene sphere with a diameter of 3 μm was employed in the experiments.

Disclosures

Supplementary materials can be published.

References

Rodrigo J A, Angulo M, Alieva T. All-optical motion control of metal nanoparticles powered by propulsion forces tailored in 3D trajectories. Photonics Res., 2021; 9(1): 1-12.

A.V. Ponomarev, S. Denisov, P. Hanggi. ac-Driven Atomic Quantum Motor. Phys. Rev. Lett., 2009; 102(23): 230601.

H. Zhang, X. Li, H. Ma, M.  Tang, H.  Li, J. Tang, et al. Grafted optical vortex with controllable orbital angular momentum distribution, Opt. Express, 2019; 27(16): 22930-22938.

X. Li, H. Ma, H. Zhang, M. Tang, H. Li, J. Tang, et al. Is it possible to enlarge the trapping range of optical tweezers via a single beam? Appl. Phys. Lett. 2019; 114: 081903.

J.E. Melzer, E. McLeod. Fundamental limits of optical tweezer nanoparticle manipulation speeds. ACS nano, 2018; 12: 2440-2447.

A. Rohrbach. Stiffness of optical traps: quantitative agreement between experiment and electromagnetic theory. Phys. Rev. Lett., 2005; 95(16): 168102.

L. Huang, H. Guo, J. Li, L. Ling, B. H. Feng, Z. Y. Li, Optical trapping of gold nanoparticles by cylindrical vector beam. Opt. Lett., 2012; 37(10): 1694-1696.