Table S1. Experimental Protocol

| Experiment | State | ECMO Configuration | ECMO Flow Direction | Starting Conditions | ECMO Flow Rate (L/min) | Procedure |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | - Normal - LVF - RVF - BVF | - VA | - Retrograde | - mAoP: 50mmHg, 70mmHg, 90mmHg | - 0 - 2 - 3 - 4 - 5 | - Increased ECMO flow rate by adjusting only pump speed - mAoP not kept constant |
| 2 | - Normal - LVF - RVF - BVF | - VA | - Retrograde | - mAoP: 50mmHg, 70mmHg, 90mmHg | - 0 - 2 - 3 - 4 - 5 | - Increased ECMO flow rate by adjusting pump speed and SVR - mAoP kept constant at 50, 70 and 90mmHg respectively |
| 3 | - LVF | - VA | - Retrograde | - mAoP: 70mmHg - LV gain: 2.4, 2.2, 2.0, 1.8, 1.6 | - 0 - 2 - 3 - 4 - 5 | - Increased ECMO flow rate by adjusting only pump speed - mAoP not kept constant |
| 4 | - LVF | - VA | - Retrograde - Antegrade | - mAoP: 70mmHg | - 0 - 2 - 3 - 4 - 5 | - Increased ECMO flow rate by adjusting only pump speed - mAoP not kept constant |

BVF = biventricular failure; ECMO = extra-corporeal membrane oxygenation; LVF = left ventricular failure; mAoP = mean aortic pressure; RVF = right ventricular failure; SVR = systemic vascular resistance; VA = veno-arterial