Supporting Material For

Hydrochemical characteristics, water quality assessment and driving forces of coal base in Shanxi Province, China

Zixuan Yan1,2, Zhanbin Li1,2, Peng Li1,2,*, Chenxu Zhao1, Yaotao Xu1,2, Zhiwei Cui1,2, Hu Sun1,2

(1 State Key Laboratory of Eco-Hydraulics in Northwest Arid Region of China, Xi’an University of Technology, Xi’an, Shaanxi 710048, China

2 State Key Laboratory of National Forestry Administration on Ecological Hydrology and Disaster Prevention in Arid Regions, Xi’an University of Technology, Xi’an, Shaanxi 710048, China)

*Corresponding author. Peng Li, Xi’an University of Technology, No.5, South Jinhua Road, Xi’an, Shaanxi 710048, China.

Email address: lipeng74@163.com

Table S1 Physico-chemical parameters and methods used in the CRB

|  | Parameter (Abbreviation) | Method |
| --- | --- | --- |
| In-situ | Electric conductivity (EC) | A multi-parameter water quality tester (HANNA HI98194) |
|  | pH |  |
|  | Dissolved oxygen (DO) |  |
|  | Water temperature (WT) |  |
| Ex-situ | Calcium ion (Ca2+) | Flame atomic absorption spectrometry (GB5749-2006, 2006)a |
|  | Magnesium ion (Mg2+) |  |
|  | Sodium ion (Na+) |  |
|  | Potassium ion (K+) |  |
|  | Chloride ion (Cl-) | Silver nitrate volumetric method (GB 11914-89)e |
|  | Sulfate ion (SO42-) | Ion Chromatography (HJ 535-2009, 2009)b |
|  | Bicarbonate ion (HCO3-) | Hydrochloric acid titration method (HJ 636-2012, 2012)c |
|  | Total dissolved solids (TDS) | Oven drying method; (GB5749-2006, 2006)a |

a Water quality - Determination of permanganate index of People's Republic of China (GB5749-2006), 2006 (in Chinese).

b Water quality - Determination of ammonium nitrogen of People's Republic of China (HJ 535-2009), 2009 (in Chinese).

c Water quality - Determination of total nitrogen of People's Republic of China (HJ 636-2012), 2012 (in Chinese).

d Water quality - Determination of total phosphorus of People's Republic of China (GB 11893-89), 1990 (in Chinese).

e Water quality - Determination of chemical oxygen demand of People's Republic of China (GB 11914-89), 1990 (in Chinese).