Appendix A. Supplementary data

## Table A1 Water quality indicators at sampling sites in April 2021

| Number | Parameters | Concentrations (mg/L) | Number | Parameters | Concentrations (mg/L) |
| --- | --- | --- | --- | --- | --- |
| 1 | pH | 8.47 | 8 | K+ | 8.05 |
| 2 | COD | 18 | 9 | Ca2+ | 41.9 |
| 3 | TP | 0.02 | 10 | Mg2+ | 68.1 |
| 4 | TN | 0.83 | 11 | Cl- | 256.6 |
| 5 | NO3-N | 0.225 | 12 | SO42- | 272.4 |
| 6 | NH4+-N | 0.166 | 13 | Salinity | 1200 |
| 7 | Na+ | 94.0 |  |  |  |

Note: Dimensionless for pH.

## Table A2 Analysis methods and their resources, equipment, and detection limits for water quality parameters

| Projects | Methods | Resources | Equipment | Detection limit |
| --- | --- | --- | --- | --- |
| pH | Measured in situ | / | pH meter (PHSJ-5, China) | 0mg/L |
| COD | Titrimetric method | HJ 828-2017 | COD digestion (HCA-100, China) | 4 mg/L |
| TP | Spectrophotometry | GB 11893-1989 | Spectrophotometer (UV-2550, Japan) | 0.01 mg/L |
| NH4+-N | Spectrophotometry | HJ 535-2009 | Spectrophotometer (UV-2550, Japan | 0.025 mg/L |
| TN | Spectrophotometry | GB 11894-1989 | Spectrophotometer (UV-2550, Japan) | 0.05 mg/L |
| NO3–-N | Spectrophotometry | HJ/T 346-2007 | Spectrophotometer (UV-2550, Japan) | 0.08 mg/L |
| K+ | Ion chromatography | American Public Health Association (Clescerl et al., 2012) | Ion chromatograph (CS–2100, USA) | 0.001 mg/L |
| Ca2+ | Ion chromatography | American Public Health Association | Ion chromatograph (CS–2100, USA) | 0.001 mg/L |
| Na+ | Ion chromatography | American Public Health Association | Ion chromatograph (CS–2100, USA) | 0.0007 mg/L |
| Mg2+ | Ion chromatography | American Public Health Association | Ion chromatograph (CS–2100, USA) | 0.002 mg/L |
| Cl– | Ion chromatography | American Public Health Association | Ion chromatograph (Advanced IC, Switzerland) | 0.04 mg/L |
| SO42– | Ion chromatography | American Public Health Association | Ion chromatograph (Advanced IC, Metrohm, Switzerland) | 0.1mg/L |
| HCO3– | Titrimetric method | Water and Wastewater Monitoring and Analysis Methods (4th Edition) (Wei, 2008) | pH meter (PHSJ-5, China) | / |
| Salinity | Gravimetric method | Water and Wastewater Monitoring and Analysis Methods (4th Edition) | Electronic balance (BSA-224CW, Germany) | 0mg/L |

References:

Clescerl L S, Greenberg A E, Eaton A D (2012). Standard Methods for Examination of Water & Wastewater: Centennial Edition. Washington: American Public Health Association

Wei F S (2008). Water and Wastewater Monitoring Analysis Method (Fourth Edition). Beijing: China Environmental Science Pres

## Fig. A1 The sampling site of Shahu Lake

## Fig. A2 ANOVA of DOM compositions under different salinity: (a), C1; (b), C2; (c), C3; (d), C4. "*" indicates a significant difference of P < 0.05, and "ns" indicates no significant difference

## Fig. A3 PARAFAC of DOM in pore water