Article title: Use of life cycle assessment for estimating impacts of waste-to-energy technologies in solid waste management systems: The case of Buenos Aires, Argentina

Journal name: Environmental Science and Pollution Research

Special issue on Life cycle thinking (LCT) applied to Solid Waste Management Systems with Appropriate Technologies (LCTSWAT-22)

Authors: Luciana Galván, Raquel Bielsa

Affiliations

Luciana Galván

Instituto del Conurbano, Universidad Nacional de General Sarmiento, Juan María Gutiérrez 1150, Los Polvorines (1613), Buenos Aires, Argentina.

Raquel Bielsa

Instituto del Conurbano, Universidad Nacional de General Sarmiento, Instituto del Conurbano, Universidad Nacional de General Sarmiento, Juan María Gutiérrez 1150, Los Polvorines (1316), Buenos Aires, Argentina.

Corresponding author:

Raquel Bielsa

Instituto del Conurbano, Universidad Nacional de General Sarmiento, Instituto del Conurbano, Universidad Nacional de General Sarmiento, Juan María Gutiérrez 1150, Los Polvorines (1316), Buenos Aires, Argentina.

rbielsa@campus.ungs.edu.ar , (54 11) 4469-7795

Appendix A: Supplementary material

Table S1: LCI related to waste collection: plastic bags, street containers (lifespan equal to 10 years). Database obtained from the WRATE v.3 libraries.

| Waste collected | Type of container | Capacity (kg) | Lifespan (year) | Construction materials per container (km) |
| --- | --- | --- | --- | --- |
| Mixed MSW from municipalities | Disposable plastic bags | 23.4 | - | LDPE and pigments 0.0224 HDPE 0.0056 |
| Wet MSW from CABA | Container for kerbside collection | 280.0 | 5 | Steel (virgin) 99.06 HDPE 5.00 Paint 1.8 Rubber 1.06 |
| Dry MSW from CABA | Container for kerbside collection | 650.0 | 15 | HDPE 90 Steel 50 |

Table S2: MSW collection and transportation operative parameters.

| Waste fraction | Vehicle type | Capacity (tonnes)** | Average distances per travel* | Construction and maintenance materials ** | Average fuel consumption ** |
| --- | --- | --- | --- | --- | --- |
| Collection Wet/dry MSW from CABA | Kerbside | 5.5 | 4.7 km 100% urban streets | Construction Steel = 8,880 kg Tires = 180kg Others metals = 1,796 kg Others = 245.3 kg Maintenance Steel = 6.73 kg Tires = 145 kg Others = 34.4 kg | Urban street 67.489 l.km-1 |
| Collection mixed MSW to transfer stations | Compactor | 12.842 | 18.16 km 30% Urban street 70% Motorway | Construction Steel = 12,070 kg Tires = 600kg Others metals = 100 kg Others = 227 kg Maintenance Steel = 12.33 kg Tires = 150 kg Others = 40.95 kg | Urban street 67.489 l.km-1 Motorway 33.99 l.km-1 |
| Collection mixed MSW to sanitary landfill | Compactor | 12.842 | 19.16 km 10% Urban street 90% Motorway | Construction Steel = 12,070 kg Tires = 600kg Others metals = 100 kg Others = 227 kg Maintenance Steel = 12.33 kg Tires = 150 kg Others = 40.95 kg | Urban street 67.489 l.km-1 Motorway 33.99 l.km-1 |

Table S2 (continuation): MSW collection and transportation operative parameters.

| Transportation to sanitary landfill | Intermodal | 17.5 | 42.2 km 5-20% urban street 95-80% motorway | Construction Steel = 10,228 kg Tires = 368 kg Others metals = 2,258 kg Others = 5kg Maintenance Steel = 6.73 kg Tires = 150 kg Others = 34.54 kg | Urban street 64 l.km-1 Motorway 66.72 l.km-1 |
| --- | --- | --- | --- | --- | --- |

*Own information collection.

**Secondary data (WRATE v.3 database).

Table S3: Emission factor for MSW collection and transportation vehicles. Database obtained from the WRATE v.3 libraries.

| Emission | Kerbside (kg.100 km-1) | Compactor (kg.100 km-1) | Intermodal (kg.100 km-1) |
| --- | --- | --- | --- |
| Nitrogen oxides (NO y NO2 as NO2) | Urban Street 1,1589 | Urban Street 1,1589 Motorway 0,5983 | Urban Street 1,0272 Motorway 1,0747 |
| Butadiene (1-3 butadiene) | Urban Street 0,0016 | Urban Street 0,0016 Motorway 0,00228 | Urban Street 0,000309 Motorway 0,00263 |
| Particulate matter (PM10) | Urban Street 0,02814 | Urban Street 0,02814 Motorway 0,01303 | Urban Street 0,02389 Motorway 0,02352 |
| Benzene | Urban Street 0,0001 | Urban Street 9,0E-5 Motorway 3,00E-05 | Urban Street 7,00E-05 Motorway 6,00E-05 |
| Carbon dioxide (fossil) | Urban Street 154,619 | Urban Street 154,611 Motorway 78,253 | Urban Street 149,64 Motorway 156,002 |
| Carbon monoxide (fossil) | Urban Street 0,3519 | Urban Street 0,3559 Motorway 0,1112 | Urban Street 0,2365 Motorway 0,2016 |
| Sulphur oxides (SO2 y SO3 as SO2) | Urban Street 0,000991 | Urban Street 0,00492 Motorway 0,00249 | Urban Street 0,00093 Motorway 0,00097 |
| Dioxins and furans (as ITEQ) | Urban Street 1,14E-12 | Urban Street 1,46E-12 Motorway 7,35E-13 | Urban Street 1,38E-12 Motorway 1,44E-12 |
| Non-methane volatile organic compounds | Urban Street 0,037883 | Urban Street 0,0354 Motorway 0,354 | Urban Street 0,1078 Motorway 0,0739 |
| Methane | Urban Street 0,002774 | Urban Street 0,0026 Motorway 0,0026 | Urban Street 0,0137 Motorway 0,0069 |
| Nitrous oxide | Urban Street 0,0026 | Urban Street 0,0026 Motorway 0,0026 | Urban Street 0,0026 Motorway 0,0026 |
| Ammonium | Urban Street 0,0003 | Urban Stree 0,00028 Motorway 0,000304 | Urban Street 0,000362 Motorway 0,000306 |

Table S4: Emission factor for MSW treatment facilities.

| Emission | Transfer station | MRF plant | MBT plant | Recycling plant |
| --- | --- | --- | --- | --- |
| Water | 27.000,00 | - | - | 1.039,95 |
| Nitrogen oxides (NO y NO2 as NO2) | 691,2 | 1.460,00 | 665,4 | 0,00591 |
| Particulate matter (PM10) | 79,49 | 117,33 | 50,34 | - |
| Carbon monoxide (fossil) | 408,67 | 845,97 | 165,54 | 0 |
| Carbon dioxide (fossil) | 72.851,42 | 148.293,20 | 34.168,80 | 7,417 |
| Non-methane volatile organic compounds | 139,32 | 297,3 | 126,6 | - |
| Dioxins and furans (as ITEQ) | 7,76E-10 | 1,58E-09 | 8,46E-07 | 0,00E+00 |
| Sulphur oxides (SO2 y SO3 as SO2) | 46,38 | 1.460,00 | 21,72 | - |
| Methane |  | - | 28,8 | 0,00019 |
| Nitrous oxide (N2O) | - | - | - | 1,17E-02 |
| Chemical oxygen demand (COD) | - | - | - | 16,58 |
| Suspended solids |  | - | - | 0,332 |

Values correspond to installations with capacity defined by default in WRATE software, these are: transfer station: 80,000 t/year; MRF plant: 50,000 t/year; TMB plant: 60,000 t/year; Plastics recycling plant: 1 t/year.

Table S5: Emission factor for MSW incineration plants.

| Emission | Emission factor grate combustion plant (kg)* | Emission factor fluidized bed plant (kg)** |
| --- | --- | --- |
| Water | 2,60E+10 | 124.105.047,18 |
| Ammonia | 1556 | - |
| Antimony (Sb) | 9,69 | 4,08 |
| Arsenic (As) | 16,37 | 0,2625 |
| Polychlorinated biphenyls (PCBs) | 0,0002137 | 0,0002008 |
| Cadmium (CD) | 11,67 | 0,37 |
| Hydrogen chloride | 17.132,00 | - |
| Cobalt (Co) | 12,34 | 4,571 |
| Copper (Cu) | 16,37 | 6 |
| Volatile organic compounds -VOCs | 637 | - |
| Volatile organic compounds other than methane | 339.81 | 140 |
| Chromium (Cr) | 16,37 | 6 |
| Biological Oxygen Demand (BOD) | - | 419,32 |
| Sulphur dioxide (SO2) | 330,09 | - |
| Carbon dioxide | 82.595,41 | - |
| carbon dioxide, fossil | 1.306.721,82 | 3.503.051,10 |
| Dioxins and furans - as ITEQ | 4,00E-05 | 3,68E-03 |

Table S5 (continuation): Emission factor for MSW incineration plants.

| Tin (Sn) | 18,94 | 6 |
| --- | --- | --- |
| Hydrogen fluoride | 37,5 | - |
| Polycyclic aromatic hydrocarbons (PAHs) | 0,2345 | 0,08686 |
| Manganese (Mn) | 16,37 | 6 |
| Particulate matter - PM10 | 1,05E+04 | 156,83 |
| Mercury (Hg) | 2,18 | - |
| carbon monoxide, fossil | 2.371.45 | - |
| Carbon monoxide, biogenic | 4.955,31 | 412,44 |
| Nitrogen Monoxide (NO) | 107.500,00 | 39.810,00 |
| Nickel (Ni) | 16,37 | 0,2625 |
| Nitrous oxide (N2O) | 64.000,00 | 457,1 |
| Sulphur oxides (SOx) SO2 and SO3 as SO2 | 57.780,00 | - |
| Nitrogen, NO and NO2 oxides as NO2 | 231.525,82 | 96.876,02 |
| Petroleum | - | 46,66 |
| Lead (Pb) | 16,37 | 6 |
| Thallium (Tl) | 10,43 | 4.949,20 |
| Vanadium (V) | 12,34 | 8,593 |

*The values correspond to a grid incineration plant with a process capacity of 225,000 t/year (default capacity of WRATE software).

**Values correspond to a fluidized bed incineration plant with a process capacity of 120,000 t/year (default capacity of WRATE software).

Table S6: Allocation of substances to MSW components and reduction factors for landfill gas.

| Component | Paper and card | Plastics (film) | Plastics (dense) | Textile materials | Sanitary waste | Wood | Combustible | Non- combustible |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Methane | 0,333 | 0,000 | 0,000 | 0,015 | 0,024 | 0,125 | 0,100 | 0,000 |
| Carbon Dioxide | 0,333 | 0,000 | 0,000 | 0,015 | 0,024 | 0,125 | 0,100 | 0,000 |
| Hydrogen | 0,333 | 0,000 | 0,000 | 0,015 | 0,024 | 0,125 | 0,100 | 0,000 |
| Alcohols | 0,333 | 0,000 | 0,000 | 0,015 | 0,024 | 0,125 | 0,100 | 0,000 |
| Aldehydes | 0,333 | 0,000 | 0,000 | 0,015 | 0,024 | 0,125 | 0,100 | 0,000 |
| Aliphatic | 0,333 | 0,000 | 0,000 | 0,015 | 0,024 | 0,125 | 0,100 | 0,000 |
| BTEXs | 0,075 | 0,031 | 0,046 | 0,024 | 0,011 | 0,026 | 0,083 | 0,024 |
| CFCs | 0,007 | 0,048 | 0,357 | 0,019 | 0,012 | 0,000 | 0,126 | 0,041 |
| Chlorinated solvents | 0,142 | 0,052 | 0,162 | 0,020 | 0,009 | 0,133 | 0,116 | 0,034 |
| Chlorinated solvent degradation products | 0,106 | 0,064 | 0,160 | 0,024 | 0,011 | 0,128 | 0,079 | 0,040 |
| Chlorobenzene | 0,381 | 0,030 | 0,098 | 0,016 | 0,020 | 0,059 | 0,047 | 0,046 |
| HCFCs | 0,000 | 0,046 | 0,320 | 0,012 | 0,009 | 0,000 | 0,069 | 0,040 |
| Hydrocarbons | 0,069 | 0,003 | 0,019 | 0,013 | 0,003 | 0,006 | 0,077 | 0,000 |
| Ketones | 0,125 | 0,006 | 0,009 | 0,021 | 0,008 | 0,040 | 0,070 | 0,000 |
| Partial combustion products | 0,132 | 0,000 | 0,000 | 0,017 | 0,007 | 0,018 | 0,066 | 0,000 |
| Aromatic substitutes | 0,099 | 0,048 | 0,173 | 0,267 | 0,011 | 0,013 | 0,154 | 0,000 |
| Sulphuric | 0,062 | 0,001 | 0,010 | 0,005 | 0,020 | 0,016 | 0,055 | 0,063 |
| Terpenes | 0,032 | 0,000 | 0,000 | 0,001 | 0,000 | 0,032 | 0,033 | 0,018 |

Base on Life Cycle Analysis, Landfill Gases Report (Hall, D., Plimmer, B., & Taylor, D., 2005. Life Cycle Assessment- Landfill emissions. http://www.golder.com).

Table S7: Leachate transfer factors in landfills.

| Substance | Transfer factor* | Total load (kg.(tonne of MSW-1)) |
| --- | --- | --- |
| Ammoniacal nitrogen | 0,005 | 3,34E-01 |
| Aniline | 0,2 | 1,10E-05 |
| Arsenic | 0,3 | 4,41E-04 |
| Biphenyl | 0,4 | 1,42E-06 |
| Cadmium | 0,3 | 1,54E-05 |
| Chlorine | 1 | 2,52E+00 |
| chromium | 0,7 | 5,17E-04 |
| Copper | 0,5 | 1,87E-04 |
| Cyanide | 1 | 6,96E-05 |
| di(2-ethylhexyl) phthalate | 1 | 8,86E-05 |
| Ethylbenzene | 0,2 | 2,31E-07 |
| Fluorine | 1 | 7,46E-03 |
| Lead | 1 | 5,37E-04 |
| MCPA (chlorinated herbicide) | 0,05 | 1,69E-05 |
| Mecoprop (chlorinated herbicide) | 0,01 | 7,02E-06 |
| Mercury | 1 | 3,54E-06 |
| MBTE (methyl tertiary butyl ether) | 1 | 8,62E-06 |
| Naphthalene | 0,05 | 2,18E-06 |
| Nickel | 0,8 | 4,77E-04 |
| Noniphenol | 0,05 | 1,94E-07 |
| Organic tin compounds | 1 | 9,58E-07 |
| Pentachlorophenol | 0,5 | 3,53E-04 |
| Phenol | 0,01 | 8,80E-07 |
| Phosphorus | 0 | 1,28E-03 |
| Potassium | 1 | 1,02E+00 |
| Sodium | 1 | 1,97E+00 |
| Trichloroethylene | 1 | 1,25E-07 |
| Toluene | 0,2 | 1,36E-06 |
| Xylene | 0,4 | 1,76E-06 |
| Zinc | 0,3 | 6,65E-03 |

*A value of 1 means that there is no reduction in concentration.