Supporting Information

A minimalistic magnetic manipulation system for manouvering nanorobots

Sayanta Goswami, Ambarish Ghosh, Debayan Dasgupta.

# Details of the movie M.1:

This movie demonstrates the evolution of the rotating magnetic field in the YZ plane for one cycle of the sinusoid (0≤ωt≤2π). The phase difference between Z pair of coils and the Y coil is 90°. Colour legend gives the profile of the norm of magnetic flux density.

# Details of the movie M.2:

This video shows the motion of a helical magnetic nanorobot in a Newtonian medium (viscosity around 100 cP) subjected to a rotating magnetic field. The amplitude of the field is 5 mT, and the frequency of rotation is 3 Hz. The trajectory of the helical magnetic nanorobot is given in the main text.

# Mathematical formulation of rotating magnetic field

To generate the rotating field, we have maintained a phase difference of 90° between the signals of the pair of coils aligned along the Z direction and the coil placed along the Y direction. In this section, we will derive the formulation of the rotating field mathematically.

Let the pair of coils aligned along the Z-direction be excited with a signal,

z=Acosωt				                         				(1)

Hence the magnetic field along the Z-direction can be written as,

zA=cos⁡(ωt)											(2)

And the signal given to the coil placed along the Y-direction is

y=Bcosπ2+ωt										(3)

The magnetic field along the Y-direction can be considered as,

yB=sin(ωt)											(4)

Now by (2) and (4), we can write,

z2A2+y2B2=cos2ωt+sin2ωt								(5)                        (z2A2+y2B2)=1											(6)

Equation (6) gives the equation of an ellipse. If the rotating magnetic field is perfectly uniform, i.e., A=B, then (6) becomes,

z2+y2=1											(7)

It is the equation of a circle.

- Variation in the b/a ratio with time:

We have taken the data of the magnetic field continuously for three minutes. We have plotted the ellipse from this data for two sinusoid cycles, and by doing the ellipse fitting, we get the b/a ratio for a couple of cycles. Variation in the b/a ratio with time is plotted for different frequencies and magnetic field strengths. Figure S.1 shows that the b/a ratio is almost the same for 200 seconds, irrespective of the magnetic field strength. Hence, we can say there is no drift in the magnetic field with time.

- (b) (c) Figure S.1: Variation in the b/a ratio at a frequency (a) 1 Hz, (b) 5 Hz, and (c) 10 Hz with time is given at different amplitudes of the magnetic field.