# Supplemental Information

Occurrence and risk evaluation of endocrine disrupting chemicals in wastewater and surface water of Lahore, Pakistan

Muhammad Ashfaq1,2, Yan Li2,3, Muhammad Zubair1*, Muhammad Saif Ur Rehman4, Sajjad Hussain Sumrra1, Muhammad Faizan Nazar5, Ghulam Mustafa1, Muhammad Tahir Fazal6, Humayun Ashraf7, Qian Sun2,*

1Department of Chemistry, University of Gujrat, Gujrat 50700, Pakistan

2CAS Key Laboratory of Urban Pollutant Conversion, Institute of Urban Environment, Chinese Academy of Sciences, Xiamen 361021, China

3School of ecological environment and Urban Construction, Fujian University of Technology, Fuzhou, Fujian 350118, China

4Department of Chemical Engineering, Khawaja Fareed University of Engineering & Information Technology, Rahim Yar Khan, Pakistan

5Division of Science and Technology, Department of Chemistry, University of Education Lahore, Multan Campus, Pakistan

6Department of Chemical Engineering, COMSATS University Islamabad, Lahore-54000 Pakistan

7Department of Geography, University of Gujrat, Gujrat 50700, Pakistan

* Corresponding Author:

Dr. Muhammad Zubair, Email: muhammad.zubair@uog.edu.pk

Dr. Qian Sun, Email: qsun@iue.ac.cn

Sample preparation and analysis

Briefly, out of 1L collected samples, 500 mL water sample of each sampling site was used for sample extraction/preparation. After sample preparation, samples were shifted to China for analysis. Samples were analyzed on liquid chromatography triple quadrupole mass spectrometry (LC-QqQ-MS) and gas chromatography mass spectrometry (GC-MS). Five of the analytes like BPA, TCS, TCC, NCC and DCC were analyzed on LC-MS using multiple reaction monitoring (MRM) mode in negative ionization mode, whereas other compounds were analyzed using GC-MS. For GC-MS analysis, the samples were derivatized before analysis. Derivatization was accomplished by spiking the nitrogen-dried samples with 50 µL of anhydrous pyridine followed by addition of 50 µL BSTFA. Samples were vortexed for 1-2 minutes and were heated at 70 oC for 1 h. After heating, samples were cooled to room temperature and diluted to 1.00 mL using n-hexane. Analysis was immediately started on GC (Model 6890A Agilent, Palo Alto, CA) coupled with quadrupole mass spectrometer (Model 5975C). Separations were accomplished on HP-5MS capillary column (30 m × 25 µm, 0.25 µm film thickness) with flow rate of Helium gas as 1 mL/min.

Table S1: Class, LogKow, MS parameters, MQLs and recoveries of the selected EDCs and their transformation products.

| Compound | Class | LogKow | GC-MS |  | LC-MS/MS | Recoveries (%) ± RSD (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Quantitative ion | Qualitative ion/ions | Q1a/Q3b, Q3c |  |
| OP | Alkylphenol | 4.12 | 278 | 179, 73 | - | 78.8 ± 3.6 |
| 4-n-NP | Alkylphenol | 3.28 | 292 | 179 | - | 68.9 ± 5.2 |
| BPA-MME | TP of Plasticizer | - | 299 | 314 | - | 108 ± 6.6 |
| BPA-DME | TP of Plasticizer | 4.06 | 241 | 256 | - | 111 ± 5.4 |
| BPA | Plasticizer | 3.44 | - | - | 227/ 211, 133 | 89.1 ± 3.9 |
| MeTCS | TP of TCS | 5.0 | 304 | 302, 254, 252 | - | 65.4 ± 4.5 |
| TCS | Antimicrobial | 4.80 | - | - | 288/ 35, 37 | 82.7 ± 6.3 |
| NCC | TP of TCC | - | - | - | 229/ 169, 185 | 71.9 ± 2.9 |
| DCC | TP of TCC | - | - | - | 254/ 92, 156 | 80.4 ± 2.2 |
| TCC | Antimicrobial | 4.90 | - | - | 279/ 124, 186 | 88.5 ± 2.6 |
| E1 | Estrogen | 3.13 | 342 | 257, 343 | - | 98.7 ± 5.4 |
| E2 | Estrogen | 3.94 | 416 | 285, 417 | - | 121 ± 6.6 |
| EE2 | Estrogen | 4.15 | 425 | 440 | - | 112± 4.9 |
| 4-OH E1 | TP of E1 | - | 430 | 345 | - | 47.9 ± 3.4 |
| 4-OH E2 | TP of E2 | - | 504 | 373 | - | 51.1 ± 4.9 |
| E3 | Estrogen | 2.45 | 504 | 414, 345 | - | 95.4 |

a, precursor ion (m/z); b, product ion 1; c, product ion 2; RSD, relative standard deviation.